| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0104-v1 |
| --- | --- |

| Title: | AHG10: Report of temporal layer dependent deblocking filter setting for HM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Han Zhang Xiang Li Shan Liu | Tel: Email: | zhhanzhang@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In response to the request in 25th JVET meeting, this document reports the objective quality of applying the same deblocking filter parameter setting of VTM-16 to HM. In VTM, the deblocking filter control parameters BetaOffset_div2 and TcOffset_div2 are adjusted based on temporal layer. The same strategy and parameter are also introduced to HM. It is reported that under CTC, compared with HM-16.24, the overall performance drop is observed as follows:

- AI: 0.49%/1.07%/1.59% with 100%EncT/101%DecT

- RA: 0.64%/0.19%/0.31% with 100%EncT/100%DecT

- LDB: 0.26%/-0.01%/0.11% with 100%EncT/100%DecT

- LDP: 1.61%/0.42%/0.78% with 100%EncT/99%DecT

Introduction

In the 25th JVET (JVET-Y) meeting, a new deblocking filter-related configuration setting was proposed in JVET-X0085 and adopted to the latest VTM version [1]. In the new setting, the deblocking filter control parameters BetaOffset_div2 and TcOffset_div2 vary with temporal level for better coding efficiency. It was requested to apply the same deblocking filter setting to HM to check the performance. This document reports the objective results of using temporal layer dependent deblocking filter control parameters in HM.

Temporal layer dependent deblocking filter control parameters

Considering the different temporal layer settings for different coding configurations, DBF control parameters in Table2.1, Table2.2, and Table2.3 are used for RA, LD, and AI, respectively. These parameters are in line with VTM-16 configuration.

Table2.1 – DBF control parameters for RA

| Tid Parameter | I slice | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- |
| BetaOffset_div2 | -2 | -2 | -2 | -2 | -2 | -2 |
| TcOffset_div2 | -5 | -4 | -3 | -2 | -1 | 0 |

Table2.2 – DBF control parameters for LD

| Tid Parameter | I slice | 0 |
| --- | --- | --- |
| BetaOffset_div2 | -3 | -3 |
| TcOffset_div2 | -3 | -3 |

Table2.3 – DBF control parameters for AI

| Tid Parameter | I slice |
| --- | --- |
| BetaOffset_div2 | -2 |
| TcOffset_div2 | -5 |

Test results

Table3.1 summarizes the overall results of the temporal layer dependent deblocking filter setting compared to HM-16.24 [2].

Table3.1- Test results of temporal layer-dependent deblocking filter setting

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.24 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.92% | 1.37% | 2.44% | 100% | 101% |
| Class A2 | 0.43% | 1.21% | 1.06% | 102% | 103% |
| Class B | 0.40% | 0.71% | 1.43% | 101% | 102% |
| Class C | 0.13% | 0.59% | 0.87% | 99% | 100% |
| Class E | 0.73% | 1.90% | 2.49% | 100% | 99% |
| Overall | 0.49% | 1.07% | 1.59% | 100% | 101% |
| Class D | 0.12% | 0.66% | 0.91% | 99% | 100% |
| Class F | 0.11% | 0.37% | 0.30% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over HM-16.24 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.99% | 0.88% | 1.07% | 100% | 98% |
| Class A2 | 0.93% | 0.47% | 0.09% | 100% | 100% |
| Class B | 0.55% | 0.04% | 0.37% | 100% | 100% |
| Class C | 0.29% | -0.36% | -0.16% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.64% | 0.19% | 0.31% | 100% | 100% |
| Class D | 0.31% | -0.41% | -0.53% | 101% | 100% |
| Class F | 0.23% | 0.10% | 0.16% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over HM-16.24 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.53% | -0.03% | 0.05% | 100% | 99% |
| Class C | 0.24% | -0.27% | -0.20% | 100% | 100% |
| Class E | -0.16% | 0.37% | 0.61% | 100% | 99% |
| Overall | 0.26% | -0.01% | 0.11% | 100% | 100% |
| Class D | 0.23% | -0.21% | -0.23% | 100% | 100% |
| Class F | 0.01% | 0.60% | -0.18% | 101% | 98% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over HM-16.24 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.77% | 0.10% | 0.42% | 101% | 100% |
| Class C | 0.93% | -0.08% | 0.25% | 101% | 98% |
| Class E | 2.25% | 1.61% | 2.09% | 100% | 97% |
| Overall | 1.61% | 0.42% | 0.78% | 100% | 99% |
| Class D | 0.63% | -0.29% | 0.43% | 99% | 99% |
| Class F | 0.29% | -0.33% | -0.71% | 100% | 102% |

Conclusion

It is reported that applying the same temporal layer dependent deblocking filter setting of VTM-16 to HM leads to coding loss for all configurations.

It is proposed to keep the original deblocking filter setting in HM.

Reference

- H. Zhang, J. Jung, X. Li, S. Liu, “AHG10: Report of Deblocking filter setting for VTM”, JVET-Y0085, Jan. 2022.

- https://vcgit.hhi.fraunhofer.de/jvet/HM/-/tree/HM-16.24

Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.