| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0107 |
| --- | --- |

| Title: | Quality evaluation of internal 10-bit versus 8-bit processing with 8-bit source content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joel Jung Xiang Li Shan Liu | Tel: Email: | joeljung@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the quality evaluation of internal 10-bit processing versus 8-bit processing, considering 8-bit input content. A selection of JVET sequences, encoded with VTM 16.0rc1, is subjectively evaluated through a CCR viewing methodology, by expert viewers. The detailed analysis of the subjective scores reveals that the difference between the two approaches is more visible at low bitrate, for class E and for low delay configuration. The reported difference remains however very slight on average, suggesting that it is safe to use 10-bit internal bit depth processing, even with 8-bit source content.

# Database selection

A database made of JVET content has been selected based on typical content features. The source reference videos (SRCs):

- Contain 7 JVET sequences: BasketBallDrive, BQTerrace, BQMall, PartyScene, FourPeople, KristenAndSara and AOV5.

- Are all 8-bit contents.

- Have resolutions from 1080p to 240p, and frame rates of 60 fps and 50 fps.

The hypothetical reference circuits (HRCs):

- Are produced with VTM16.0rc1 encoder.

- Consider 3 rate points: R1, R3, R4.

- Consider 2 coding configurations: RA, LB.

The “anchor” corresponds to a 10-bit internal processing. The “tested” approach corresponds to an 8-bit internal processing.

# Viewing session methodology

A Comparative Category Rating approach (CCR), as defined in annex E of ITU-T P.800 [ITU-T P.800], has been used for the viewing test. The test has been organized in 4 sessions of 8 minutes each. Each sessions followed the scheme depicted in figure 1, where A and B are the anchor or tested methods, in random order. The viewer does not know which sample is the anchor and which one is the tested stimuli. The advantage of a CCR approach compared to an Absolute Category Rating (ACR) is the increased discriminatory power.

Figure 1: description of the voting flow for one test cell / pair.

The viewing methodology follows the REV guidelines [REV AG5], except that a 7-grade opinion scale [ITU-T P.800] has been used, as shown in figure 2. The 7-grade opinion scale provides more granularity to the experts for rating the stimuli, reduces the confidence intervals, and removes the randomness of the rating and its corresponding bias, when anchor and tested have a close visual quality.

Figure 2: 7-grade opinion scale.

The viewing session also included a training session, and a detailed set of instructions, to ensure the methodology and rating scale are perfectly understood by the viewer. 2 trapping tests cells were hidden in the viewing sessions, where A and B consisted in extremely different quality levels that had to be scored with a 3 (or a -3). Any different rating would be a clear indicator that the viewer either did not have a sufficient visual acuity, or was in an unsuitable viewing environment to perform the test in a reliable way.

The displayed content is 8-bit, for anchor and tested method. The “-d 8” option (right shift 10-bit decoded YUV to 8-bit) is used when decoding the streams with internal 10-bit processing.

The scores of 11 expert viewers, all working in video coding field, have been averaged, producing the comparison mean opinion score (CMOS) and no viewer has been rejected as an outlier. The overall confidence interval is 0.62.

# Subjective viewing results

Table 1 shows the naming convention used to represent each sequence in the following graphs and tables. It includes an information on the class of the sequence.

| B_S04 | BasketBallDrive |
| --- | --- |
| B_S05 | BQTerrace |
| C_S02 | BQMall |
| C_S03 | PartyScene |
| E_S01 | FourPeople |
| E_S03 | KristenAndSara |
| F_S02 | AOV4 |

Table 1: list of sequences.

Figures 3 and 4 respectively report the CMOS for the random access and the low delay coding configurations. A positive CMOS indicates that 10-bit is better than 8-bit. Tables 2 and 3 additionally report the bitrate difference between the anchor and the proposal for each sequence and rate points. A positive value indicates that the bitrate of the anchor is higher than the bitrate of the proposal. Bitrates are close, except for B_S05 RP1.

Figure 3: subjective results for random access configuration.

|  | B_S04 | C_S02 | F_S02 | C_S03 | B_S05 |
| --- | --- | --- | --- | --- | --- |
| RP1 | 2% | -2% | 1% | -1% | 22% |
| RP3 | 2% | 0% | 0% | 1% | 3% |
| RP4 | 2% | 0% | 0% | 1% | 3% |

Table 2 bitrate difference for random access configuration (positive: more bits for anchor).

Figure 4: subjective results for low delay configuration.

|  | B_S04 | C_S02 | F_S02 | C_S03 | B_S05 | E_S03 | E_S01 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RP1 | 5% | 3% | 4% | 3% | 24% | -2% | -5% |
| RP3 | 4% | 5% | 5% | 6% | 6% | 2% | 4% |
| RP4 | 5% | 6% | 6% | 7% | 5% | 4% | 5% |

Table 3: bitrate difference for low delay configuration (positive: more bits for anchor).

It can be observed that the difference between 10-bits and 8-bit internal processing is hardly visible under practical viewing conditions. While there is a trend in favor of 10-bit, which is even more clear for the low delay configuration, the CMOS remains below 1, while “1” means “slightly better” with the 7-grade opinion scale. In the same spirit, we can observe that the confidence interval overlaps the x-axis for all content, indicating that there is no clear degradation resulting from the 8-bit processing. It is however expected that with more viewers (15 to 20), this confidence interval would further decrease, and that it would not cross the x-axis anymore for some of the test points.

This result is confirmed in figure 5: for more than 60% of the test cells, 10-bit and 8-bit are classified as equal pairs. In the other cases, the 10-bit approach receives more votes.

Figure 5: comparison of 8-bit and 10-bit.

Figures 3 and 4 report average CMOS. It is questionable whether the obtained CMOS, below 1, results from a general agreement of the viewers, or a disagreement that vanishes with the averaging process. The graphs below provide answers to the following questions: is the difference between 10-bit and 8-bit more visible for a given encoding configuration (figure 6), at low or high rates (figure 7), for some sequences (figure 8), for some viewers (figure 9)?

From figures 6 to 9, we can derive that:

- The difference between 10-bit and 8-bit internal processing is more visible for low-delay configuration.

- The difference between 10-bit and 8-bit internal processing is more visible at low bitrate.

- The difference between 10-bit and 8-bit internal processing is more visible on class E.

- For 7 viewers, the difference is hardly visible. For 2 viewers (viewers 4 and 5), a difference can clearly be seen between 10-bit and 8-bit for a majority of test cells. None of the viewers prefers the 8-bit approach.

Figure 6: split per encoding configuration.

Figure 7: split per rate point.

Figure 8: split per sequence.

Figure 9: split per viewer.

# Objective quality results

6 objective metrics were computed. Besides the well-known VMAF, PSNR, SSIM and MS-SSIM metrics, we report results of LPIPS [LPIPS], Learned Perceptual Image Patch Similarity), using squeezeNet architecture. LPIPS is a deep learning-based objective, full-reference metric, that evaluates the similarity of an image with a reference image, rather than the quality itself. A positive score in tables 4 and 5 indicates an advantage for the 10-bit approach, in respectively the random access and the low delay configurations.

For fair comparison, the objective scores are computed on 8-bit decoded YUVs, even if internal 10-bit is used for internal computation.

The objective metrics confirm the subjective results presented in section 3, with the following information:

- The difference is more visible on low delay than on random access coding configuration, as confirmed by each metric, with an average gain for 10-bit of 7.0% and 3.2% respectively.

- The difference is higher for class E than for other content. It seems that metrics additionally “see” a difference between 10-bit and 8-bit for B_S05 that is not perceived by humans under practical viewing conditions. A visual inspection, made on still images confirms that some areas of B_S05 are better when using 10-bit approach. This difference might result from the difference of bitrates between anchor and tested for this specific sequence as reported in tables 2 and 3, rather than a concrete quality difference.

While the metrics report, as most of the time, the correct trend, they face difficulties to quantify the amount of improvement correctly.

| Metric | B_S04 | B_S05 | C_S02 | C_S03 | F_S02 |
| --- | --- | --- | --- | --- | --- |
| VMAF | 2.3% | 2.4% | 2.1% | 2.0% | 3.7% |
| PSNR | 2.5% | 4.1% | 3.2% | 3.0% | 3.8% |
| SSIM | 3.8% | 5.7% | 4.0% | 2.9% | 5.4% |
| LPIPS- squeeze | -2.9% | 12.7% | -1.0% | -0.2% | 2.4% |
| MS-SSIM | 3.4% | 4.2% | 3.6% | 2.0% | 4.7% |
| All | 1.8% | 5.8% | 2.4% | 1.9% | 4.0% |

Table 4: objective metrics results for random access configuration.

| Metric | B_S04 | B_S05 | C_S02 | C_S03 | E_S01 | E_S03 | F_S02 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VMAF | 5.9% | 3.8% | 2.8% | 3.5% | 5.3% | 6.5% | 4.3% |
| PSNR | 5.2% | 5.0% | 3.5% | 2.9% | 5.9% | 6.6% | 4.8% |
| SSIM | 6.9% | 8.7% | 3.7% | 2.7% | 6.2% | 7.1% | 5.9% |
| LPIPS-squeeze | 11.3% | 21.2% | 11.4% | 7.9% | 12.2% | 20.0% | 13.8% |
| MS-SSIM | 7.0% | 5.7% | 4.1% | 2.6% | 6.7% | 8.2% | 5.9% |
| All | 7.3% | 8.9% | 5.1% | 3.9% | 7.3% | 9.7% | 6.9% |

Table 5: objective metrics results for low delay configuration.

# Conclusion

This contribution compares the quality of 10-bit and 8-bit internal processing. When sequences are displayed under normal viewing conditions, the difference between 10-bit and 8-bit is only slightly visible. Most of the viewers cannot notice significant differences between 10-bit and 8-bit, while a minority of them can see it very clearly (for many sequences / configurations).

The slight differences are more visible for LB configuration, on class E, and at low bitrate.

The results of the subjective evaluation are aligned with the conclusion provided by objective metrics, also the objective metrics do not correctly quantify the amplitude of the quality difference.

As a conclusion, internal 10-bit processing shows slightly better or no worse visual quality than internal 8-bit for 8-bit input contents, while the coding efficiency benefit is more visible by objective metrics. As no negative visual quality impact is observed, it is safe to use internal 10-bit to code 8-bit content.

# Reference

[REV-AG5] 		J. Jung, M. Wien, V. Baroncini, “Guidelines for remote experts viewing sessions”, AG5-N00040, October 2021.

[ITU-T P.800] 		Methods for objective and subjective assessment of quality, ITU-T P.800, 08/1996.

[LPIPS]			R. Zhang, P. Isola, A.A. Efros, E. Shechtman, O. Wang, “The Unreasonable Effectiveness of Deep Features as a Perceptual Metric”, CVPR 2018.

# Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.