| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0149_v1 |
| --- | --- |

| Title: | Non-EE2: Adaptive Filter Shape Switch for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

In the current adaptive loop filter (ALF) design, online-trained filters only have one fixed filter shape. In this contribution, a CTB-level adaptive filter shape switch (AFSS) method for ALF is proposed. With AFSS, a filter shape index which is associated with Luma filters is signaled for each APS. Each CTB could select the best luma filter shape from the available APSs.

On top of ECM 4.0, simulation results (BD-rate changes, encoding time and decoding time) are reported as below:

RA: -0.10%, 0.02%, 0.00%, 101%, 99%.

LDB: -0.24%, 0.13%, -0.05%, 104%, 100%.

Introduction

In ALF of ECM-4.0, only a fixed diamond filter shape is used for the online-trained luma filters. This kind of single filter shape design may not achieve the optimal compression performance. In this contribution, an adaptive filter shape switch (AFSS) method for ALF is proposed.

Proposed method

With AFSS, two candidate filter shapes: a diamond shape as shown in Figure 2-1, which is currently applied in ECM and a cross shape as shown in Figure 2-2, which is newly introduced, can be adaptively selected by the luma filters in ALF. The number of coefficients that need to be trained and signaled for a luma filter is 22 for both the filter shapes. Please note that these 22 taps are constituted with 20 spatial taps and 2 offline-trained-filters based taps in both of the filter shapes.

In each APS, a shape index for the online-trained luma filters is signaled to decoder. Each APS contains the luma filters that are associated with the filter shape index.

For each CTB, an APS index is signaled to indicate which luma filter shape is used to filter the current CTB. When filtering a luma sample, the coefficients and clip indices are also rearranged according to the corresponding filter shape.

It should be noted that all the sample positions of spatial taps in the newly introduced filtering shape may also be accessed by an existing fixed filter in the current ALF.

Figure 2-1 The Currently Used Diamond Filter Shape in ALF.

Figure 2-2 The Newly Introduced Filter Shape for ALF.

Simulation results

The proposed method is implemented on top of the ECM-4.0 [1]. Simulations are performed following the common test conditions [2]. More detailed results could be found in the attached excel sheets. The results compared to ECM-4.0 are shown in TABLE 3-1. The proposed method is disabled in All-Intra configurations.

TABLE 3-1 Performance of Adaptive Filter Shape Switch for ALF

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.06% | 0.08% | 101% | 99% |  |  |  |  |  |
| Class A2 | -0.10% | -0.07% | 0.02% | 101% | 101% |  |  |  |  |  |
| Class B | -0.06% | 0.04% | -0.02% | 101% | 99% | -0.21% | -0.01% | -0.22% | 104% | 100% |
| Class C | -0.18% | 0.01% | -0.03% | 101% | 99% | -0.25% | 0.17% | -0.03% | 102% | 101% |
| Class E |  |  |  |  |  | -0.29% | 0.30% | 0.22% | 107% | 97% |
| Overall | -0.10% | 0.02% | 0.00% | 101% | 99% | -0.24% | 0.13% | -0.05% | 104% | 100% |
| Class D | -0.32% | 0.11% | 0.03% | 103% | 101% | -0.29% | 0.87% | 0.57% | 105% | 100% |
| Class F | -0.08% | -0.05% | -0.21% | 106% | 99% | 0.17% | 0.66% | -0.48% | 107% | 99% |

Conclusions

In this contribution, an adaptive filter switch method for ALF is proposed. The proposed method can achieve a promising coding performance with negligible encoding time increase and no decoding time increase. It is recommended to adopt the proposed method into ECM.

Reference

- ECM-4.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-4.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).