| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0153-v1 |
| --- | --- |

| Title: | EE2-3.2: IBC Reference Area Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu*, Na Zhang† *8910 University Center Lane San Diego, CA92122, USA †Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | +1-619-436-7216 xujizheng@bytedane.com |
| Source: | ByteDance Inc. |  |  |

_____________________________

# Abstract

This contrubution describes the design of EE2-3.2 and presents the test results. By extending the reference area for intra block copy, the average BD-rate (Y/U/V) results for classF and classTGM are listed below:

AI: classF: –3.78%/–3.51%/–3.60%; classTGM: –7.92%/–7.36%/–7.33%;

RA: classF: –2.43%/–2.38%/–2.62%; classTGM: –5.14%/–4.94%/–5.07%;

LB: classF: –1.15%/–0.57%/–1.73%; classTGM: –4.04%/–3.66%/–3.77%.

# Introduction

Compared with VTM, some new coding tools in ETM require more memory. For example, the search range at the decoder for IntraTMP covers 5 times of the block width to the left and 5 times of the block height above. To support intraTMP for a 64x64 block, it needs to access 320 upper samples and 320 left samples. The extra memory can also help to improve the coding efficiency of intra block copy (IBC). At the JVET-Y meeting, it was proposed to extend the reference area for IBC [1] and an EE test was established to have more detailed investigation.

# Proposed design

In this test, the reference area for IBC is extended to two CTU rows above. Figure 1 illustrates the reference area for coding CTU (m,n). Specifically, for CTU (m,n) to be coded, the reference area includes CTUs with index (m–2,n–2)…(W,n–2),(0,n–1)…(W,n–1),(0,n)…(m,n), where W denotes the maximum horizontal index within the current tile, slice or picture. This setting ensure that for CTU size being 128, IBC does not require extra memory in the current ETM platform. The per-sample block vector search (or called local search) range is limited to [–(C << 1), C >> 2] horizontally and [–C, C >> 2] vertically to adapt to the referecnce area extension, where C denotes the CTU size.

Figure 1. Reference area for IBC when CTU (m,n) is coded. The blue block denotes the current CTU; green blocks denote the reference area; and the white blocks denote invalid reference area.

# Experimental results

Table 1. The summarized result compared with ECM-4.0 anchor.

Table 1 list the summary. Detailed results can be found in the accompanying excel file.

# Reference

[1] J. Xu, “Non-EE2: Intra Block Copy with An Extended Reference Area,” JVET-Y0124

# Patent rights declaration(s)

ByteDance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).