| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0160-v1 |
| --- | --- |

| Title: | EE2-3.5: BVP candidate adjustment based on IBC reference region |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Damian Ruiz Coll, Alexey Filippov, Vasily Rufitskiy, Tae Meon Bae | Tel: Email: | duizcoll@ofinno.com, afilippov@ofinno.com, vrufitskiy@ofinno.com, taemeon.bae@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This document reports the results of the EE2-3.5 test proposed in the JVET-0133 contribution, which proposes two techniques for the IBC Merge/AMVP list construction: (1) the clipping of the invalid candidates pointing outside the IBC reference region, (2) replacing the zero-padding candidates with a set of valid predictor candidates distributed in the IBC reference region.

On top of the ECM-4.0, simulation results for tests EE2-3.5a, EE2-3.5b, and EE2-3.5c are reportedly as follows for class F and class TGM:

EE2-3.5a:

AI:  Class F -0.25%, 100%,   99%; Class TGM -0.10%, 100%,   96%

RA: Class F -0.10%, 103%, 100%; Class TGM -0.17%, 103%, 100%

EE2-3.5b:

AI:  Class F -0.26%, 102%,   99%; Class TGM -0.15%, 100%,    97%

RA: Class F -0.13%, 103%, 100%; Class TGM -0.23%, 101%, 100%

EE2-3.5c:

AI:  Class F -0.27%, 101%, 100%; Class TGM -0.18%, 100%,   95%

RA: Class F -0.13%, 103%, 100%; Class TGM -0.26%, 103%, 100%

# Introduction

Modifications of IBC merge/AMVP list construction was proposed in JVET-Y0058 [2] and adopted in ECM., which removes the invalid block vector (BV) candidates from the IBC merge/AMVP candidate list.

In the current ECM design, when the IBC Merge/AMVP list is not complete with the spatial, HMVP, and pairwise candidates, the list is padded with one or more zero vectors until the maximum number of candidates is encountered. However, in contrast to the inter prediction, a null candidate is invalid as a block vector predictor for the current block in IBC merge mode.

# Proposed methods

In EE2 [3], Test 3.5a studies the clipping of the BVP candidates pointing outside the IBC reference region before being validated for inclusion in the IBC Merge/AMVP list. The method proposes the clipping of one or both components of an invalid candidate to the nearest IBC buffer boundaries, as is shown in Figure 1, where BVP* represents the clipped BVP candidate:

Figure 1. Clipping of the BVP candidates at the IBC buffer boundaries

A second method is studied in the EE2-3.5b test, which replaces the zero vectors’ candidates to pad the IBC Merge/AMVP list with a set of BVP candidates located in the IBC reference region. A zero vector is invalid as a block vector in IBC merge mode, and consequently, it is discarded as BVP in the IBC candidate list.

Three candidates are located on the nearest corners of the reference region, and three additional candidates are determined in the middle of the three sub-regions (A, B, and C), whose coordinates are determined by the width, and height of the current block and the ΔX and ΔY parameters, as is depicted in Figure 2.

Test 3.5c in [3] studies the combination of both proposed methods, the clipping of the invalid BVP candidates and the replacement of the invalid zero-padding candidates in the IBC Merge/AMVP list.

Figure 2. Padding candidates for the replacement of the zero-vector in the IBC list

# Experimental Results

The proposed methods have been implemented on top of the ECM-4.0 [4]. Test results 3.5a, 3.5b, and 3.5c under AI/RA/LB configurations are summarized in Table 1, Table 2, and Table 3, respectively. The simulations were performed following the common test conditions [5]. More detailed results can be found in the attached spreadsheets.

Table 1: Test 3.5a. Candidates’ VBPs clipping.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.25% | -0.06% | -0.14% | 100% | 99% |
| Class TGM | -0.10% | -0.09% | -0.07% | 100% | 96% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.10% | -0.38% | -0.25% | 103% | 100% |
| Class TGM | -0.17% | -0.12% | -0.12% | 103% | 101% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

Table 2: Test 3.5b. Candidates’ list padding.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.26% | -0.06% | -0.11% | 102% | 99% |
| Class TGM | -0.15% | -0.09% | -0.06% | 100% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.13% | -0.32% | -0.28% | 103% | 100% |
| Class TGM | -0.23% | -0.21% | -0.19% | 101% | 100% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

Table 2: Test 3.5c. Combined clipping and padding proposals.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.27% | 0.00% | -0.10% | 101% | 100% |
| Class TGM | -0.18% | -0.09% | -0.14% | 100% | 95% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.13% | -0.36% | -0.28% | 103% | 100% |
| Class TGM | -0.26% | -0.21% | -0.24% | 103% | 100% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

- Conclusions

This contribution proposes the IBC Merge/AMVP candidates clipping method and the replacement of the invalid zero-padding predictors with valid predictors’ candidates. The test results report that both individual and the combined methods can improve the performance of IBC on top of ECM-4.0 for AI/RA/LB configurations with a slight increase in the encoding time. We recommend to adopt these tools into the next version of the ECM software.

# References

- D. Ruiz Coll, A. Filippov, V. Rufitskiy, T. Meon Bae, “EE2-related: BVP candidate adjustment based on IBC reference region implemented on top of test EE2-3.13.” JVET-Y0133, January 2022.

- Na Zhang, Kai Zhang, Li Zhang, Jizheng Xu, “EE2-3.13: Modifications of IBC Merge/AMVP List Construction.” JVET-Y0058, January 2022.

- Vadim Seregin, Jie Chen, Ling Li, Karam Naser, Jacob Ström, Martin Winken, Xiaoyu Xiu, Kai Zhang “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2).” JVET-Y2024, January 2021.

- https://vcgit.hhi.fraunhofer.de/ecm/jvet-y-ee2/ECM

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, January 2022.

# Patent rights declaration(s)

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).