| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0174-v1 |
| --- | --- |

| Title: | Cross-check of JVET-Z0049 (EE2-1.1: Slope adjustment for CCLM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Luhang Xu | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking of contribution JVET-Z0049[1], in which a slope adjustment is added to cross-component linear model (CCLM) prediction. The adjustment is tilting or rotating the linear function which maps luma values to chroma values with respect to a center point determined by the average luma value of the reference samples as proposed originally in JVET-Y0055.

The proposed code has been tested in AI and RA configurations under CTCs[2]. The obtained results match the proponent provided.

# Simulation results

The tests are conducted under CTC and results are confirmed to be identical to those reported in JVET-Z0049.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -1.74% | -1.73% | 107% | 103% |
| Class A2 | -0.05% | -1.25% | -0.61% | 105% | 101% |
| Class B | -0.04% | -1.47% | -1.60% | 104% | 99% |
| Class C | -0.05% | -0.96% | -0.89% | 105% | 98% |
| Class E | -0.03% | -0.23% | -0.21% | 103% | 95% |
| Overall | -0.04% | -1.16% | -1.07% | 105% | 99% |
| Class D | -0.04% | -0.68% | -0.71% | 107% | 105% |
| Class F | -0.08% | -0.69% | -1.06% | 104% | 94% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.47% | -0.73% | 109% | 102% |
| Class A2 | -0.02% | -0.63% | -0.28% | 109% | 101% |
| Class B | -0.03% | -0.72% | -0.61% | 111% | 106% |
| Class C | -0.04% | -0.20% | -0.14% | 110% | 109% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.51% | -0.44% | 110% | 105% |
| Class D | 0.02% | -0.42% | -0.55% | 111% | 107% |
| Class F | 0.01% | -0.54% | -0.58% | 110% | 106% |

(Please kindly note that running time in this contribution is not reliable.)

# Conclusion

Implementation and simulation results of JVET-Z0049 have been confirmed.

# Reference

[1] J. Lainema, A. Aminlou, P. Astola, R. G. Youvalari, “EE2-1.1: Slope adjustment for CCLM”, Apr. 2022.

[2] M. Karczewicz, Y. Ye	, “Common Test Conditions and evaluation procedures for enhanced compression tool testing	”, Apr. 2022.