| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0186-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0086 (EE1-1.4: ALF improvement for NNVC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chaoyi Lin Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China | Tel: Email: | linchaoyi.cy@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution is the cross-check of contribution JVET-Z0086. The simulations results under all intra (AI) configuration and random access (RA) configuration are matched with the results reported in JVET-Z0086, although, results under show small deviations. This deviations may due to the floating point of the neural network parameters.

# Introduction

Simulations with the code provided by JVET-Z0086 [1] are performed according to the common test conditions defined in [2]. Class A1 and A2 is not cross checked for RA configuration of EE1-1.4.2.

In this cross-check, while small deviations from an exact match are found, no significant differences have been found. The neural network in JVETZ0086 is implemented using floating point arithmetic. Hence, the results may be different due to the implementation details, such as PyTorch/gcc version.

# Results

Results of test EE1-1.4.1 are shown in Table 1, where anchor results are provided in JVET-Z0086.

Table 1 Cross check results of EE1-1.4.1

|  | RA |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0.01% | 0.07% | -0.01% | 0.00% | 0.01% | 0.00% |
| Class A2 | 0.05% | -0.05% | -0.03% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.03% | -0.06% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | -0.11% | 0.01% | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  | 0.00% | 0.00% | 0.00% |
| Overall | 0.01% | -0.02% | -0.03% | 0.00% | 0.00% | 0.00% |
| Class D | 0.02% | -0.10% | -0.02% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.01% | 0.01% | 0.00% | 0.00% | 0.00% |

Partial results of test EE1-1.4.2 are shown in Table 2, where anchor results are provided in JVET-Z0086.

Table 2 Cross check results of test 1.4.2

|  | RA |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | 0.00% | 0.01% | 0.00% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | 0.00% | 0.00% | 0.00% |
| Class B | -0.01% | 0.10% | -0.06% | 0.00% | 0.00% | 0.00% |
| Class C | -0.02% | -0.01% | -0.13% | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  | 0.00% | 0.00% | 0.00% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | -0.11% | 0.01% | 0.00% | 0.00% | 0.00% |
| Class F | 0.02% | 0.05% | 0.00% | 0.00% | 0.00% | 0.00% |

# References

- W. zou, Y. Zhou, C. Huang, Y. X. Bai, “EE1-1.4: ALF improvement for NNVC”, JVET-Z0086

- Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-Y2016, S. Liu, A. Segall, E. Alshina, R.-L. Liao