| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0198 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0165 (EE2-3.6: Combination tests of EE2-3.2+EE2-3.3+EE2-3.4+EE2-3.5): Tests EE2-3.6a, EE2-3.6b, EE2-3.6e and EE2-3.6h) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Damian Ruiz Coll | Tel: Email: | druizcoll@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of Tests EE2-3.6a, EE2-3.6b, EE2-3.6e, and EE2-3.6h, which are the combination of three tools for the IBC [1]. These tools are described in tests 3.2, 3.3, and 3.4, which propose extending the IBC reference area for intra-block copy (IBC) up to 3 CTU-rows, extending the HMVP table up to 25 candidates in the IBC merge/AMVP list, and the IBC candidates’ refinement using the template matching technique.

The cross-checked source code follows the description in the proposal, and the results are matched with the results provided by the proponents.

# Introduction

In EE2, several tools have been proposed for IBC. Three of these tests are:

- EE2-3.2: it proposed an extended IBC reference region up to two CTU rows above in addition to the CTU-row of the current CTU. Figure 1 illustrates the reference area for coding CTU (m,n). Specifically, for CTU (m,n) to be coded, the reference area includes CTUs with index (m–2,n–2)…(W,n–2),(0,n–1)…(W,n–1),(0,n)…(m,n), where W denotes the maximum horizontal index within the current tile, slice or picture two methods to adjust the BVP candidates.

- Figure 1. Figure 1. Reference area for IBC when CTU (m,n) is coded. The blue block denotes the current CTU; the green blocks denote the reference area, and the white blocks denote the invalid reference area.

- EE2-3.3: it is proposed to increase the HMVP table size for IBC to 25. After up to 20 IBC merge candidates are derived with full pruning, they are reordered together. After reordering, the first 6 candidates with the lowest template matching costs are selected as the final candidates in the IBC merge list.

- EE2-3.4: In this proposal, the IBC-TM merge list has been modified compared to the one used by the regular IBC merge mode such that the candidates are selected according to a pruning method with a motion distance between the candidates as in the regular TM merge mode.

The ending zero motion fulfillment (which is nonsense regarding Intra coding) is removed, and three new candidates are always introduced into the IBC Merge/AMVP list as BVP to the left (-W, 0), top (0, -H), and top-left (-W, -H), where W is the width and H the height of the current CU.

In the IBC-TM merge mode, the selected candidates are refined with the Template Matching method prior to the RDO or decoding process. The IBC-TM merge mode has been put in competition with the regular IBC merge mode and a TM-merge flag is signaled.

In the IBC-TM AMVP mode, up to 3 candidates are selected from the IBC-TM merge list. Each of those 3 selected candidates is refined using the Template Matching method and sorted according to their resulting Template Matching cost. Only the 2 first ones are then considered in the motion estimation process as usual.

Cross-check of the source code and the simulation results have been conducted. Inspected source code follows the description in the proposal. The simulation is performed on top of ECM-4.0. The cross-check results are provided in the tables below and match the results provided by the proponents. Note that coding time is not accurate.

Below is the result for the EE2-3.6a test:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -4.13% | -3.71% | -3.80% | 111% | 96% |
| Class TGM | -9.06% | -8.33% | -8.30% | 103% | 84% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -2.56% | -2.67% | -2.74% | 105% | 99% |
| Class TGM | -6.17% | -5.89% | -6.09% | 103% | 95% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.59% | -1.09% | -2.14% | 105% | 100% |
| Class TGM | -5.00% | -4.78% | -4.94% | 100% | 90% |

It is confirmed that the obtained results perfectly match the BD-rate gains reported in EE2. Obtained runtimes relative to anchor are in the same range as those reported in EE2.

Below is the result for the EE2-3.6b test:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -4.51% | -4.29% | -4.28% | 115% | 97% |
| Class TGM | -8.53% | -7.87% | -7.85% | 106% | 92% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -2.83% | -2.84% | -2.91% | 106% | 100% |
| Class TGM | -5.60% | -5.30% | -5.45% | 103% | 97% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.92% | -1.05% | -2.82% | 109% | 100% |
| Class TGM | -4.45% | -3.98% | -4.18% | 106% | 94% |

It is confirmed that the obtained results perfectly match the BD-rate gains reported in EE2. Obtained runtimes relative to anchor are in the same range as those reported in EE2.

Below is the result for the EE2-3.6e test:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.69% | -0.44% | -0.61% | 107% | 104% |
| Class TGM | -1.68% | -1.42% | -1.41% | 106% | 112% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.62% | -0.76% | -0.63% | 104% | 100% |
| Class TGM | -1.50% | -1.44% | -1.39% | 103% | 103% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.76% | 0.67% | -0.42% | 105% | 101% |
| Class TGM | -1.21% | -1.08% | -1.12% | 104% | 100% |

It is confirmed that the obtained results perfectly match the BD-rate gains reported in EE2. Obtained runtimes relative to anchor are in the same range as those reported in EE2.

Below is the result for the EE2-3.6h test:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -4.63% | -4.18% | -4.40% | 117% | 99% |
| Class TGM | -9.60% | -8.80% | -8.77% | 108% | 90% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -3.01% | -3.27% | -2.96% | 109% | 99% |
| Class TGM | -6.62% | -6.33% | -6.47% | 105% | 96% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -2.08% | -1.30% | -2.55% | 111% | 102% |
| Class TGM | -5.35% | -5.10% | -5.44% | 105% | 94% |

It is confirmed that the obtained results perfectly match the BD-rate gains reported in EE2. Obtained runtimes relative to anchor are in the same range as those reported in EE2.

# Conclusion

Results of EE2 tests 3.6a, 3.6b, 3.6e, and 3.6h have been confirmed.

# Reference

[1] Na Zhang, Jizheng Xu, Kai Zhang, Li Zhang A. Robert, K. Naser, T. Poirier, Y. Chen, F. Galpin, D. Ruiz Coll, A. Filippov, V. Rufitskiy, “EE2-3.6: Combination tests of EE2-3.2+EE2-3.3+EE2-3.4+EE2-3.5”, JVET-Z0165, April 2022