| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26nd Meeting, by teleconference, 20–29 March 2022 | Document: JVET-Z0205 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0047 (AHG13: Improvements of film grain analysis) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Fangjun Pu 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | +1 408 330 3281 fpu@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results for contribution JVET-Z0047 on improvements of film grain analysis on base of VTM-15.2. The implementation of improved analysis algorithm was confirmed. The cross-checking used the same sequences tested in the contribution. Two cases were tested: 1) internal MCTF + Canny + Morphology method; 2) external input denoised image (VTM reconstructed yuv with QP22 and QP26). The results show that proposed FG analysis method provides better visual effect with more reasonable grain strength and distribution for most of the sequences. In addition, external input denoised image (reconstructed frame) generates slightly higher amplitude of scaling factors compared with MCTF + Canny method. Visually it is still better than the original film grain analysis approach.

# Introduction

JVET-Z0047 [1] proposes an enhanced VTM software implementation focusing on the film grain analysis and parameter estimation module at the encoder side. It describes a method to automatically estimate intensity intervals and associated scaling factors used in the film grain synthesis process with higher accuracy. One bug-fix related to the number of intensity intervals and further improvement of use of morphological operations in the mask derivation process were introduced in the contribution.

The software is available within the proposal package (file source_FG_analysis_improvts.zip). Proposed changes are implemented on top of the VTM-15.2. Software usage remains unchanged as previous version. To enable the proposed film grain analysis improvements, macro FG_IMPROVEMENT is set to 1.

The default configuration film_grain_characteristics_test_analysis.cfg is used to provide the encoder with additional parameters. Two new options are added in the configuration parameters:

--SEIFGCExternalDenoised=path/denoised_sequence_name.yuv

--SEIFGCExternalMask= path/mask_sequence_name.yuv

These new options provide means to use any denoising algorithm and any mask (for flat blocks detection) calculation algorithm. If those parameters are provided, VTM will use external sources for film grain analysis part. If the parameter is not provided (empty string by default) VTM will use MCTF and Canny edge detector in conjunction with morphological operations (MCTF and Canny for film grain analysis are already part of VTM).

# Experimental Results

Cross-checking uses the JVET SDR-CTC testset [2] and internal test sequences including: AutoWelding, bikeSparklers, crowd_run, Market3, ShowGirl, MagicHourFountainToTable, WarmNightTorchTotable, InToTree, OldTownCross.

The results show that the film grain parameters especially the scaling factor derived by original analysis method are too aggressive for some sequences (Figure 1.b). Proposed method provides better visual effect with more reasonable grain strength and distribution for most of the sequences (Figure 1.c).

In addition, external input denoised image (reconstructed frame) generates slightly higher amplitude of scaling factors compared with default MCTF + Canny method. Visually it still gives better visual effect than the original film grain analysis approach (Figure 1.d).

Figure 1.a. Decoded frame with QP32 of CrowdRun

Figure 1.b. Synthesized frame with parameters derived from FGA in VTM-15.2

Figure 1.c. Synthesized frame with parameters derived from proposed MCTF+Canny method

Figure 1.d. Synthesized frame with parameters derived from external denoised image (rec QP22)

# Reference

- P. de Lagrange, E. Francois, Z. Ameur, M. Radosavljević, AHG13: Improvements of film grain analysis”, JVET-Z0047, Teleconference, 20–29 March 2022.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Teleconference, 7–16 Oct. 2020.