| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0208-v2 |
| --- | --- |

| Title: | Cross-check of JVET-Z0136 (EE2-Test2.2a: Enhanced bi-directional motion compensation) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Antoine Robert | Email: | antoine.robert@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking of JVET-Z0136 (EE2-Test2.2a: Enhanced bi-directional motion compensation). JVET-Z0136 test 2.2a proposes to consider only the samples that are not Out of Boundary (OOB) to generate the bi-directional motion compensation even if BCW applies.

It is reported that the cross-check confirms the results.

# Introduction

Since the OOB reference samples are generated through the padding process, the motion compensation (MC) predictors generated using the OOB reference samples may be less effective. In this test, when combining more than one prediction blocks generated by the motion compensation process, the OOB predictors are discarded and only the non-OOB predictors are used to generate the final predictor.

To be specific, the positions of the predictors within the current block are denoted as Pos_xi,j and Pos_yi,j, the MV of the current block are denoted as Mv_xi,jLx and Mv_yi,jLx (x could be 0 or 1 for list 0 and list1, respectively). PosLeftBdry, PosRightBdry, PosTopBdry, PosBottomBdry are the positions of four boundaries of a picture. Since 1/16-pel MV is used in the ECM, all variables are denoted in the unit of the 1/16 sample and thus the value of half_pixel is set equal to 8.

The predictor Pi,jLx is regarded as OOB when at least one of the following conditions holds:

(Pos_xi,j+Mv_xi,jLx) > (PosRightBdry + half_pixel),

(Pos_xi,j+Mv_xi,jLx) < (PosLeftBdry - half_pixel),

(Pos_yi,j+Mv_yi,jLx) > (PosBottomBdry + half_pixel),

or (Pos_yi,j+Mv_yi,jLx) < (PosTopBdry - half_pixel)

otherwise, when none of the above conditions holds, the predictor Pi,jLx is regarded as non-OOB.

After the OOB condition is determined for each predictor, the proposed scheme is applied to the bi-directional MC blocks to generate the final predictors where the following procedures are deployed. Please be noted that the same checking mechanism is also applied when BCW is enabled. For those predictors which are modified from bi-directional to uni-directional, the BDOF offsets are discarded since they are no more bi-directional predicted.

If Pi,jL0 is OOB and Pi,jL1 is non-OOB

Pi,jfinal=Pi,jL1

else if Pi,jL0 is non-OOB and Pi,jL1 is OOB

Pi,jfinal=Pi,jL0

else

Pi,jfinal=(Pi,jL0+Pi,jL1+1)>>1

# Simulation results

The proposal has been tested under ECM-4.0 CTCs and the simulation results are presented in Table 1.

Table 1: Proposed JVET-Z0136 test 2.2a results on top of ECM-4.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.20% | -0.02% | 102% | 103% |
| Class A2 | -0.15% | -0.20% | -0.12% | 102% | 103% |
| Class B | -0.24% | -0.12% | -0.18% | 103% | 104% |
| Class C | -0.29% | -0.16% | -0.13% | 102% | 103% |
| Overall | -0.22% | -0.16% | -0.12% | 102% | 103% |
| Class D | -0.79% | -0.41% | -0.54% | 101% | 103% |
| Class F (optional) | -0.04% | -0.04% | -0.17% | 103% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.02% | -0.13% | -0.16% | 102% | 102% |
| Class C | -0.07% | 0.04% | -0.10% | 101% | 102% |
| Class E | -0.01% | -0.25% | 0.24% | 101% | 101% |
| Overall | -0.03% | -0.10% | -0.04% | 101% | 102% |
| Class D | -0.23% | 0.62% | -0.06% | 101% | 102% |
| Class F (optional) | -0.07% | 0.57% | 0.06% | 101% | 102% |

It is confirmed that the obtained results perfectly match the BD-rate gains reported in JVET-Z0136. Obtained runtimes relative to anchor are in the same range as those reported in JVET-Z0136.

# Conclusion

Results of JVET-Z0136 test 2.2a have been confirmed.

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.