| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0231-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0069 (AHG12: Longer chroma filters for RPR in ECM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Z. Zhang | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of crosschecking JVET-Z0069 proposed by Ericsson. JVET-Z0069 proposes to increase the chroma interpolation filter for RPR to 6-tap or 8-tap. It proposes to align the chroma filter design for RPR picture and non-RPR picture, and for non-RPR picture the 6-tap chroma filter is proposed in JVET-Z0068 [4]. This contribution reports the crosscheck results of 6-tap chroma interpolation filter.

The update of the code had been reviewed and confirmed consistent with the proposal description.

The verification simulation was run for RA configuration with RPR enabled. The verification simulation results matched the results in JVET-Z0069 in terms of BD-rate.

# Verification

Ericsson had provided the software and results of their proposed methods in JVET-Z0069[1]. The code was updated according to the description in JVET-Z0069.

The simulation was conducted under the common test conditions as defined in [2] with RPR enabled. The anchor is ECM-4.0 with RPR enabling fix in JVET-Z0062[3]. This crosscheck reports the results of 6-tap chroma interpolation filter for RPR picture in JVET-Z0069 and 6-tap chroma interpolation filter for non-RPR picture in JVET-Z0068.

Two RPR configuration setting are tested:

| RPR setting 1.5x | RPR setting 2.0x |
| --- | --- |
| (Resolution switch after every 16 frames) | (Resolution switch after every 16 frames) |
| ScalingRatioHor : 1.5 | ScalingRatioHor : 2.0 |
| ScalingRatioVer : 1.5 | ScalingRatioVer : 2.0 |
| FractionNumFrames : 1.0 | FractionNumFrames : 1.0 |
| RPR : 1 | RPR : 1 |
| UpscaledOutput : 2 | UpscaledOutput : 2 |
| SwitchPocPeriod : 16 | SwitchPocPeriod : 16 |

The BD-rate results match the results reported in JVET-Z0069. The simulation results for random access configuration are as follows:

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 with RPR enabled |  |  |  |  |  |  |  |  |  |
|  | RPR setting 1.5x |  |  |  |  | RPR setting 2.0x |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.09% | -4.61% | -4.52% | 98% | 104% | -0.07% | -6.62% | -6.48% | 98% | 104% |
| Class C | -0.09% | -6.40% | -5.61% | 99% | 105% | -0.07% | -7.88% | -7.34% | 99% | 104% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.08% | -7.41% | -8.24% | 99% | 104% | -0.06% | -8.68% | -9.90% | 99% | 105% |

# Reference

[1] K. Andersson, R. Yu (Ericsson), “AHG12: Longer chroma filters for RPR in ECM”, JVET-Z0069, Apr, 2022.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

[3] R. Yu, K. Andersson (Ericsson), “AHG12: RPR luma filter modifications and RPR enabling fixes in ECM”, JVET-Z0062, Apr, 2022.

[4] K. Andersson, R. Yu (Ericsson), “EE2-2.6 related: Longer chroma interpolation filters”, JVET-Z0068, Apr 2022.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).