| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0241-v1 |
| --- | --- |

| Title: | Cross check of JVET-Z0118 (AHG7: GDR implementation for ECM 4.0) |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra | Email: | akotra@qti.qualcomm.com |
| Source: | Qualcomm Inc., |  |  |

_____________________________

# Abstract

This document reports the crosscheck results of JVET-Z0118 (AHG7: GDR implementation for ECM 4.0).
The partial results obtained using the source code provided by the proponent match with the proponent’s results.  The cross checker mainly studied the part of the contribution which deals with the virtual boundary (VB) processing for the new in-loop filter tools of ECM. The source code generally matches to the description however few cases are reported where few virtual boundaries may not be handled correctly.

# Introduction

Contribution JVET-Z0118 [5] proposes a GDR implementation for ECM-4.0. JVET-Y0163 [1] of the previous teleconference meeting also provided a GDR implementation on top of ECM 3.0. However, in JVET-Y0163, the following loop filter tools were turned off as the respective tools did not have “virtual boundary” processing enabled:

- ALF_IMPROVEMENT

- JVET_V0094_BILATERAL_FILTER

- JVET_X0071_CHROMA_BILATERAL_FILTER

- JVET_W0066_CCSAO

- JVET_X0071_LONGER_CCALF

- JVET_X0071_ALF_BAND_CLASSIFIER

JVET-Z0118 [3] provides a GDR implementation on top of ECM-4.0 where the VB processing for all the above-mentioned loop filter tools is also enabled.

# Simulation results

The simulation experiments are conducted using the test conditions specified in LLCC AhG [3]. Table 1 below shows the BD-Rate impact. The partial results obtained using the source code provided by the proponent match with the proponent’s results

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 1.50% | 3.04% | 3.35% | 105% | 109% |
| Class E | 5.87% | 9.76% | 10.79% | 110% | 117% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 2.31% | 5.10% | 6.37% | 104% | 110% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 1. Simulation Results over ECM-4.0

# Source code analysis

It is reported that the Luma Bilateral filter and the Chroma Bilateral filter VB do not handle the case when the virtual boundary location is in between the start and end location of a given Transform unit/Coding unit. As we can observe in the code snippet below, the starting position of the luma position of the TU and the ending Luma position of the TU (startposX +Width) or (startPosY + Height) locations are only checked with the VB position. 

      if (picHeader->getVirtualBoundariesPresentFlag())

{

for (int i = 0; i < picHeader->getNumHorVirtualBoundaries(); i++)

{

if (lumaY == picHeader->getVirtualBoundariesPosY(i))

{

topAltAvailable = false;

}

if ((lumaY + lumaHeight) == picHeader->getVirtualBoundariesPosY(i))

{

bottomAltAvailable = false;

}

}

for (int i = 0; i < picHeader->getNumVerVirtualBoundaries(); i++)

{

if (lumaX == picHeader->getVirtualBoundariesPosX(i))

{

leftAltAvailable = false;

}

if ((lumaX + lumaWidth) == picHeader->getVirtualBoundariesPosX(i))

{

rightAltAvailable = false;

}

}

}
                  Table 2. Bilateral Filter Virtual Boundary Handling

For SAO and CCSAO VB processing, only the vertical virtual boundaries are checked. The code snippet below shows one such example.

#if GDR_ENABLED

if (isCtuCrossedByVirtualBoundaries && isProcessDisabled(x, y, numVerVirBndry, 0, verVirBndryPos, horVirBndryPos))

{

signUpLine[x - 1] = -signDown;

continue;

}

#endif
 Table 3. SAO and CCSAO VB handling always use numHorVirBndry value as 0 (highlighted in yellow)

It is reported that the proposed VB solution might be sufficient for a GDR implementation which only uses vertical virtual boundaries, but the VB solution may not be sufficient to handle other use cases which also need horizontal virtual boundary processing.

# References

- L. Wang, S. Hong, K. Panusopone, M. Hannuksela, and J. Lainema, “AHG7: GDR without encoder constraints for ECM”, JVET-Y0163, January 2022.

- ECM-4.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-4.0.

- G. Martin-Cocher, S. Puri, T. Poirier, K. Naser, J. Xu, D. Nicholson, M. Sychev, L. Wang, S. Liu, W. Yang, J.M. Tiesse, and M. Karczewicz, “AHG-7, LLCC Scenarios and baseline configurations”, JVET -Y0043, Jan. 2022.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, January 2022.

- S. Hong et al. AHG7: GDR Implementation for ECM 4.0, JVET-Z0118, April 2022.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).