| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting: by teleconference, 13–22 July 2022 | Document: JVET-AA0003-v1 |
| --- | --- |

| Title: | AHG report: Test model software development (AHG3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Frank Bossen Xiang Li Karsten Suehring (co-chairs) Yuwen He Karl Sharman Vadim Seregin Alexis Tourapis (vice chairs) | Email: | frank@bossentech.com xlxiangli@google.com karsten.suehring@hhi.fraunhofer.de yuwen.he@bytedance.com Karl.Sharman@sony.com vseregin@qti.qualcomm.com atourapis@apple.com |
| Source: | AhG3 on Test model software development |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AhG3 on Test model software development that has taken place between the 26th and 27th JVET meetings.

# Introduction

The mandates given to the AHG are:

- Coordinate development of test model (VTM, HM, SCM, SHM, HTM, MFC, MFCD, JM, JSVM, JMVM, 3DV-ATM, 360Lib, and HDRTools) software and associated configuration files.

- Produce documentation of software usage for distribution with the software.

- Enable software support for recently standardized additional SEI messages.

- Discuss and make recommendations on the software development process.

- Propose improvements to the guideline document for developments of the test model software.

- Perform comparative tests of test model behaviour using common test conditions.

- Suggest configuration files for additional testing of tools.

- Investigate how to minimize the number of separate codebases maintained for group reference software.

- Coordinate with AHG on Draft text and test model algorithm description editing (AHG2) to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate.

- Prepare drafts of merged CTC documents for HM and VTM, as applicable.

The software model versions prior to the start of the meeting were:

- VTM 17.0 (Jun. 2022)

- HM-16.25 (Apr. 2022)

- HM-16.21+SCM-8.8 (Mar. 2020)

- SHM 12.4 (Jan. 2018)

- HTM 16.3 (Jul. 2018)

- JM 19.0

- JSVM 9.19.15

- JMVC 8.5

- 3DV ATM 15.0 (no version history)

- HDRTools 0.23 (October 2021)

Software for MFC and MFCD is only available as published by ITU-T and ISO/IEC. It is planned to create repositories with the latest versions available in ITU-T H.264.2 (02/2016). All development history is lost.

# Software development

Development was continued on the GitLab server, which allows participants to register accounts and use a distributed development workflow based on git.

The server is located at:

https://vcgit.hhi.fraunhofer.de

The registration and development workflow are documented at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/VVC-Software-Development-Workflow

Although the development process is described in the context of the VTM software, it can be applied to all other software projects hosted on the GitLab server as well.

# VTM related activities

The VTM software can be found at

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

The software development continued on the GitLab server. VTM versions 16.1 and 16.2 were tagged on May 3, and VTM version 17.0 was tagged on Jun. 3. VTM 17.1 is expected during the 27th JVET meeting.

VTM 16.1 was tagged on May 3, 2022. Changes include:

- Fix: Check the value of NoOutputBeforeRecoveryFlag prior to marking no output

- JVET-Y0072: Unlimited level support for all video profiles

- JVET-S0078: No output of prior pics flag

- Avoid potential buffer overflow in illegal bitstream

- Fix #1546: Tune range for m_cpbRemovalDelayDelta

- Change video_decoding_interface to VDI_sei_envelope

- JVET-Y0044: Signal Green metadata and VDI SEI messages

- Fix #1542: Correct QPA rate allocation for bit depths other than 10

- Add checks for activated APS ALF flags

- Fix #1543: BIM index out of bounds error

- Cleanup PU::getIntraMPMs and add const qualifiers

- Remove unused field

- Remove commented defaults and unnecessary nullptr cast

- Use nullptr instead of NULL

- Remove unnecessary braces

- Remove unused code, fix braces

- Fix #1541

VTM 16.2 was tagged on May 3, 2022. Changes include:

- Remove macros from previous cycle

VTM 17.0 was tagged Jun. 3, 2022. Changes include:

- Slightly improved readability for fix #1552

- Fix range check of vps_num_ptls_minus1 when it is not signaled

- JVET-Z0244: NN post-filter SEI

- Revert unnecessary includes from !2225

- JVET-Z0120: SII SEI support and illustration of use case "Backwards-compatible HFR video"

- Allocate large buffer on heap instead of stack

- Fix allocation size for VPS olsHrdParams

- Remove variable that is never read

- Remove old HEVC code and clean up related functions

- JVET-Z0046: Green Metadata SEI

- Clean up weighted pred code

- Clean up

- JVET-Z0099: enabling deblocking in RDO and adjusting tc and beta offset for random access and low delay fix

- JVET-Z0072: enhanced referencing configs for random access and low delay

- Clean up variable names, spacing, and a few other things

- Clean up variable naming

- JVET-Z0111: Adaptive bypassing affine ME (default off)

VTM 17.1 is expected to be tagged during the 27th JVET meeting. Changes include so far:

- Remove unnecessary mrgTypeNeighbours field

- JVET-Z0047: Improved film grain analysis

- Fix which ptl_mulltlayer_enabled_flag is used

- Fix setNumOutputLayerSets() to accept value 256

- Use chromaFormatIDC stored inside Picture

- Fix: lintf undef

- Avoid floating-point operations when computing HAD

- Clean up variable names, indentation, braces

- Rename BcwIdx to bcwIdx, indentation fixes

- Fix #1557: change dependencies for writing ccv_max_luminance_value and ccv_avg_luminance_value

- Fix #1551: check coefficient range

- Add missing braces

- Avoid using std::vector and use static_vector instead

- Fix variable names

- Avoid goto in SEIFilmGrainAnalyzer.cpp

- Reduce size of temporary memory for deblocking

- Precalculate fixed-point weights for MSE

- Y4M support at both encoder and decoder (#206)

- Fix variable names

- Fix variable names

- Consistently use helper functions and define them inline

- Don't use reference for simple bool parameters

- Clean up InterPrediction::xPredAffineBlk and associated functions

- Enable inlining of simple PelStorage methods

- Replace #define for constants in interpolation filter with constexpr

- Use constexpr instead of const for compile time constants

- Branchless version of Area::contains

- Clean up filter management for MC interpolation

- Clean up SIMD interpolation filter code and add support of horizontal 6-tap filter

- Fix for PQ when EncDbOpt is used

## CTC Performance

The following tables still show VTM 15.0 performance over HM 16.25 for a fair comparison with aligned tool configurations. The upcoming version HM 16.26 is expected to be aligned with VTM 17.0:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.25 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -29.03% | -32.17% | -34.07% | 1564% | 171% |
| Class A2 | -29.29% | -23.92% | -21.06% | 2483% | 183% |
| Class B | -21.73% | -26.96% | -30.76% | 2731% | 182% |
| Class C | -22.54% | -18.95% | -22.70% | 3923% | 187% |
| Class E | -25.75% | -25.91% | -24.45% | 2209% | 171% |
| Overall | -25.06% | -25.37% | -26.85% | 2563% | 179% |
| Class D | -18.46% | -13.31% | -13.41% | 4463% | 167% |
| Class F | -39.33% | -39.73% | -42.22% | 5299% | 171% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over HM-16.25 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -39.85% | -39.48% | -46.15% | 670% | 164% |
| Class A2 | -43.19% | -40.54% | -39.68% | 746% | 178% |
| Class B | -36.30% | -48.60% | -47.20% | 740% | 166% |
| Class C | -33.16% | -34.83% | -36.95% | 997% | 168% |
| Class E |  |  |  |  |  |
| Overall | -37.55% | -41.49% | -42.75% | 787% | 169% |
| Class D | -31.45% | -31.40% | -31.26% | 1125% | 165% |
| Class F | -45.76% | -49.18% | -50.10% | 580% | 147% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over HM-16.25 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -29.17% | -34.97% | -32.34% | 761% | 166% |
| Class C | -25.87% | -17.39% | -17.85% | 910% | 164% |
| Class E | -28.93% | -33.29% | -26.46% | 371% | 136% |
| Overall | -28.01% | -28.69% | -26.04% | 675% | 157% |
| Class D | -25.08% | -12.54% | -12.25% | 961% | 168% |
| Class F | -40.20% | -41.56% | -41.87% | 509% | 136% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over HM-16.25 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -33.74% | -37.63% | -34.94% | 697% | 175% |
| Class C | -27.71% | -17.50% | -17.86% | 831% | 175% |
| Class E | -32.18% | -37.21% | -31.09% | 358% | 147% |
| Overall | -31.34% | -30.82% | -28.28% | 626% | 167% |
| Class D | -26.32% | -11.70% | -10.99% | 876% | 170% |
| Class F | -39.97% | -41.10% | -41.48% | 542% | 143% |

According to common test conditions in random access configuration HM is using a GOP size of 16 pictures compared to VTM using a GOP of 32 pictures. Random access points are inserted approximately every second aligned with a GOP boundary of GOP 32 in both VTM and HM. VTM uses two more reference pictures in random access than HM (due to more memory being availably in typical level settings).

The following tables show VTM 17.0 performance compared to VTM 16.0:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-16.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 96% |
| Class B | 0.00% | 0.00% | 0.00% | 93% | 94% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class E | 0.00% | 0.00% | 0.00% | 103% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 96% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 97% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-16.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.06% | -1.76% | -1.63% | 111% | 98% |
| Class A2 | -0.95% | -1.45% | -1.31% | 111% | 102% |
| Class B | -1.19% | -1.84% | -1.48% | 110% | 100% |
| Class C | -1.04% | -0.89% | -1.17% | 108% | 96% |
| Class E |  |  |  |  |  |
| Overall | -1.08% | -1.49% | -1.39% | 110% | 99% |
| Class D | -0.68% | -0.94% | -1.03% | 109% | 99% |
| Class F | -0.49% | -0.09% | 0.11% | 106% | 96% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-16.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.23% | -3.52% | -2.93% | 105% | 101% |
| Class C | -2.88% | -4.32% | -3.98% | 106% | 100% |
| Class E | -2.30% | -4.18% | -5.17% | 95% | 88% |
| Overall | -2.05% | -3.95% | -3.84% | 103% | 97% |
| Class D | -2.36% | -4.65% | -4.65% | 105% | 98% |
| Class F | -2.84% | -3.01% | -2.90% | 100% | 94% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-16.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.67% | -0.70% | -0.77% | 106% | 103% |
| Class C | -0.54% | -0.28% | -0.25% | 106% | 102% |
| Class E | -0.73% | -2.26% | -2.44% | 95% | 90% |
| Overall | -0.64% | -0.95% | -1.02% | 103% | 99% |
| Class D | -0.45% | -0.55% | -0.59% | 106% | 100% |
| Class F | -0.16% | -0.02% | 0.26% | 100% | 98% |

Full results are attached to this AHG report as Excel files.

JVET-AA0194 reports coding results for HDR CTC and proposes to verify performance impact for non-normative changes also HDR CTC. The AHG recommends reviewing the contribution.

## Issues in VTM 16 affecting conformance

The following issues in VTM master branch (Jan. 11, 2022) affect conformance:

- Missing HLS features (see sections below)

There are no known issues in VTM that affect processing of current VVC v1 conformance bitstreams.

## Status of implementation of proposals of previous JVET meetings

The following list contains all adoptions of the Q and R meetings that were not marked as merged (or submitted) or specification only change in the software coordinator tracking sheet:

- JVET-Q0112

- JVET-Q0154: Disallow mixing of GDR and IRAP (Disallow mixing of GDR with any non-GDR).

- JVET-Q0164

- JVET-Q0402

- JVET-R0178: Require that when no_aps_constraint_flag is equal to 1, sps_lmcs_enabled_flag and sps_scaling_list_enabled_flag shall be equal to 0

- JVET-R0221

- JVET-R0046: Change the description of the bitstream extraction process per the value of max_tid_il_ref_pics_plus1[ ][ ] (aspect 1.2 per JVET-R0046-v4).

- JVET-R0065: Specify that GDR AUs shall be complete – i.e., all of the layers in the CVS shall have a picture in the AU (as with IRAP AUs).

- JVET-R0191: Update the range value for num_ols_hrd_params_minus1.

- JVET-R0222 aspect 1: Infer vps_max_sublayers_minus1 to be equal to 6 when sps_video_parameter_set_id is equal to 0 (i.e. VPS is not present). The exact editorial expression is at the discretion of the editor.

- JVET-S0196 (JVET-S0144 item 17)

- JVET-S0227 (JVET-S0144 item 22)

- JVET-S0077 (JVET-S0139 item 5)

- JVET-S0174 aspect 2 (JVET-S0139 item 18.b)

- JVET-S0156 aspect 3 (JVET-S0139 item 21)

- JVET-S0139 item 26 (no source listed, text only?)

- JVET-S0188 aspect 1 (JVET-S0139 item 28)

- JVET-S0139 item 40 (item does not exist)

- JVET-S0042 (JVET-S0142 item 1.b)

- JVET-S0174 aspect 1 (JVET S0143 item 19)

- JVET-S0096 aspect 3 (JVET-S0140 item 10)

- JVET-S0096 aspect 4 (JVET-S0140 item 13)

- JVET-S0159 aspect 3 (JVET-S0140 item 16)

- JVET-S0171 (JVET-S0256)

- JVET-S0118 (JVET-S0141 item 7)

- JVET-S0102 (JVET-S0141 item 9.a)

- JVET-S0157 item 2 (JVET-S0141 item 13)

- JVET-S0157 item 4 (JVET-S0141 item 14)

- JVET-S0175 aspect 3 (JVET-S0141 item 16)

- JVET-S0175 aspect 1, 2 (JVET-S0141 item 17)

- JVET-S0175 aspects 4 and 5 (JVET-S0141 item 18)

- JVET-S0175 aspect 6 (JVET-S0141 item 19)

- JVET-S0198/ JVET-S0223 (JVET-S0141 item 24)

- JVET-S0173 aspect 2 (JVET-S0141 item 40.b)

- JVET-S0173 item 1 (JVET-S0141 item 51)

- JVET-S0173 item 3 (JVET-S0141 item 52)

- JVET-S0173 item 5 (JVET-S0141 item 53)

- JVET-S0173 item 6 (JVET-S0141 item 54)

- JVET-S0173 item 4 (JVET-S0141 item 56)

- JVET-S0176 item 4 (JVET-S0141 item 60)

- JVET-S0154 aspect 5 (JVET-S0141 item 68)

- JVET-S0154 aspect 6 (JVET-S0141 item 69)

- JVET-S0154 aspect 8 (JVET-S0141 item 71)

- JVET-S0095 aspect 5 (JVET-S0145 item 5)

- JVET-S0095 aspect 6 (JVET-S0145 item 6)

- JVET-S0100 aspect 1, depends on JVET-R0193 (JVET-S0147 item 2)

- FINB ballot comments

- Make high tier support up to 960.

# HM related activities

HM 16.26 is expected to be tagged during or shortly after the 27th JVET meeting. Changes include so far:

- JVET-Y0155: Fixes for motion-compensated temporal prefilter

- JVET-Y0105: An improved VVC rate control scheme

- JVET-Y0077: Block Importance Mapping

- Silence compiler warning

- Update copyright date to include 2022

- fix ticket #1516

The following MRs are pending:

- JCTVC-AD0021(JVET-T0056) SEI manifest & SEI prefix indication

- Mark the current picture as short-term ref (for SCM)

The HM SCC branch was not updated for recent HM versions. It appears possible to merge the SCC branch into the main HM branch. It may though be helpful to move SCC related functionality into separate source files. Volunteer work towards merging the branches would be appreciated.

As reported in the previous reports, further information on lambda optimisation in HM would be appreciated, including comparison of allocation of bits within the GOP structures between HM and VTM.

The HEVC bug tracker lists:

- 38 tickets for “HM”, most of which are more than 5 years,

- 1 ticket for “HM RExt”, which was created during this reporting period,

- 7 tickets for “HM SCC”, all of which are at least 3 years old,

Help to address these tickets would be appreciated.

One merge request is available related to HM SCC for ticket #1511 on SCC reference picture marking. We would appreciate help to confirm that the proposed change matches the SCC text.

# 360Lib related activities

The latest 360Lib software (360Lib-13.2) can be found at https://vcgit.hhi.fraunhofer.de/jvet/360lib

The following table is for the projection formats comparison using VTM-17.0 according to 360-degree video CTC (JVET-U2012) compared to that using VTM-16.0 (VTM-16.0 as anchor).

|  | PERP: VTM-17.0 over VTM-17.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -0.48% | -2.87% | -2.63% | -0.49% | -2.87% | -2.63% |
| Class S2 | -0.65% | -2.56% | -2.48% | -0.65% | -2.56% | -2.49% |
| Overall | -0.55% | -2.74% | -2.57% | -0.56% | -2.75% | -2.58% |

The following table compares generalized cubemap (GCMP) coding and padded equi-rectangular projection (PERP) coding using VTM-17.0 (PERP as anchor).

|  | GCMP Over PERP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -11.53% | -5.55% | -6.04% | -11.51% | -5.49% | -5.99% |
| Class S2 | -3.72% | 0.80% | 1.26% | -3.69% | 0.90% | 1.33% |
| Overall | -8.40% | -3.01% | -3.12% | -8.38% | -2.93% | -3.06% |

The following tables are for PERP and GCMP coding comparison between VTM-17.0 and HM-16.22 (HM as anchor), respectively.

|  | VTM-17.0 PERP Over HM-16.22 PERP (anchor) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -30.91% | -38.80% | -41.29% | -30.90% | -38.85% | -41.29% |
| Class S2 | -36.88% | -37.35% | -39.69% | -36.87% | -37.39% | -39.75% |
| Overall | -33.30% | -38.22% | -40.65% | -33.29% | -38.27% | -40.67% |

|  | VTM-17.0 GCMP Over HM-16.22 PCMP (anchor) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -35.61% | -40.88% | -42.79% | -35.57% | -40.82% | -42.74% |
| Class S2 | -39.84% | -39.56% | -41.51% | -39.85% | -39.56% | -41.55% |
| Overall | -37.30% | -40.35% | -42.28% | -37.28% | -40.31% | -42.27% |

# SCM related activities

There had not been any further developments to SCC’s SCM during this meeting cycle.

# SHM related activities

There had not been any further developments to SHVC’s SHM during this meeting cycle.

# HTM related activities

There had not been any releases ofHTM of MV-HEVC and 3D-HEVC.

The next release will include the following changes:

- JVET-Z0209: Early termination during calculating RDcost of depth

# HDRTools related activities

There had not been any updates of HDRTools.

New development is being added under the branch named 0.24-dev.

# JM, JSVM, JMVM related activities

There had not been any updates to the JM, JSVM and JMVM software.

# Bug tracking

The bug tracker for VTM and specification text is located at:

https://jvet.hhi.fraunhofer.de/trac/vvc

The bug tracker uses the same accounts as the HM software bug tracker. Users may need to log in again due to the different sub-domain. For spam fighting reasons account registration is only possible at the HM software bug tracker at

https://hevc.hhi.fraunhofer.de/trac/hevc

Bug tracking for HDRTools is located at:

https://gitlab.com/standards/HDRTools/-/issues

Please file all issues related to the VVC reference software and HDRTools into the appropriate bug tracker. Try to provide all the details, which are necessary to reproduce the issue. Patches for solving issues and improving the software are always appreciated.

# Software repositories

Git repositories that were previously assigned to the JCT-VC group on the GitLab server were re-assigned to the JVET group. The old URLs are still working and will forward the user to the new location, with the display of a warning suggesting to update bookmarks to the new location. The SVN repository for 360Lib was converted to git and development was moved to the GitLab server. Historical branches can still be accessed in the SVN repository.

# CTC alignment and merging

JVET-Z2011 was produced as JVET output document containing the merged VTM and HM CTC for SDR/WCG video.

Merged CTC for high bit depth and high bit rate video coding are proposed in JVET-AA0130.

Further merging of HM RExt CTC into the appropriate VVC CTC was investigated (non 4:2:0 chroma formats), but proper comparable HM configuration files were not yet available by the beginning of this meeting. It is planned to provide an input to the 28th meeting.

It was reported that CTC documents may contain an error in the formulas describing PSNR computation. For example, JVET-Y2010 states:

“For 10-bit video, PSNR is calculated as 10*(log10((255 << 2) / MSE), 8-bit content is converted to 10-bit input in the encoder by shifting 2 bits to the left and 10-bit PSNR calculation is used to report testing results.”

This should be corrected to:

“For 10-bit video, PSNR is calculated as 10*(log10((255 << 2)2 / MSE), 8-bit content is converted to 10-bit input in the encoder by shifting 2 bits to the left and 10-bit PSNR calculation is used to report testing results.”

# Recommendations

The AHG recommends to:

- Continue to develop reference software

- Improve documentation, especially the software manual

- Encourage people to test VTM and other reference software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bit-streams/test cases that trigger bugs in VTM and other reference software.

- Encourage people to submit non-normative changes that either reduce encoder run time without significantly sacrificing compression performance or improve compression performance without significantly increasing encoder run time

- Design and add configuration files to the VTM software for testing of HLS features

- Review VTM-related contributions and determine whether features should be added (or removed) from the software

- Continue to investigate the merging of branches.

- Continue to investigate merging of CTC documents.

- Verify correctness of CTC documents and issue updates as appropriate

- Keep common test conditions aligned for the different standards.

- Consider documents (including late documents) related to AHG3 activities