| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0006-v1 |
| --- | --- |

| Title: | JVET AHG report: ECM software development (AHG6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Fabrice Le Leannec Kai Zhang | Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com fabricel@xiaomi.com zhangkai.video@bytedance.com |
| Source: | AHG6 on ECM software development |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AHG6 on ECM software development that has taken place between the 26th and 27th JVET meetings.

# Introduction

The mandates for the AHG are as follows.

- Coordinate development of the ECM software and associated configuration files.

- Produce documentation of software usage for distribution with the software.

- Prepare and deliver ECM-5.0 software version and the reference configuration encodings according to JVET-Y2017 common test conditions.

- Investigate encoder speedup and other encoder software optimization.

- Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate.

# Software development

ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

The following adopted aspects were integrated into ECM-4.0:

JVET-Z0131: Block vector difference binarization

JVET-Z0127 (option 2): GPM intra-inter mode

JVET-Z0050 (Test 1.3b): CCLM slop adjustment, DIMD chroma with fusion mode

JVET-Z0054 (Test 2.1b): Block level reference picture reordering

JVET-Z0136 (test EE2 - 2.2a): Enhanced bi-prediction with out-of-boundary prediction samples

JVET-Z0061 (test EE2 – 2.3): Template matching based OBMC

JVET-Z0117 (version where no switching between 4-tap and 6-tap filters): 6-tap chroma interpolation filters

JVET-Z0118: GDR

JVET-Z0056 (test EE2 - 2.4): Template matching based reordering for GPM split modes

JVET-Z0139 (version EE2-2.7c): History based and non-adjacent affine candidates

JVET-Z0153 (test 3.2): IBC reference area extension

JVET-Z0075 (test 3.3): Enlarged HMVP table for IBC

JVET-Z0084 (test 3.4): Template Matching for IBC

JVET-Z0160 (test 3.5b): Replacement of zero-padding candidates

JVET-Z0135 (test 4.3b): Temporal CABAC, weighted states, windows adjustment

JVET-Z0085: Enabling AMVP-merge mode when DMVD is off

JVET-Z0102: No ARMC for Zero candidates for Regular, TM and BM merge modes

JVET-Z0105 (not CTC): Virtual boundary handling for in-loop filters

JVET-Z0083 (solution 1): Fix for MHP parsing condition

JVET-Z0072: Enhanced reference picture structures (encoder)

JVET-Z0099: Enable RDO-DBF in RA, LDB, and LDP (encoder)

JVET-Z0150: Print memory usage

JVET-Z0067/MR105: RPR fixes

A fix for invalid memory access when EncDbOpt=1 was also ported into ECM software from the latest VTM software.

ECM-5.0 was tagged on June 2 2022.

GDR implementation requires software optimization, so GDR macro was disabled in ECM-5.0 release. It is planned to enable it in ECM-5.1.

## CTC Performance

In this section, ECM-5.0 test results following ECM CTC configuration descried in JVET-Y2017 are summarized.

The below tables show ECM-5.0 performance over ECM-4.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -2.34% | -2.28% | 109% | 104% |
| Class A2 | -0.20% | -2.09% | -1.52% | 113% | 105% |
| Class B | -0.23% | -2.29% | -2.55% | 110% | 107% |
| Class C | -0.24% | -1.78% | -1.83% | 111% | 104% |
| Class E | -0.27% | -1.79% | -1.63% | 110% | 108% |
| Overall | -0.24% | -2.07% | -2.02% | 111% | 106% |
| Class D | -0.21% | -1.43% | -1.30% | 111% | 106% |
| Class F | -5.49% | -6.34% | -6.50% | 135% | 104% |
| Class TGM | -11.42% | -11.32% | -11.10% | 125% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.14% | -2.79% | -2.97% | 123% | 113% |
| Class A2 | -2.27% | -4.15% | -3.99% | 124% | 118% |
| Class B | -2.72% | -4.63% | -4.96% | 122% | 114% |
| Class C | -2.58% | -5.42% | -5.20% | 124% | 116% |
| Class E |  |  |  |  |  |
| Overall | -2.48% | -4.38% | -4.43% | 123% | 115% |
| Class D | -2.71% | -6.45% | -6.98% | 120% | 114% |
| Class F | -5.27% | -6.80% | -7.06% | 126% | 107% |
| Class TGM | -12.55% | -12.67% | -12.89% | 117% | 102% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.87% | -4.83% | -4.61% | 122% | 113% |
| Class C | -4.92% | -8.15% | -7.89% | 130% | 118% |
| Class E | -3.82% | -5.26% | -5.70% | 121% | 110% |
| Overall | -3.79% | -6.04% | -5.97% | 124% | 114% |
| Class D | -4.43% | -7.91% | -7.81% | 127% | 125% |
| Class F | -7.08% | -7.91% | -8.99% | 129% | 114% |
| Class TGM | -14.86% | -15.50% | -15.64% | 119% | 105% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.83% | -2.32% | -2.41% | 118% | 112% |
| Class C | -1.60% | -3.77% | -3.27% | 117% | 112% |
| Class E | -1.94% | -2.43% | -2.21% | 115% | 109% |
| Overall | -1.78% | -2.83% | -2.65% | 117% | 111% |
| Class D | -2.63% | -4.06% | -3.85% | 117% | 114% |
| Class F | -4.70% | -6.17% | -6.40% | 131% | 111% |
| Class TGM | -9.31% | -9.36% | -9.82% | 122% | 103% |

Next tables show ECM-5.0 performance over VTM-11.0ecm anchor, the software is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.84% | -14.30% | -19.53% | 390% | 266% |
| Class A2 | -6.42% | -15.04% | -16.00% | 380% | 259% |
| Class B | -5.92% | -16.12% | -15.78% | 433% | 276% |
| Class C | -7.11% | -10.83% | -11.38% | 424% | 254% |
| Class E | -7.92% | -13.65% | -14.49% | 401% | 277% |
| Overall | -6.75% | -14.05% | -15.25% | 409% | 267% |
| Class D | -5.92% | -9.33% | -8.84% | 421% | 268% |
| Class F | -15.79% | -20.74% | -20.14% | 355% | 270% |
| Class TGM | -26.19% | -28.03% | -27.29% | 317% | 275% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -16.80% | -18.70% | -24.21% | 436% | 536% |
| Class A2 | -17.87% | -24.31% | -26.27% | 415% | 626% |
| Class B | -15.58% | -23.66% | -22.40% | 432% | 550% |
| Class C | -17.51% | -21.09% | -20.65% | 454% | 555% |
| Class E |  |  |  |  |  |
| Overall | -16.80% | -22.11% | -23.07% | 435% | 563% |
| Class D | -18.49% | -22.02% | -22.71% | 444% | 579% |
| Class F | -18.31% | -23.02% | -23.03% | 403% | 371% |
| Class TGM | -24.41% | -28.17% | -28.50% | 425% | 299% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -14.25% | -27.64% | -25.57% | 396% | 419% |
| Class C | -15.40% | -24.03% | -23.37% | 449% | 434% |
| Class E | -12.99% | -18.62% | -20.64% | 400% | 317% |
| Overall | -14.32% | -24.18% | -23.61% | 414% | 396% |
| Class D | -17.44% | -25.35% | -24.26% | 416% | 489% |
| Class F | -16.96% | -24.65% | -24.21% | 399% | 328% |
| Class TGM | -23.33% | -30.18% | -29.91% | 415% | 294% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -14.09% | -28.08% | -26.41% | 355% | 418% |
| Class C | -15.67% | -23.82% | -23.15% | 333% | 419% |
| Class E | -13.11% | -18.84% | -20.34% | 344% | 317% |
| Overall | -14.37% | -24.35% | -23.81% | 345% | 390% |
| Class D | -18.52% | -25.59% | -24.04% | 312% | 431% |
| Class F | -17.47% | -25.22% | -24.88% | 372% | 308% |
| Class TGM | -23.20% | -30.41% | -30.28% | 418% | 290% |

The Excel files with the complete ECM results are attached to this report.

# Recommendations

The AHG recommends to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.