| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0012-v1 |
| --- | --- |

| Title: | JVET AHG report: Enhanced compression beyond VVC capability (AHG12) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | M. Karczewicz Y. Ye L. Zhang B. Bross X. Li K. Naser H. Yang | Author(s) or Contact(s): | martak@qti.qualcomm.com yan.ye@alibaba-inc.com lizhang.idm@bytedance.com benjamin.bross@hhi.fraunhofer.de xlxiangli@google.com karam.naser@interdigital.com haitao.yang@huawei.com |
| Source: | AHG 12 |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on enhanced compression beyond VVC capability (AHG12) between the 26th meeting by teleconference, 20-29 April 2022, and the 27th meeting by teleconference, 13-22 July 2022.

# Introduction

The mandates given to the AHG are:

- Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC.

- Discuss and propose refinements to the ECM5 algorithm description JVET-Z2025.

- Study the performance and complexity tradeoff of these video coding tools.

- Coordinate with AHG6 on ECM software development.

- Support AHG6 in generating anchors according to the test conditions in JVET-Y2017.

- Analyse the results of exploration experiments described in JVET-Z2024 in coordination with the EE coordinators.

- Coordinate with AHG11 to study the interaction with neural network-based coding tools.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). A few emails were exchanged regarding EE2.

# Activities

The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-Z2024). The combined improvements of the ECM-5.0 over VTM-11.0ecm anchor for AI, RA and LB configurations are:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.84% | -14.30% | -19.53% | 390% | 266% |
| Class A2 | -6.42% | -15.04% | -16.00% | 380% | 259% |
| Class B | -5.92% | -16.12% | -15.78% | 433% | 276% |
| Class C | -7.11% | -10.83% | -11.38% | 424% | 254% |
| Class E | -7.92% | -13.65% | -14.49% | 401% | 277% |
| Overall | -6.75% | -14.05% | -15.25% | 409% | 267% |
| Class D | -5.92% | -9.33% | -8.84% | 421% | 268% |
| Class F | -15.79% | -20.74% | -20.14% | 355% | 270% |
| Class TGM | -26.19% | -28.03% | -27.29% | 317% | 275% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -16.80% | -18.70% | -24.21% | 436% | 536% |
| Class A2 | -17.87% | -24.31% | -26.27% | 415% | 626% |
| Class B | -15.58% | -23.66% | -22.40% | 432% | 550% |
| Class C | -17.51% | -21.09% | -20.65% | 454% | 555% |
| Class E |  |  |  |  |  |
| Overall | -16.80% | -22.11% | -23.07% | 435% | 563% |
| Class D | -18.49% | -22.02% | -22.71% | 444% | 579% |
| Class F | -18.31% | -23.02% | -23.03% | 403% | 371% |
| Class TGM | -24.41% | -28.17% | -28.50% | 425% | 299% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -14.25% | -27.64% | -25.57% | 396% | 419% |
| Class C | -15.40% | -24.03% | -23.37% | 449% | 434% |
| Class E | -12.99% | -18.62% | -20.64% | 400% | 317% |
| Overall | -14.32% | -24.18% | -23.61% | 414% | 396% |
| Class D | -17.44% | -25.35% | -24.26% | 416% | 489% |
| Class F | -16.96% | -24.65% | -24.21% | 399% | 328% |
| Class TGM | -23.33% | -30.18% | -29.91% | 415% | 294% |

The rate reduction for natural sequences over VTM for RA configuration for {Y, U, V} increased from {-15.83%, -19.85%, -20.78%} to {-16.80%, -22.11%, -23.07%}. For SCC sequences (class TGM) the rate reduction for RA configuration increased from {-16.11%, -19.04%, -19.39%} to {-24.41%, -28.17%, -28.50%}.

# Contributions

In addition to 21 EE2 contributions, 38 (same as last meeting) related contributions were received which can be subdivided as follows:

## In Loop Filters (2)

JVET-AA0139, "Non-EE2: Longer deblocking filter for luma", K. Andersson J. Enhorn (Ericsson)

JVET-AA0147, "EE2-Related: Extended Offline-Filtering Taps for ALF", W. Yin, K. Zhang, Z. Deng, L. Zhang (Bytedance)

## Intra (13)

JVET-AA0043, "IntraTMP Adaptation for Camera Captured Contents", K. Naser, T. Poirier, F. Galpin, A. Robert (InterDigital)

JVET-AA0044, "IntraTMP for chroma Components", K. Naser, T. Dumas, T. Poirier, F. Galpin (InterDigital)

JVET-AA0045, "[EE2-1.4 related] Reduced Complexity Spatial GPM", K. Naser, Y. Chen, A. Robert, K. Reuze (InterDigital)

JVET-AA0103, "EE2-related: CCLM with non-linear term",	X. Li, Y. Ye, R.-L. Liao, J. Chen (Alibaba)

JVET-AA0104, "Non-EE2: On planar horizontal mode and planar vertical mode", X. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)

JVET-AA0114, "[EE2-related] Division-free operation and mean-compensation for convolutional cross-component model (CCCM)", A. Aminlou, J. Lainema, P. Astola (Nokia)

JVET-AA0120, "Non-EE2: Template-based multiple reference line intra prediction", L. Xu, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AA0121, "Non-EE2: Template-based MIP", Z. Xie, Y. Yu, H. Yu, D. Wang, Y. Liu, M. Li (OPPO), J. Huo, W. Qiao, X. Hao, Y. Ma, F. Yang (Xidian University),

JVET-AA0127, "EE2-1 related: Encoder optimization of EE2-1.2 and 1.3", C.-W. Kuo, H.-J. Jhu, X. Xiu, N. Yan, W. Chen, X. Wang (Kwai)

JVET-AA0136, "Non-EE2: On CCCM improvement", Y.-J. Chang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AA0137, "Non-EE2: Intra Prediction Fusion", K. Cao, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AA0138, "EE2-related: On Gradient Linear Model (GLM)", X. Li, Y. Ye, R.-L. Liao, J. Chen (Alibaba)

JVET-AA0140, "EE2-related: Self-Aware Filter Estimation for CCLM", K. Zhang, Z. Deng, L. Zhang (Bytedance)

## Inter (12)

JVET-AA0069, "Non-EE2: AmvpMerge for low delay", H.Jang, J.Nam, N.Park, J.Lim, S.Kim (LGE)

JVET-AA0075, "Non-EE2: Template matching based BCW index derivation for merge mode", R.-L. Liao, J. Chen, Y. Ye, X. Li (Alibaba)

JVET-AA0119, "EE2-1.4a-related: Modifications of Spatial GPM", F. Wang, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AA0124, "Non-EE2: Enable amvpMerge mode on scaled reference pictures when DMVD is disabled", Z. Zhang, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AA0128, "EE2-related: On regression based affine candidate derivation", W. Chen, X. Xiu, C.-W. Kuo, H.-J. Jhu, N. Yan, X. Wang (Kwai)

JVET-AA0134, "Non-EE2: POC based BCW weights derivation", Z. Zhang, H. Huang, C.-C. Chen, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AA0141, "Non-EE2: Enhanced temporal motion information derivation", L. Zhao, K. Zhang, L. Zhang (Bytedance)

JVET-AA0142, "AHG12/Non-EE2: Picture-Level Geometry Transform", W. Jia, K. Zhang, Y. Wang, T. Fu, Y. Li, L. Zhang (Bytedance)

JVET-AA0143, "Non-EE2: Simplification methods for OBMC", K. Kim, D. Kim, J.-H. Son, J.-S. Kwak (WILUS)

JVET-AA0144, "Non-EE2: DMVR for affine merge coded blocks", J. Chen, R.-L. Liao, X. Li, Y. Ye (Alibaba)

JVET-AA0148, "Non-EE2: On MHP (Multi-Hypothesis Prediction)", K. Sato, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AA0149, "EE2-1.4 related: Improvements on Spatial GPM", A. Natesan, J. N. Shingala, J. R. Arumugam, V. Valvaiker (Ittiam), T. Lu, P. Yin, F. Pu, T. Shao, A. Arora, S. McCarthy (Dolby)

## Transform Coding (2)

JVET-AA0064, "AHG12: A study on non-separable primary transform", J. Choi, M. Koo, J. Lim, J. Zhao, S. Kim (LGE)

JVET-AA0073, "Non-EE2: Modification of LFNST for MIP coded block", J.-Y. Huo, W.-H. Qiao, X. Hao, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), J. Ren, M. Li (OPPO)

## Entropy Coding (1)

JVET-AA0150, "AHG12: On CIPF (CABAC Initialization from the Previous Frame)", K. Sato, Y. Yu, H. Yu, D. Wang (OPPO)

## Screen Content Coding (3)

JVET-AA0053, "AHG12: Using block vector derived from IntraTMP for IBC", W. Lim, D. Kim, S.-C. Lim, J. S. Choi (ETRI)

JVET-AA0077, "AHG12: On BVD coding for IBC", A. Filippov, V. Rufitskiy (Ofinno)

JVET-AA0108, "AHG12: IBC AMVP candidates clustering", D. Ruiz Coll, V. Warudkar (Ofinno)

## 360 Video (1)

JVET-AA0146, "AHG12/Non-EE2: Fixes on ECM for 360-degree video coding", Y. Wang, K. Zhang, Z. Deng, L. Zhang (Bytedance)

## ECM Encoder and Test Conditions (4)

JVET-AA0097, "ECM fix for block-level out-of-bound checking", F. Le Léannec, P. Andrivon, M. Radosavljevć (Xiaomi)

JVET-AA0098, "AHG12: Encoder configuration proposal to reduce worst case encoding time", F. Le Léannec, P. Andrivon, M. Radosavljevć (Xiaomi)

JVET-AA0129, "Non-EE2/AHG10: Improved inter hash RDO considering OBMC off in ECM5.0", X. Xiu, C.-W. Kuo, H.-J. Jhu, W. Chen, N. Yan, X. Wang (Kwai)

JVET-AA0132, "AHG6: ECM software configuration parameters for template matching tools", C.-C. Chen, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)

# Recommendations

The AHG recommends to:

- To review all the related contributions.