| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0013-v1 |
| --- | --- |

| Title: | JVET AHG report: Film grain technologies (AHG13) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JVET |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | W. Husak M. Radosavljević W. Wan D. Grois Y. He P. de Lagrange Andrew Segall A. Tourapis | Email: | wjh@dolby.com milosr@xiaomi.com wade.wan@broadcom.com Dan_Grois@comcast.com yonghe@qti.qualcomm.com philippe.delagrange@interdigital.com asegall@amazon.com atourapis@apple.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document summarizes the activities of AHG13: Film grain technologies, between the 26th meeting (teleconference, 20-29 April 2022) and the 27th meeting (teleconference, 13-22 July 2022).

# Mandates

At the 26th meeting of the ITU-T/ISO/IEC Joint Video Experts Team (JVET), an ad hoc group on Film grain technologies was established with the following mandates:

- Study the benefits and characteristics of film grain technologies, including autoregressive and frequency-filtering technologies.

- Discuss the JVET-Y2020 draft of the Technical Report on Film grain synthesis technology for video applications and suggest improvements as necessary.

- Study alternative film grain models and their associated documentation.

- Study preprocessing and encoder technologies for determining values for FGC (Film Grain Characteristics) SEI message syntax elements.

- Identify potential need for additional film grain technology and signaling, if needed.

- Study categorization and/or classification of FGC implementations.

- Study the implication of identifying FGC reference implementations.

- Coordinate development of film grain technology software and configuration files.

- Coordinate with AHG3 for software support of the FGC SEI message.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). A single e-mail related to AHG13 activity was sent to the JVET reflector during the AHG period.

# Discussion

## Conformance

The topic of conformance for film grain SEI implementations was explored.  A key aspect of the discussion was the history of film grain implementations in other groups.  Another video codec group explored the concept of making conformance mandatory but realized that any implementation would be dependent on the post-decode video processing platform.  Codec conformance generally ends at the decode buffer while further video processing may occur downstream.  This is the same issue ISO/IEC/ITU faces where a potential reference model would also need to specify downstream video pipeline details – something that is typically out of scope of JVET.

Film grain that is modeled by the encoder is targeted to a nominal resolution that is the anticipated decoded video output.  The display device will then adapt the decoded output to the display using brand-specific and/or display technology-specific video pipelines.  Some example video processing steps that may impact conformance are internal working color spaces, video pixel representations, chroma sub-sampling, and image resizing.

Another relevant point is VVC v1 has been commercially available but without any specified film grain conformance and therefore a class of decoders will exist that cannot inherently meet any post facto conformance specification.  These reasons result in an acknowledgement that hard conformance will not be practical for film grain.  Soft conformance is currently undefined but more attractive for discussion within the context of SEI post processing.  Application/implementation level specifications might be better places to define or specify conformance due to the above constraints.

The following conformance definitions were discussed:

Hard conformance – an implementer must follow a specific set of guidelines for an expected output.

Soft conformance – an implementer should follow specific guidelines but may have other options for implementation. The frame packing SEI is a good example of an application that could have soft conformance.  HDR SEIs can also be a model due to display adaptation.

Not required – implementations are completely optional.

The artifact masking use case is a context where strict conformance could be difficult to define and may be unnecessary since the desired outcome is subjective image improvement.

## Implementations (Identification)

The definition differences between a reference model and technology example were also discussed.  The reference model is traditionally a working example used throughout the development of the standard and/or report.  In the film grain case, there are equations that could benefit from a software model.  It is believed that the “reference model/implementation” and “example” are interchangeable for this application. The group recommends there is language highlighting that implementers can use the reference implementation, but they may choose to use alternative implementations.

## Classification/Categorization

Classification was discussed and the result was any technology could be categorized and/or classified by use case.  Three use cases identified are:

- Film grain insertion (creative intent)

- Film grain removal/replacement

- Artifact masking.

More use cases could be identified at a later date.  Also explored was transcoding between autoregressive and frequency-based models although that could be a different aspect and requires more research.  Other differences in models, such as implementation complexity, may also be important.

An important question is whether there are technology examples that favor one or more of the use cases?  The main differences are in the implementations themselves – different use cases may need more detailed parameter sets.  An example is bitrate and application can create a dependency for at least artifact masking (less bitrate may need more film grain energy).  Another example is mobile devices could require less grain due screen size or even implementation complexity.  There could be other parameters that are unique to the content (motion etc.) or the display/environment.

Different implementation examples may be chosen based on the target application in order to favor specific parameters.  Automatic parameter estimation or manual parameter settings may be better done using one implementation example over another.  The encoder and decoder should be a matched set (or closely related).  There may need to be a translation step required in this type of scenario.  This may not be a big issue but should be further explored.

Additional thoughts explored are:

- What happens with display devices compositing local text or graphics?

- HDR video pipeline design could present a problem for film grain insertion.

- Arbitrary image scaling for final display could change the look of the grain.

This could lead to a note in the technical report.

# Related contributions

Three contributions related to AHG13 were identified as of 07/12/2022:

- One was the AHG report:

  - JVET-AA0013 JVET AHG report: Film grain technologies (AHG13)

- Two were related to the technical report and explore profiling:

  - JVET-AA0051 Film grain synthesis technology for video applications (Draft 2)

  - JVET-AA0052 AHG13: On VSEI film grain profiles

## Contributions

### JVET-AA0051 Film grain synthesis technology for video applications (Draft 2)

This draft technical report provides guidance on the use of film grain synthesis technology for video applications. Such technology may be used in conjunction with metadata signalling mechanisms, such as the supplemental enhancement information messages available in several video coding standards. The purpose of this document is to provide a publicly referenceable overview of end-to-end processing steps for film grain synthesis, that may include content analysis, noise/film grain removal and film grain model parameter estimation, parameter encoding, encapsulation, and decoding, and film grain synthesis and blending for consumer distribution applications.

### JVET-AA0052 AHG13: On VSEI film grain profiles

This contribution proposes two VSEI profiles, frequency filtering film grain synthesis profile and auto-regressive film grain synthesis profile, for the film grain characteristics (FGC) SEI message. The profiles specify constraints on FGC SEI and the requirements for the capabilities needed to enable the use of FGC SEI.

# Recommendation

The AHG recommends:

- the related input contributions are reviewed;

- the draft TR text be edited during the JVET meeting period;

- the draft TR includes notes list above; and

- continue the study of film grain technologies in JVET.