| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0045-r1 |
| --- | --- |

| Title: | [EE2-1.4 related] Reduced Complexity Spatial GPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Naser Y. Chen A. Robert K. Reuze | Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution is related to spatial geometrical partition mode (SGPM) that is currently being studied in EE2. It is proposed to reduce the number of tested combinations of split and intra modes by selecting only two intra modes from TIMD process. Furthermore, reduced signaled SGPM modes and reduced block sizes are tested to achieve a reasonable compromise between the gain and coding complexity.

The following results are obtained:

Test1: #intraModes = 2 + reduced SGPM modes + reduced block sizes

  - AI: -0.07%, -0.06%, -0.05% with 105%% EncT and 100%DecT

Test2: #intraModes = 2 + reduced block sizes

  - AI: -0.18%, -0.14%, -0.14% with 120%% EncT and 106%DecT

# Introduction

SGPM is a new intra mode currently being studied in EE2. It is an extension of GPM mode for intra coded block. The process of SGPM can be summarized as follows:

- Pre-select 8 intra modes from MPM

- Pre-select 26 GPM splits (out of 64)

- Perform template analysis to yield 16 possible combinations of intra modes and split directions

The template analysis part is performed by examining all the possible combinations of 8 intra modes for first side of the split, 7 modes for the second side of the split (avoiding using the same mode on both sides) with the 26 split directions. This leads to 8 * (8-1) * 26 = 1456 tests at both encoder and decoder. Additionally, the encoder performs SATD tests to select 8 out of 16 possible combinations that are further tested on full-RDO.

# Proposed method

This contribution proposes to reduce the complexity associated with SGPM by the following:

- Reduced intra modes set: it is proposed to reduce the number of modes to 2. Those two modes are derived by TIMD process where the first mode is derived from the upper template while the second is from the left template.

- Reduced number of signaled SGPM modes: Instead of signaling 16 modes, only 2 modes are considered. This way, both SATD and full-RDO tests are reduced.

- Reduced block sizes: SGPM is only applied to block whose number of samples is less than or equal 1024.

In Test1, all the above points are considered, whereas in Test2 the second point is not considered.

# Results

The proposed methods are implemented on top of ECM-5.0. The following test results are obtained on top of ECM-5.0 using the common test conditions:

Table 1 proposed method (Test1) on top of ECM-5.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.04% | -0.08% | 103% | 96% |
| Class A2 | -0.09% | -0.16% | -0.07% | 106% | 102% |
| Class B | -0.06% | -0.05% | -0.02% | 102% | 96% |
| Class C | -0.06% | -0.05% | 0.06% | 106% | 104% |
| Class E | -0.08% | -0.02% | -0.18% | 109% | 106% |
| Overall | -0.07% | -0.06% | -0.05% | 105% | 100% |
| Class D | -0.02% | 0.13% | -0.04% | 102% | 105% |
| Class F | -0.09% | 0.06% | -0.01% | 101% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.05% | -0.19% | 96% | 98% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | -0.20% | -0.25% | 101% | 102% |
| Class C | -0.03% | 0.05% | -0.02% | 104% | 109% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.02% | 0.04% | -0.16% | 104% | 112% |
| Class F | -0.01% | -0.01% | -0.04% | 105% | 108% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2 proposed method (Test2) on top of ECM-5.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.14% | -0.18% | 124% | 105% |
| Class A2 | -0.21% | -0.23% | -0.23% | 117% | 100% |
| Class B | -0.16% | -0.09% | -0.14% | 120% | 101% |
| Class C | -0.15% | -0.15% | -0.01% | 115% | 105% |
| Class E | -0.26% | -0.15% | -0.22% | 128% | 120% |
| Overall | -0.18% | -0.14% | -0.14% | 120% | 106% |
| Class D | -0.08% | 0.07% | 0.06% | 122% | 114% |
| Class F | -0.04% | 0.04% | 0.03% | 115% | 111% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | 0.03% | -0.28% | 102% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.10% | -0.05% | -0.40% | 104% | 103% |
| Class C | -0.09% | -0.07% | -0.03% | 106% | 110% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.07% | -0.38% | 0.31% | 107% | 112% |
| Class F | -0.09% | 0.00% | -0.12% | 107% | 109% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

# This contribution proposes simplification to SGPM in the intra mode derivation part. It is proposed to use template analysis to deduce only 2 intra modes, instead of 8, that are used in SGPM. It is proposed to further study this method in EE using different value of signaled SGPM modes in order to achieve a good balance between gain and run time.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).