| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0059-v1 |
| --- | --- |

| Title: | EE1-3.1: Supplementary experiments based on JVET-Z0077 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Qipu Qin Cheolkon Jung Dan Zou Ming Li | Email: | kippqin@stu.xidian.edu.cn zhengzk@xidian.edu.cn zoudan@oppo.com myron.li@oppo.com |
| Source: | Xidian University, OPPO |  |  |

_____________________________

- Abstract

This contribution reports an extension of the deep omnidirectional video compression framework (DOVC) to regular 2D videos. Specifically, we provide supplementary experiments based on JVET-Z0077. For I frames, we use VTM intra coding instead of BPG for image compression. For B frames, we generate five DOVC models for both luma and chroma channels. Compared with VTM11.0 NNVC-1.0, DOVC achieves average BD-rate reductions of {3.64% (Y), 4.53% (U) and 3.41% (V)} and average BD-PSNR gains of {-0.1495dB (Y), -0.2141dB (U), and -0.1411dB (V)} on the all 2D video dataset under RA configuration. Furthermore, DOVC takes advantage of GPU parallel processing and thus the average encoding time of DOVC is only 0.022 times that of VTM11.0 NNVC-1.0.

Introduction

Two DOVC frameworks were provided in JVET-X0043 [2] and JVET-Y0051 [3], namely DOVC-RGB and DOVC-YUV, respectively, and DOVC-YUV is conversion of DOVC-RGB from the RGB domain to the YUV 4:2:0 domain. In JVET-Z0077, DOVC-YUV was applied to the regular 2D video compression after retraining to verify its effectiveness. According to the recommendations of the last JVET-Z meeting, this contribution provides supplementary experiments based on JVET-Z0077 [1]. Specifically, VTM11.0 NNVC-1.0 is used as anchor to compare with DOVC, which is configured with VTM intra coding instead of BPG for image compression. Experimental results on 2D video datasets (Class A1, Class B, Class C and Class D) are reported in Section 3.

Overview of Proposed DOVC

Encoder

The overall framework of the DOVC encoder is shown in Fig. 1. The DOVC encoder is mainly composed of the following modules: projection module, separate channels, bidirectional motion estimation, motion encoder/decoder, bidirectional motion compensation, residual encoder/decoder, quality enhancement and entropy coding.

Fig. 1. Overall framework of the DOVC encoder. Note that the main contributions of this framework lie in taking omnidirectional video with YUV 4:2:0 domain as the compression target, bidirectional motion estimation and motion compensation (i.e. blue blocks) and quality enhancement (i.e. purple block) components.

Decoder

For I frame, DOVC adopts VTM intra coding instead of BPG for image compression. The bitstreams of I frame, B/P frame and sequence parameters are transmitted together from the encoder side to the decoder side. The DOVC decoder utilizes a stand-alone version of the arithmetic coder as shown in Fig. 2. This library encodes a feature map into a bit-stream or decodes a bitstream into a feature map.

Fig. 2. Overall framework of the DOVC decoder. Note that this framework is the inverse process of the encoder.

Experimental Results

This contribution compares DOVC with VTM11.0 NNVC-1.0 under random access (RA) and GOP=16 configurations. In the experiments, we use a PC with Intel Xeon CPU (dual processor) and 32G RAM for VTM11.0 NNVC-1.0, GeForce GTX3090 (24G) GPU for testing DOVC, and NVIDIA Tesla V100 GPUs (2×32G) for training DOVC.

Inference Stage

DOVC uses PyTorch-based deep learning to perform the inference/test. The network information in the inference stage is provided in Table 1.

Table 1. Network information for DOVC in the inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory |  |  |
|  | GPU Type | GeForce GTX3090 (24G) |
|  | Framework: | Python3.9, PyTorch1.6, CUDA10.1 |
|  | Number of GPUs per Task | 1 |
|  | Total Parameter Number | Including five model, i.e. λ= 256, 512, 1024, 2048 and 4096 for luma and chroma, respectively. Each luma’s model is 10.8M and each chroma’s model is 5.4M. |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 67.24 MB for each model |
|  | Test data information: | Class A1, Class B and Class D (CTC) |
|  | MAC (Giga) | kernel2×in_channels×out_channels |
| Optional | Total Conv. Layers | DOVC-YUV is composed of a series of CNNs, including a total of 7 modules. |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) | / |
|  | Batch size: | 4 |
|  | Patch size | Whole frame |
|  | Changes to network configuration or weights required to generate rate points | / |
|  | Peak Memory Usage | / |
|  | Other information: | VTM intra for image compression |

Training Stage

DOVC also utilizes PyTorch-based deep learning framework to train all models. The network information in the training stage is provided in Table 2.

Table 2. Network information for DOVC in the training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Nvidia Tesla V100, 2×32G |
|  | Number of GPUs per Task | 2 |
|  | Framework: | Python3.6, PyTorch1.6, CUDA10.0 |
|  | Epoch: | 100 |
|  | Batch size: | 4 |
|  | Loss function: | RDO: R +λ∙MSE Note that R represents the total number of bits for encoding, λ is Lagrangian multiplier, and MSE is a measure of distortion. |
|  | Training time: | About 48 hours / luma model About 20 hours / chroma model |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | λ = 256, 512, 1024, 2048, 4096 |
| Optional | Number of iterations: | 2000000 |
|  | Patch size: | 256 × 256 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | See description in main text |
|  | Mini-batch selection process: | / |
|  | Other information: | VTM intra for image compression |
|  | Preprocessing: | Projection and channel separation |

Rate-Distortion Curve

Fig. 3. shows the rate-distortion comparison on different 2D video datasets. It is obvious that DOVC with VTM intra achieves better performance than DOVC with BPG in terms of the PSNR at all bit-rates. On the all 2D video datasets, DOVC with VTM intra performs slightly worse than the latest video coding standard VTM11.0 NNVC-1.0.

Fig. 3. Rate-distortion curves on Class A1 and Class B.

BD-BR and BD-PSNR

The BD-BR and BD-PSNR of DOVC-YUV are provided in Tables 3 in comparison with VTM11.0 NNVC-1.0. It can be observed from Table 3 that DOVC with VTM intra achieves average {3.64% (Y), 4.53% (U) and 3.41% (V)} reductions in BD-BR and average {-0.1495dB (Y), -0.2141dB (U), and -0.1411dB (V)} gains in BD-PSNR when compare with the latest video coding standard VTM11.0 NNVC-1.0 in terms of BDBR and BD-PSNR.

Table 3. Performance comparison with VTM11.0 NNVC-1.0 on all 2D video datasets in terms of BDBR (%) and BD-PSNR (dB).

|  | BD-BR (%) |  |  | BD-PSNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A1 | 5.66 | 3.66 | 3.86 | -0.2543 | -0.1681 | -0.1935 |
| Class B | 3.70 | 3.93 | 1.56 | -0.1806 | -0.2054 | -0.0048 |
| Class C | 3.56 | 4.39 | 5.56 | -0.1573 | -0.2146 | -0.2387 |
| Class D | 1.63 | 6.13 | 2.67 | -0.0056 | 0.2684 | 0.1273 |
| Average | 3.64 | 4.53 | 3.41 | -0.1495 | -0.2141 | -0.1411 |

Running time

We also provide runtime comparison of encoding and decoding with DOVC and VTM11.0 NNVC-1.0 on 2D video sequences as shown in Table 4. Since DOVC performs global optimization at the frame level and takes advantage of implementing GPU (i.e. DGX Station with GTX3090) parallel processing for encoding and decoding, the average encoding time of DOVC is only 0.025 times that of VTM11.0 NNVC-1.0. For decoding time, DOVC is a bit higher than VTM-11.0 NNVC-1.0 because the GPU-based high performance computing is not very effective for decoding with low complexity.

Table 4. Comparison of running time between DOVC and VTM11.0 NNVC-1.0.

|  | VTM11.0 NNVC-1.0 |  | DOVC |  |
| --- | --- | --- | --- | --- |
|  | Enc T (s) | Dec T (s) | Enc T (s) | Dec T (s) |
| Class A1 | 201283.27 | 101.23 | 3824.53 | 132.57 |
| Class B | 90587.71 | 49.85 | 2306.32 | 96.46 |
| Class C | 23396.54 | 10.41 | 1363.18 | 32.69 |
| Class D | 5917.81 | 2.95 | 602.65 | 22.02 |
| Average | 80296.33 | 41.11 | 2024.17 | 70.94 |

Conclusion

In this contribution, we provide supplementary experiments based on JVET-Z0077 [1]. Specifically, VTM11.0 NNVC-1.0 is used as the anchor to compare with DOVC, which is configured with VTM intra coding instead of BPG for image compression. Experimental results demonstrate that DOVC can be successfully applied to the omnidirectional video and regular 2D video compression.

Therefore, we recommend continuing the study of DOVC in EE1 as a test case for end-to-end video compression.

References

[1] Q. Qin, C. Jung, D. Zou, and M. Li, “[AHG11] Deep Omnidirectional Video Compression in YUV domain,” JVET-Z0077, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20 – 29 April. 2022.

[2] Q. Qin, C. Jung, D. Zou, and M. Li, “[AHG11 & AHG6] DOVC: Deep Omnidirectional Video Compression,” JVET-X0043, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6 – 15 Oct. 2021.

[3] Q. Qin, C. Jung, D. Zou, and M. Li, “[AHG11] Deep Omnidirectional Video Compression in YUV domain,” JVET-Y0051, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12 – 21 Jan. 2022.

Patent rights declaration(s)

Xidian University and Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).