| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0065-v1 |
| --- | --- |

| Title: | AHG11: CNN Filter for Super-Resolution with RPR functionality in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shimin Huang Cheolkon Jung Yang Liu Ming Li | Email: | shimin_huang2022@163.com zhengzk@xidian.edu.cn serena@oppo.com myron.li@oppo.com |
| Source: | Xidian University, OPPO |  |  |

_____________________________

- Abstract

This contribution presents a super-resolution network that combines CNN with existing RPR functionality in VVC, called MMSDANet. In MMSDANet, we propose a new basic block, multi-mixed scale and depth information with attention block (MMSDAB) to extract multi-scale and convolutional layer depth information, and shared convolution is used to reduce network parameters. Compared with VTM-11.0 RPR anchor, MMSDANet achieves {-6.72%, -26.89%, -28.19%} and {-8.16%, -25.32%, -26.30%} BD-rate gains on average for {Y, Cb, Cr}, under RA and AI configurations, respectively. Compared with VTM-11.0-NNVC MMSDANet achieves {-4.21%, 4.53%, -9.55%} and {-8.5%, 18.78%, -12.61%} BD-rate gains on average for {Y, Cb, Cr}, under RA and AI configurations, respectively.

Introduction

With the significant success of deep learning in computer vision and image processing, some researchers are trying to combine deep learning with video compression to develop novel video coding frameworks, named as deep learning-based video coding (DLVC) or deep video compression (DVC). The neural network for video coding (NNVC) was first discussed at the 20-th JVET meeting, which mainly used deep learning tools to replace some modules of VVC such as loop filtering, inter-frame prediction, upsampling, and specified common test conditions (CTC). JVET NNVC exploration activities are mainly organized by AHG11. VVC provides the reference picture resampling (RPR) functionality that upsamples the decoded video frame to the original resolution. Recently, deep learning has resulted in great breakthroughs in image super-resolution that reconstructs a low resolution image to the target resolution. Thus, CNN-based super-resolution can replace the upsampling algorithm in RPR functionality. In the RPR functionality, the current frame for encoding is first downsampled to reduce the transmission bitstream under the limited bandwidth. At the decoding end, the downsampled frame should be upsampled to its original resolution after decoding. However, the traditional interpolation methods have a limit of considering complicated characteristics of videos. In JVET-W0132 [1], Ysagui et al. introduced neural network-based super-resolution that mainly extracted features by stacking residual blocks and verified several different upsampling methods: 1) One convolutional layer + one pixel shuffle layer, 2) interpolation. In JVET-Y0070 [2], Lin et al. proposed a network structure for super-resolution that was similar to JVET-W0132. Unlike JVET-W0132 (the input of JVET-W0132 was only low resolution Y channel), the input of JVET-Y0070 [2] was composed of the low resolution Y channel, the prediction frame Y channel and QP information, thus its super-resolution performance is significantly improved thanks to the multiple inputs. In JVET-Y0087[3], Peng et al. added channel attention into the residual block to enhance important features and suppress noise.

In this contribution, we propose a CNN filter for super-resolution with RPR functionality in VVC. We design a multi-level mixed scale and depthwise attention network for RPR-based SR in VVC, called MMSDANet. The main contributions are as follows:

- CNN filter for RPR-based SR in VVC: MMSDANet is a CNN filter for RPR-based SR in VVC which can be embedded within the VVC codec. MMSDAnet is based on residual learning to accelerate network convergence and reduce training complexity. MMSDANet effectively extracts low-level features in a U-Net structure by stacking MMSDABs, and transfers them to the high-level feature extraction module through U-Net connections.

- Multi-scale and layerwise network based on an attention block: We use a novel MMSDAB as the basic block for RPR-based SR. MMSDAB firstly extracts multi-scale and layerwise information, and then uses the attention mechanism to enhance the important information weakening the unimportant information.

- Lightweight network based on shared convolution: We use shared convolutional layers to capture multi-scale and depth layer information, which can reduce a lot of network parameters.

Proposed method

MMSDANet for Luma Channel

MMSDANet consists of 8 MMSDABs, three convolutional layers, one concatenate layer, one shuffle layer and five shortcut connections. Except for the 1x1 and last convolutional layers, all convolutional layers and MMSDABs have 64 input and output channels. 1x1 convolution is used to reduce the number of channels and reduce the complexity of the operation. The output channel of the last convolutional layer is 4 so that the pixel shuffle layer can do 2x upsampling. Since the input and output resolutions are different, the input needs to be upsampled when applying global connections. This model can be roughly divided into three parts: head, backbone, and upsampling:

The head part consists of a convolutional layer, which is used to extract the shallow features of the input image. The convolutional layer is followed by an ReLU activation function. Given input YLR, through the head network ψ, we get the shallow feature f0 as follows:

f0=ψ(YLR)

The backbone part is the key component of the network which is the most unique in the proposed network. The backbone part is composed of M MMSDABs. The backbone part uses f0 as input, concatenates the MMSDAB outputs, and finally reduces the number of channels by a 1x1 convolution to get fftand put it to the reconstruction part. To make full use of low-level features, we use the connection method in U-Net to add fi and fM-i as the input of ωM-i+1 as follows:

fM-i+1=ωM-i+1fi+fM-i      0<i<M/2

fft=Conv(C[ωM,ωM-1,…ω1f0])+ f0

where ωi represents the M-th MMSDAB, C[.] represents the channel concatenation, and fi represents the output of the M-th MMSDAB.

The upsampling part consists of one convolutional layer and one pixel shuffle. The upsampling network is expressed as:

YHR=PS(Convfft)+US(YLR)

where YHR is the upsampled image, PS is the pixel shuffle layer, Conv represents the convolutional layers and US represents the upsampling.

Fig. 1 Illustration of the proposed MMSDANet for luma channel.

MMSDANet for Chroma Channels

For the chroma channels, the backbone and pixel shuffle layer of MMSDANet are consistent with the luma channel, and their main difference lies in the input and head of the network. Figs. 2 and 3 illustrate the network architecture of MMSDANet for U and V channels, respectively. To utilize the cross-component correlation, the reconstructed luma component is taken as one input. For the network applied to the chroma channels, its input contains three channels of Y, U, and V. The size of the guided component Y is twice of the U/V channel, thus the Y channel needs to be first downsampled. We choose to use a 3x3 convolution with stride 2 for downsampling.

Fig. 2 Illustration of the proposed MMSDANet for U channel.

Fig. 3 Illustration of the proposed MMSDANet for V channel.

MMSDAB

Fig. 3 shows the network structure of MMSDAB. MMSDAB consists of seven convolutional layers, one concatenate layer, one channel shuffle layer and one SE attention layer. The convolutional layer is used to extract the scale information and depth information of the image features, while the SE attention [4] layer is used to enhance important information.

Fig. 4 Illustration of the proposed MMSDAB.

Train and inference

The network information in the inference stage is provided in Table 1.

Table 1 Network information for the proposed model in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Intel Core i9 12900k @3.9GHz |
|  | Framework: | LibTorch v1.9 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | luma up-sampling model: 2.0M/model chroma up-sampling model: 2.1M/model |
|  | Total Parameter Number | 31M |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 15 models in total: 118 MB |
|  | Multiply Accumulate (MAC) | 2044kMAC/pixel |
| Optional | Total Conv. Layers | 75 for up-sampling the luma, 77 for up-sampling the chroma |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 |
|  | Patch size | Whole frame |

DIV2K [5] and BVI_DVC [6] datasets are used for training. The official NNVC anchor (VTM-11.0 [7] + new MCTF [8]) is utilized to compress these images with QPs {22, 27, 32, 37, 42}. In the context of RPR scheme, the training data will be down-sampled by a factor of 2 before encoding.

Then, we use the decoded low-resolution images and the corresponding original images for training. We train a separate model for each QP for the three channels Y, U, V, thus there are 15 models in total. The models are fixed. Compared to VTM 11.0, the bitstream is reduced a lot because the video frame is downsampled to half of the original size. Compared to RPR2, the bitstream is increased a little due to the flag bit of using the proposed CNN filter.

Table 2 Network information for the proposed model in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA 3090 24GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | luma:50, chroma:30 |
|  | Batch size: | 64 |
|  | Training time: | 50h/model |
|  | Training data information: | DIV2K, BVI_DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {22, 27, 32, 37, 42} |
|  | Loss function: | L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Experimental results

The proposed CNN-based up-sampling filter is tested on the official NNVC anchor (VTM-11.0 + new MCTF), according to the common test conditions (CTC) [6]. The QP values are {22, 27, 32, 37,42} for tests.

Visual Comparison

Fig. 6 shows visual quality comparison in BasketballDrill and RaceHorses sequences of Class C. It can be observed that the upsampling results by MMSDANet are better than those by the RPR configuration in VTM. However, its visual quality is not good as the compressed image without the RPR configuration. It is obvious that MMSDANet recovers more details and boundary information than RPR upsampling. Compared with (c) without the RPR configuration, some areas in (e) are over-smooth.

Fig. 5 Visual quality comparison in BasketballDrill and RaceHorses sequences of Class C. (a) Low-resolution image compressed at QP 22 after downsampling of the original image. (b) Uncompressed high-resolution image. (c) Image compressed at QP 22 without the RPR configuration by VTM 11.0 NNVC_1.0. (d) Upsampling result by the RPR configuration. (d) Upsampling result by MMSDANet.

Quantitative Measurements

We compare the performance of the proposed CNN filter with those of two anchors: VTM 11.0 NNVC_1.0 anchor (NNVC anchor) and VTM 11.0 NNVC_1.0 with RPR functionality (RPR anchor). Since the bitrate difference is too large between RPR anchor and NNVC anchor, it may cause inaccurate calculation of BD-rate. Therefore, we also provide the comparison results with the RPR anchor. Under the condition of the same bitrate, the calculated BD-rate gain may be more accurate. RPR2 indicates that the video sequence is compressed using the RPR configuration with the scaling factor of 2.

For all intra (AI) encoding, all video frames to be encoded will be encoded as I-frames. The so-called I frame means that it can be encoded independently without other references and using a specified QP in the configuration file for all frames.

For random access (RA) encoding, For RA encoding, it is encoded in groups, and a group is called a GOP. There are two I-frames in a GOP, while the rest are B-frames. The I-frames are independently encoded, and the B-frames are encoded with reference to the encoded I-frames or B-frames. In addition, QP of B-frames during encoding is adjusted based on QP in the configuration file, which is no longer a fixed parameter.

Tables 3 and 4 show the experimental results of RPR with factor 2 (RPR2) as:

- Under RA configuration, the proposed method can bring {-6.72%, -26.89%, -28.19%} BD-rate changes on average for {Y, U, V} components, respectively.

- Under AI configuration, the proposed method can bring {-8.16%, -25.32%, -26.30%} BD-rate changes on average for {Y, U, V} components, respectively.

Table 3 Results of the proposed method for RA and RPR2 configurations.

|  |  | Random Access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + New MCTF (QP 22, 27, 32, 37, 42) |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT | bit DIFF |
| Cass A1 4K | Tango2 | -2.13% | -25.15% | -21.55% | #DIV/0! | #DIV/0! | 0.00 |
|  | FoodMarket4 | -2.69% | -12.95% | -15.14% | #DIV/0! | #DIV/0! | 0.00 |
|  | Campfire | -9.02% | -21.69% | -35.82% | #DIV/0! | #DIV/0! | 0.00 |
| Class A2 4K | CatRobot1 | -9.10% | -32.85% | -33.57% | #DIV/0! | #DIV/0! | 0.00 |
|  | DaylightRoad2 | -11.68% | -25.11% | -25.48% | #DIV/0! | #DIV/0! | 0.00 |
|  | ParkRunning3 | -5.70% | -45.36% | -37.56% | #DIV/0! | #DIV/0! | 0.00 |
| Average on A1 |  | -4.61% | -19.93% | -24.17% | #DIV/0! | #DIV/0! | 0.00 |
| Average on A2 |  | -8.82% | -34.44% | -32.20% | #DIV/0! | #DIV/0! | 0.00 |
| Overall |  | -6.72% | -27.19% | -28.19% | #DIV/0! | #DIV/0! | 0.00 |

Table 4 Results of the proposed method for AI and RPR2 configurations.

|  |  | Random Access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + New MCTF (QP 22, 27, 32, 37, 42) |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT | bit DIFF |
| Cass A1 4K | Tango2 | -3.84% | -19.42% | -22.18% | #DIV/0! | #DIV/0! | 0.00 |
|  | FoodMarket4 | -3.88% | -11.94% | -14.27% | #DIV/0! | #DIV/0! | 0.00 |
|  | Campfire | -11.91% | -33.32% | -39.87% | #DIV/0! | #DIV/0! | 0.00 |
| Class A2 4K | CatRobot1 | -9.61% | -27.58% | -28.43% | #DIV/0! | #DIV/0! | 0.00 |
|  | DaylightRoad2 | -12.40% | -21.74% | -20.44% | #DIV/0! | #DIV/0! | 0.00 |
|  | ParkRunning3 | -7.31% | -37.89% | -32.58% | #DIV/0! | #DIV/0! | 0.00 |
| Average on A1 |  | -6.54% | -21.56% | -25.44% | #DIV/0! | #DIV/0! | 0.00 |
| Average on A2 |  | -9.77% | -29.07% | -27.15% | #DIV/0! | #DIV/0! | 0.00 |
| Overall |  | -8.16% | -25.32% | -26.30% | #DIV/0! | #DIV/0! | 0.00 |

Table 3 and table 4 shows the experimental results of RPR with factor 2 compared to VTM-11.0_NNVC:

- Under RA configurations, the proposed method can bring {-4.21%, 4.53%, -9.55%} BD-rate changes on average for {Y, U, V} components, respectively.

- Under AI configurations, the proposed method can bring {-8.5%, 18.78%, -12.61%} BD-rate changes on average for {Y, U, V} components, respectively.

Table 5 Results of the proposed method for RA configurations

|  |  | Random Access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + New MCTF (QP 22, 27, 32, 37, 42) |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT | bit DIFF |
| Cass A1 4K | Tango2 | -7.21% | -25.62% | -15.34% | #DIV/0! | #DIV/0! | 55% |
|  | FoodMarket4 | -6.11% | -9.98% | -11.33% | #DIV/0! | #DIV/0! | 54% |
|  | Campfire | -17.64% | 37.67% | -23.28% | #DIV/0! | #DIV/0! | 65% |
| Class A2 4K | CatRobot1 | 3.54% | -11.06% | -3.36% | #DIV/0! | #DIV/0! | 55% |
|  | DaylightRoad2 | 12.81% | -19.84% | -15.21% | #DIV/0! | #DIV/0! | 58% |
|  | ParkRunning3 | -10.65% | 55.99% | 11.25% | #DIV/0! | #DIV/0! | 68% |
| Average on A1 |  | -10.32% | 0.69% | -16.65% | #DIV/0! | #DIV/0! | 58% |
| Average on A2 |  | 1.90% | 8.36% | -2.44% | #DIV/0! | #DIV/0! | 60% |
| Overall |  | -4.21% | 4.53% | -9.55% | #DIV/0! | #DIV/0! | 59% |

Table 6 Results of the proposed method for AI configurations

|  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + New MCTF (QP 22, 27, 32, 37, 42) |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT | bit DIFF |
| Cass A1 4K | Tango2 | -9.45% | -17.31% | -15.16% | #DIV/0! | #DIV/0! | 47% |
|  | FoodMarket4 | -4.48% | -6.65% | -6.73% | #DIV/0! | #DIV/0! | 45% |
|  | Campfire | -12.88% | 149.67% | -23.80% | #DIV/0! | #DIV/0! | 68% |
| Class A2 4K | CatRobot1 | -7.72% | -11.63% | -9.82% | #DIV/0! | #DIV/0! | 52% |
|  | DaylightRoad2 | -2.67% | -22.70% | -19.24% | #DIV/0! | #DIV/0! | 55% |
|  | ParkRunning3 | -13.80% | 21.27% | -0.90% | #DIV/0! | #DIV/0! | 63% |
| Average on A1 |  | -8.94% | 41.90% | -15.23% | #DIV/0! | #DIV/0! | 53% |
| Average on A2 |  | -8.06% | -4.35% | -9.99% | #DIV/0! | #DIV/0! | 57% |
| Overall |  | 8.50% | 18.78% | -12.61% | #DIV/0! | #DIV/0! | 55% |

Conclusions

This contribution proposes a CNN filter for super-resolution with RPR functionality in VVC. We have embedded the CNN filter into VTM and replaced RPR upsampling to achieve super-resolution reconstruction from compressed video frames. MMSDAnet uses residual learning to accelerate network convergence and reduce training complexity. MMSDANet adopts a U-Net structure as the network backbone and stacks MMSDABs to extracting low-level features. Various experiments show that the proposed CNN filter achieves SR reconstruction while successfully removing compression artifacts.

We recommend further study on the proposed CNN filter by including it in EE1.

References

- Y. Ysagui, T. Chujoh, T. Ikai, AHG11: 1.5x/2.0x Upsample method for NN-Based Super-Resolution Post-Filters, JVET-W0132.

- C. Lin, Y. Li, and K. Zhang, EE1-2.4: CNN-based Super Resolution for Video Coding Using Separate Networks for Chroma Components, JVET-Y0070.

- S. Peng, D. Jiang, and J. Lin, AHG11: An Improved CNN-based Super Resolution Method, JVET-Y0087.

- H. Jie, S. Li, and S. Gang, “Squeeze-and-Excitation Networks,” Proceedings of the IEEE Conference on Computer Vision and Pattern Recognition (CVPR), pp. 7132-7141, 2018.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/, R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- D. Ma, F. Zhang, and D. R. Bull, BVI-DVC: A Training Database for Deep Video Compression (https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn).

- VTM-11.0 https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-11.0

- P. Wennersten, C. Hollmann, and J. Ström, GOP-based temporal filter improvements, JVET-V0056.

- S. Liu, A. Segall, E. Alshina, and R. -L. Liao, JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-X2016.

Patent rights declaration(s)

Xidian University and Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).