| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0070-v2 |
| --- | --- |

| Title: | EE2-3.2: Reconstruction-Reordered IBC for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2-3.2 on Reconstruction-Reordered IBC for screen content coding. In the EE test, the IBC coding efficiency can be further improved by allowing reconstruction block flipping. Simulation results based on ECM-5.0 with PLT turned on are reported as below:

AI: Class F -1.43%, -1.74%, -1.67%, 102%, 100%; Class TGM -3.49%, -3.45%, -3.43%, 100%, 99%

RA: Class F -1.00%, -0.83%, -0.96%, 101%, 100%; Class TGM -1.39%,	-1.16%,	-1.18%,	100%, 99%

LB: Class F -0.65 %, -1.32%, -0.82%, 101%, 100%; Class TGM -0.60%,	-0.46%,	-0.40%,	101%, 100%

In v2, an updated simulation results are reported. With the EE frozen code, the encoder may result in one-frame difference in few sequences on different platforms, due to different implementations of the C++ standard library. This was identified by the crosschecker who is using a testing environment with a different C++ complier. After solving this cross-platform issue, the simulation results were confirmed match between the proponent and crosscheckers. The updated code has been uploaded to the EE2 git repository before the JVET-AA meeting. Below are the simulation results of the updated source code based on ECM-5.0 with PLT turned on:

AI: Class F -1.43%, -1.74%, -1.67%, 102%, 99%; Class TGM -3.49%, -3.44%, -3.42%, 101%, 100%

RA: Class F -1.00%, -0.83%, -0.96%, 101%, 99%; Class TGM -1.38%, -1.14%, -1.16%, 100%, 100%

LB:

# Introduction

Symmetry is often observed in video content, especially in text character regions and computer-generated graphics in screen content sequences. In JVET-Z0159 [1], a Reconstruction-Reordered IBC (RRIBC) mode was proposed for screen content video coding to further improve the coding efficiency of IBC in the ECM. At the JVET-Z meeting, RRIBC was included in exploration experiment (EE) 2 Test 3.2 [2] for further evaluation. This contribution reports the results of EE2-3.2.

# Proposal

When RRIBC is applied, the samples in a reconstruction block are flipped according to the flip type of the current block. At the encoder side, the original block is flipped before motion search and residual calculation, while the prediction block is derived without flipping. At the decoder side, the reconstruction block is flipped back to restore the original block.

Specifically, in EE2-3.2, two flip methods, horizontal flip and vertical flip, are supported for RRIBC coded blocks. A syntax flag is firstly signalled for an IBC AMVP coded block, indicating whether the reconstruction is flipped, and if it is flipped, another flag is further signaled specifying the flip type. For IBC merge, the flip type is inherited from neighbouring blocks, without syntax signalling. Considering the horizontal or vertical symmetry, the current block and the reference block are normally aligned horizontally or vertically. Therefore, when a horizontal flip is applied, the vertical component of the BV is not signaled and inferred to be equal to 0. Similarly, the horizontal component of the BV is not signaled and inferred to be equal to 0 when a vertical flip is applied.

To better utilize the symmetry property, a flip-aware BV adjustment approach is applied to refine the block vector candidate. For example, as shown in Figure 1, (xnbr, ynbr) and (xcur, ycur) represent the coordinates of the center sample of the neighbouring block and the current block, respectively, BVnbr and BVcur denotes the BV of the neighbouring block and the current block, respectively. Instead of directly inheriting the BV from a neighbouring block, the horizontal component of BVcur is calculated by adding a motion shift to the horizontal component of BVnbr (denoted as BVnbrh) in case that the neighbouring block is coded with a horizontal flip, i.e., BVcurh =2(xnbr -xcur) + BVnbrh. Similarly, the vertical component of BVcur is calculated by adding a motion shift to the vertical component of BVnbr (denoted as BVnbrv) in case that the neighbouring block is coded with a vertical flip, i.e., BVcurv =2(ynbr -ycur) + BVnbrv.

- (b)

Figure 1. Illustration of BV adjustment for (a) horizontal flip, and (b) vertical flip, respectively.

# Experimental Results

The proposed method was implemented on top of ECM-5.0 [3]. Experiments were conducted following the common test conditions [4] for enhanced compression tool testing and EE2 description for screen content coding (SCC) tools. Simulation results of PLT turned on is tabulated in Table 1.

Table 1: Performance of EE2-3.2 (PLT turned on)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 + PLT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.43% | -1.74% | -1.67% | 102% | 100% |
| Class TGM | -3.49% | -3.45% | -3.43% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 + PLT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.00% | -0.83% | -0.96% | 101% | 100% |
| Class TGM | -1.39% | -1.16% | -1.18% | 100% | 99% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-5.0 + PLT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.65% | -1.32% | -0.82% | 101% | 100% |
| Class TGM | -0.60% | -0.46% | -0.40% | 101% | 100% |

Simulation results of the updated source code is tabulated in Table 2.

Table 2: Performance of EE2-3.2 (PLT turned on)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 + PLT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.43% | -1.74% | -1.67% | 102% | 99% |
| Class TGM | -3.49% | -3.44% | -3.42% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 + PLT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.00% | -0.83% | -0.96% | 101% | 99% |
| Class TGM | -1.38% | -1.14% | -1.16% | 100% | 100% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-5.0 + PLT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

# Conclusions

The contribution reports the results of EE2-3.2 on Reconstruction-Reordered IBC for screen content coding. It is asserted that the proposed method can provide promising gains with a minor hardware/software implementation cost. It is proposed to adopt the proposed method to the ECM.

# References

- Zhipin Deng, Kai Zhang, Li Zhang, “Non-EE2: Reconstruction-Reordered IBC for screen content coding”, JVET-Z0159, Apr. 2022.

- V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-Z2024, Apr. 2022.

- ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-5.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).