| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0073-v2 |
| --- | --- |

| Title: | Non-EE2: Modification of LFNST for MIP coded block |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Wenhan Qiao, Xue Hao, Yanzhuo Ma, Fuzheng Yang, Jing Ren, Ming Li | Email: | jyhuo@mail.xidian.edu.cn whqiao@stu.xidian.edu.cn xhao@stu.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn renjing1@oppo.com myron.li@oppo.com |
| Source: | Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

In ECM5.0, for a MIP coded block, LFNST transform set 0 is used and LFNST transpose flag is equal to 0. In this contribution, modifications of LFNST for MIP coded blocks are proposed. It is proposed to use DIMD to derive a traditional intra predition mode based on the prediction samples before interpolation of MIP modes, then the LFNST transform set and LFNST transpose flag are both determined by the derived intra mode. Moreover, LFNST is enable for MIP coded blocks of width and height greater than or equal to 4. It is reported that the coding performance to ECM 5.0 are as below:

For AI configuration: -0.11%, -0.08%, and -0.12%, with 105% EncT, 100% DecT.

For RA configuration: -0.07%, -0.06%, and -0.12%, with xxx% EncT, xxx% DecT.

Introduction

In ECM5.0[1], the LFNST transform set and LFNST transpose flag are both determined by the intra prediction mode predModeIntra of the current transform block. With the predModeIntra, the following operation is conducted:

- If the current block is MIP coded block, predModeIntra is mapped to PLANAR;

- If the current block is CCLM coded block, predModeIntra is mapped to the co-located luma intra prediction mode;

Then, predModeIntra is further derived from wide angle intra prediction mapping with a range of [-14, 83].

- 1.1 Selection of LFNST transform sets

There are totally 35 transform sets and 3 non-separable transform matrices (kernels) per transform set in LFNST in ECM5.0. The transform set idx lfnstTrSetIdx is defined according to predModeIntra list in Table 1.

Table 1. LFNST transform set selection

- 1.2 Determination of LFNST transpos flag

The LFNST transpose flag determines the scan order of the LFNST output (Decoder). Figure 1 shows the scan order with different LFNST transpose flag.

Figure 1. Scan order with different LFNST transpose flag

The LFNST transpose flag is determined by predModeIntra as:

- if predModeIntra is less than or equal to 34, the LFNST transpose flag is set to 0

- else, the LFNST transpose flag is set to 1

For MIP coded blocks, it is mapped to the PLANAR mode, the LFNST transform set 0 is used and LFNST transpose flag is always equal to 0.

In ECM5.0, LFNST is enable for the MIP codeds blocks with the width and height greater than or equal to 16.

- 1.3 Prediction Process of MIP

Matrix weighted intra prediction (MIP) takes one line of H reconstructed neighbouring boundary samples left of the block and one line of W reconstructed neighbouring boundary samples above the block as input. The generation of the prediction samples is based on the following three steps, which are boundary downsampling, matrix vector multiplication and MIP prediction upsampling as shown in Figure 2.

Figure 2. Matrix weighted intra prediction process

Specifically, MIP first downsamples the reference samples, and then multiplies the downsampled reference samples with the prediction matrix to generate part of the prediction samples. Finally, it is upsampled to generate predicted samples at the remaining positions.

Proposed Methods

In this contribution, it is proposed to utilize DIMD to derive the LFNST transform set and determine LFNST transpose flag.

The proposed method uses the DIMD to derive the intra prediction mode of the current block based on the MIP predicted samples before upsampling. Specifically, a horizontal gradient and a vertical gradient are calculated for each predicted sample to build a HoG, as shown in Figure 3. Then the intra prediction mode with the largest histogram amplitude values is used to determine the LFNST transform set and LFNST Transpose flag.

Figure 3. MIP prediction samples to build HOG

Futhermore, LFNST is enable for MIP coded blocks of width and height greater than or equal to 4.

Experimental results

The proposed method was implemented based on ECM-5.0 under common test condition [2]. Table 2 presents a summary of the results with all intra and random access configurations.

Table 2. Simulation results of the proposed method

|  |  |  | All Intra Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.02% | -0.23% | 105% | 100% |
| Class A2 | -0.08% | -0.09% | -0.10% | 105% | 99% |
| Class B | -0.09% | -0.12% | -0.13% | 105% | 100% |
| Class C | -0.14% | -0.16% | -0.03% | 104% | 99% |
| Class E | -0.17% | 0.06% | -0.13% | 105% | 101% |
| Overall | -0.11% | -0.08% | -0.12% | 105% | 100% |
| Class D | -0.15% | 0.05% | -0.04% | 104% | 100% |
| Class F | -0.09% | 0.02% | -0.04% | 103% | 102% |

Table 2. Simulation results of the proposed method

|  |  |  | Random Access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.01% | -0.21% | #NUM! | #NUM! |
| Class A2 | -0.06% | 0.02% | 0.04% | #NUM! | #NUM! |
| Class B | -0.06% | -0.14% | -0.21% | #NUM! | #NUM! |
| Class C | -0.08% | -0.06% | -0.07% | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | -0.07% | -0.06% | -0.12% | #NUM! | #NUM! |
| Class D | -0.07% | -0.04% | 0.06% | #NUM! | #NUM! |
| Class F | -0.04% | -0.05% | -0.17% | #NUM! | #NUM! |

Conclusions

In this contribution, an alternative method of LSNFT transform set selection and LFNST transpose flag determination are designed for MIP coded blocks. The performance of the proposed method is -0.11% (AI) and    -0.07% (RA) over ECM 5.0. It is suggested to investigate in EE.

- 5 References

- ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).