| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0079-v1 |
| --- | --- |

| Title: | AHG9: Decoded picture hash SEI message extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P.Bordes, F.Galpin, P. de Lagrange, E.Francois | Email: | philippe.bordes@interdigital.com franck.galpin@interdigital.com philippe.delagrange@interdigital.com edouard.francois@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes syntax and semantics for extension of the Decoded picture hash SEI message to support several checksum hashes at different steps of the decoded pictures reconstruction process.

# Introduction

The Decoded picture hash supplemental enhancement information (SEI) message [1] provides a hash for the current decoded picture. This message is intended for debugging and interoperability testing and allows a decoder determining whether the reconstructed picture exactly matches that of the encoder.

At the last JVET meeting, a neural-network (NN) post-filter SEI draft text [2] has been adopted for VSEI amendment, which endorses an additional post-processing of the decoded picture to reconstruct enhanced picture. In VSEI draft [3], other SEIs are also including, signaling parameters for post-decoding processes (Film grain characteristics SEI, Colour transform information SEI).

Figure 1: example of pipeline process for decoding pictures.

# Proposed amendment to the Decoded picture hash SEI

The VVC specification includes a mechanism for supporting further extension of SEI messages. One proposes some extension to the Decoded picture hash SEI message to support the following additional features:

- The amended syntax should be backward compatible with the previous one, so that the deployed decoders (compliant with current H.274 VSEI format) can correctly decode the part of the syntax related to the former Decoded picture hash SEI message and ignore the additional information.

- The new syntax can signal additional hash checksums corresponding to other conformance points than decoded picture H1.

- The new syntax should allow supporting the definition of future conformance points in further amendments possibly.

Table 1: proposed syntax of decoded picture hash SEI extension.

| decoded_picture_hash_extension( payloadSize ) { |  |
| --- | --- |
| dph_sei_nb_conformance_point_minus1 | u(8) |
| for( i = 0; i < dph_sei_nb_conformance_point; i++ ) |  |
| { |  |
| dphm_sei_hash_conf_id[ i ] | u(8) |
| dphm_sei_hash_type[ i ] | u(8) |
| dphm_sei_single_component_flag[ i ] | u(1) |
| dphm_sei_reserved_zero_7bits[ i ] | u(7) |
| for( cIdx = 0; cIdx < ( dph_sei_single_component_flag[ i ] ? 1 : 3 ); cIdx++ ) |  |
| if( dph_sei_hash_type = = 0 ) |  |
| for( j = 0; j < 16; j++) |  |
| dph_sei_picture_md5[ i ] [ cIdx ][ j ] | b(8) |
| else if(dph_sei_hash_type = = 1 ) |  |
| dphm_sei_picture_crc[ i ] [ cIdx ] | u(16) |
| else if(dph_sei_hash_type = = 2 ) |  |
| dphm_sei_picture_checksum[ i ] [ cIdx ]. | u(32) |
| } |  |
| } |  |

dph_sei_nb_conformance_point_minus1 plus 1 specifies the number of additional checksum points in the Decoded picture hash SEI message.

dphm_sei_hash_conf_id[ i ] indicates the position of the i-th checksums of the decoded picture in the pipeline process of reconstructing the pictures as specified in Table 2. Values of dphm_sei_hash_conf_id[ i ] that are not listed in Table 2 are reserved for future use by ITU-T | ISO/IEC and shall not be present in payload data conforming to this version of this Specification. Decoders shall ignore decoded picture hash SEI messages that contain reserved values of dphm_sei_hash_conf_id[ i ].

Table 2:Interpretation of dphm_sei_hash_conf_id.

| dphm_sei_hash_conf_id[ i ] | Position of checksum |
| --- | --- |
| 0 | Rec samples after last post-filter enhancement |
| 1 | Rec samples after film grain characteristics |
| 2 | Rec samples after colour transformation information |
| 3 | GDR rec samples in the clean area |

The semantics of dphm_sei_hash_type[i], dphm_sei_single_component_flag[i], dphm_sei_picture_md5[i], dphm_sei_picture_crc[i], dphm_sei_picture_checksum[i] are same as dph_sei_hash_type, dph_sei_single_component_flag, dph_sei_picture_md5, dph_sei_picture_crc, dph_sei_picture_checksum for the i-th conformance point.

# References

- ITU-T H.274, “Versatile supplemental enhancement information messages for coded video bitstreams.”

- M. Hannuksela et al., ” AHG9: NN post-filter SEI,” document JVET-Z0244, 26th Meeting, by teleconference, 20–29 April 2022.

- J.Boyce, G.Sullivan, Y.Wang, “Additional SEI messages for VSEI (Draft 4),” document JVET-W2006, 23rd Meeting, by teleconference, 7–16 July 2021.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).