| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0081-v1 |
| --- | --- |

| Title: | EE1-1.2: NN intra model without attention, partitioning and boundary strength. |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jacob Ström, Du Liu, Mitra Damghanian, Kenneth Andersson, Yun Li, Per Wennersten and Ruoyang Yu Torshamnsgatan 23 164 83 Stockholm | Tel: Email: | +46702672192 {jacob.strom, du.liu, mitra.damghanian, kenneth.r.andersson, yun.y.li, per.wennersten, ruoyang.yu} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents additional results to the ablation study in JVET-Z0106. In that study, the intra model from JVET-Y0143 was retrained several times with some aspect of the neural network changed. In this contribution, further combinations are tested. Test EE1-1.2.1 involves removing the partitioning input as well as the attention branch from the intra model. Test EE1-1.2.2 involves removing both the partitioning and boundary strength inputs as well as the attention branch from the intra model. The reported BD-rate results over the VTM-11.0 + newMCTF anchor are:

EE1-1.2.1: -7.56% (Y), -18.48% (U) -19.09% (V) for AI, worst case complexity 635 kMAC/s.

EE1-1.2.2: -7.48% (Y), -18.64% (U) -19.17% (V) for AI, worst case complexity 625 kMAC/s.

It is proposed to adopt one of these two models as the intra luma model.

# Introduction

The same training procedure was used as in JVET-Z0106 [1]: Training for 371 epochs with L1 error metrics and learning rate 10-4, and then training another 7 epochs with MSE error metrics and learning rate 10-5, resulting in 378 epochs in total. Adding the results of this EE to the last two rows of Table 1 from JVET-Z0106 gives the following table. Complexity numbers were obtained using the thop library version 0.0.31-200524190.

Table 1: Summary of ablation study for the intra luma model

| Test | BDR-Y (AI) | Inputs used | kMACs per sample (Y) | kMACs per pixel (YUV) | Num. of param total (106) | Mem total (106 bytes) | Epochs trained | Training time (h) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JVET-Y0143 [2] | -7.39% | rec, pred, part, BS, QP | 429 | 649 | 6.24 | 24.94 | 901 | 601 |
| Retrain 60 h | -7.38% | rec, pred, part, BS, QP | 429 | 649 | 6.24 | 24.94 | 155 | 60 |
| Retrain 146 h | -7.57% | rec, pred, part, BS, QP | 429 | 649 | 6.24 | 24.94 | 378 | 146 |
| Without part | -7.57% | rec, pred, BS, QP | 418 | 638 | 6.22 | 24.89 | 378 | 138 |
| Without BS | -7.56% | rec, pred, part, QP | 418 | 638 | 6.22 | 24.89 | 378 | 138 |
| Without part and BS | -7.42% | rec, pred, QP | 408 | 628 | 6.21 | 24.84 | 378 | 131 |
| Without pred | -7.35% | rec, part, BS, QP | 418 | 638 | 6.22 | 24.89 | 378 | 138 |
| 4 res. blocks | -6.67% | rec, pred, part, BS, QP | 261 | 481 | 5.56 | 22.26 | 378 | 131 |
| 16 res. blocks | -8.28% | rec, pred, part, BS, QP | 765 | 985 | 7.58 | 30.31 | 378 | 235 |
| Attention only at last res. block | -7.56% | rec, pred, part, BS, QP | 426 | 646 | 6.22 | 24.89 | 378 | 132 |
| No attention | -7.58% | rec, pred, part, BS, QP | 426 | 646 | 6.22 | 24.89 | 378 | 132 |
| EE1-1.2.1: without part, no attention | -7.56% | rec, pred, BS, QP | 415 | 635 | 6.21 | 24.84 | 378 | 122 |
| EE1-1.2.1: without part and BS, no attention | -7.48% | rec, pred, QP | 405 | 625 | 6.20 | 24.80 | 378 | 114 |

# Results

## EE1-1.2.1: Removing partitioning input and removing attention

Results for AI:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -6.67% | -16.01% | -17.60% | 153% | 21097% |
| Class A2 | -6.92% | -18.88% | -16.10% | 123% | 16877% |
| Class B | -6.92% | -18.92% | -20.07% | 125% | 15829% |
| Class C | -7.63% | -18.36% | -20.94% | 122% | 10701% |
| Class E | -10.05% | -19.98% | -19.44% | 129% | 16947% |
| Overall | -7.56% | -18.48% | -19.09% | 129% | 15561% |
| Class D | -7.64% | -18.66% | -20.88% | 116% | 8655% |
| Class F | -5.13% | -11.87% | -11.56% | 105% | 11992% |

The encoding and decoding times are not reliable.

## EE1-1.2.2: Removing partitioning input, removing boundary strength input and removing attention

Results for AI:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -6.52% | -16.57% | -17.74% | 154% | 20226% |
| Class A2 | -6.85% | -19.07% | -16.46% | 125% | 16232% |
| Class B | -6.86% | -19.00% | -20.03% | 122% | 14927% |
| Class C | -7.55% | -18.42% | -21.01% | 112% | 10047% |
| Class E | -9.97% | -19.98% | -19.43% | 121% | 16382% |
| Overall | -7.48% | -18.64% | -19.17% | 125% | 14810% |
| Class D | -7.60% | -18.43% | -20.67% | 121% | 8540% |
| Class F | -5.19% | -11.88% | -11.57% | 108% | 12504% |

The encoding and decoding times are not reliable.

# Conclusion

The results are in comparison to the NNVC CTC [3]. The complexity measured in kMAC/sample and the number of parameters were calculated using the thop library [4].

The results for EE1-1.2.1 indicate that very little (0.01%) BD-rate gain is sacrificed when removing the partitioning input and the attention branch from the luma intra network. The reduction in kMACs for the intra luma component from 429 to 415 is -3.3%, and the total worst-case complexity including chroma goes from 649 to 635, a change of -2.2%. This clearly seems like a good tradeoff.

The results for EE1-1.2.2 indicate that a small loss of 0.09% is obtained when also removing the boundary strength input. The reduction in kMACs for the intra luma component from 429 to 415 is -5.6%, and the total worst-case complexity including chroma goes from 649 to 625, a change of -3.7%. The loss is larger, but the complexity reduction is also larger.

Both models also result in fewer parameters and quicker training.

The contribution proposes to adopt either the model from EE1-1.2.1 or the one from EE1-1.2.2 as the new intra luma model.

# Training and inference information

## Training

| Table 2. Network Information for NN-based video coding tool testing in training stage for the intra luma model. The intra chroma, inter luma and inter chroma models are the same as in JVET-Y0143. |  |  |
| --- | --- | --- |
| Network Information in Training Stage |  |  |
| Mandatory | GPU Type | GPU: NVIDIA A100-SXM-80GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | see Table 1 |
|  | Batch size: | 64 |
|  | Loss function: | L1 and L2 |
|  | Training time: | see Table 1 |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, qp {20, 25, 30, 35, 40} |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128x128 |
|  | Learning rate: | 1e-4 and 1e-5 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

## Inference

| Table 3. Network Information for NN-based video coding tool testing in inference stage for the intra luma model. The intra chroma, inter luma and inter chroma models are the same as in JVET-Y0143. |  |  |
| --- | --- | --- |
| Network Information in Inference Stage |  |  |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | libtorch |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | see Table 1 |
|  | Parameter Precision (Bits) | fp32 |
|  | Memory Parameter (MB) | see Table 1 |
|  | Multiply Accumulate (MAC) | see Table 1 |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128, 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

# References

[1] J. Ström, D. Liu, M. Damghanian, K. Andersson, Y. Li, P. Wennersten, R. Yu, “EE1-related: Reduced complexity NN loop filter and ablation study”, JVET-Z0106, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 26th Meeting, by teleconference, 20–29 April 2022

[2] Y. Li, K. Zhang, L. Zhang, H. Wang, K. Reuze, A.M. Kotra, M. Karczewicz, “EE1-1.2: Test on Deep In-Loop Filter with Adaptive Parameter Selection and Residual Scaling”, JVET-Y0143, 25th Meeting, by teleconference, 12–21 January 2022

[3] Common Test Conditions and evaluation procedures for neural network-based video coding technology, E. Alshina, R.-L. Liao, S. Liu, A. Segall, JVET-X2016, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 24th Meeting, by teleconference, 6–15 October 2022

[4] https://pypi.org/project/thop/

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Data from this row is taken from the JVET-Y0143 contribution. The number of epochs trained and the training time are likely not comparable to those of the other rows.