| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0083-v2 |
| --- | --- |

| Title: | AHG9: NNR post-filter SEI message extension for flexible decoding capabilities |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Franck Galpin Thierry Dumas Philippe Bordes Edouard François | Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to amend the NNR post-filter SEI message proposed in JVET_Z0244. The NNR post-filter SEI message may carry an ISO/IEC 15938-17 bitstream that specifies a neural-network-based post-filter. It also carries information necessary for a decoder to infer the neural network. It is reported that the current design limits flexibility in the inference implementation, that can be for instance required because of the targeted platform capabilities. The NNR post-filter SEI message extension proposed in this contribution allows signaling information used at the decoder side to choose between several Neural Network inference strategies.

# Introduction

The contribution JVET-Z0244 proposes an NNR post-filter SEI message, adopted into a VSEI amendment working draft. The NNR post-filter may carry an ISO/IEC 15938-17 bitstream that specifies a neural-network-based post-filter.

It contains information needed to operate the filter (inputs layout, padding, patch size etc.). It also contains some information which might be needed by a decoder or post-decoding device to decide the best strategy to operate the filter, depending for example on the complexity, the constant block size or not etc.

For example, depending on the model properties a decoder with a large working memory but a lower clock speed might decide to process the frame at once, while a decoder with less working memory but higher clock speed or several independent computation units might decide to process the frame by blocks. This block-based approach is similar to the concept of VPDU already expressed in a video decoder: a decoder can rely on a minimum unit size where dependencies are restricted to be inside the unit but is free to process larger units. Parallel smaller computation unit also allows lower latency since processing can start once a block of the frame is ready.

However, some information is still missing in the current SEI message for a decoder to decide a particular strategy for inference.

# Example of flexible inference

In the following we denote by RF the receptive field parameter, that specifies the margin size of the region in the input which produces an output value.

Figure 1 Receptive field

In Figure 1, we show the receptive field notation used in the following: it is the margin size around the blue value in the output such as this value only depends on the values in the yellow part of the input.

## Example of model characteristics in EE1

An analysis has been performed on already proposed models in JVET-Z-EE1, the following parameters were found:

| Model | Patch size | OverlapSize | Receptive field | Subblock | Superblock |
| --- | --- | --- | --- | --- | --- |
| EE1.4/1.5 | 128x128 | 8 | 62 | possible | Not possible |
| EE1.6 | Non constant | 0 | - | - | Not possible |
| EE1.7 | 128x128 | 4 (8 in frame) | 13 (26 in frame) | possible | Not possible |

In the table, not possible means that it is not possible to do the process on a smaller/larger block size and obtain the same results as the one on the original block size.

All models have a RF larger than the overlapSize, making superblock processing impossible. The overlapSize is defined in JVET-Z0244 and defines the border around the patch.

In case of EE1.2 and EE1.7, subblocks processing is possible.

In EE1.6, the patch inputs are adapted to the block location and a pixel shuffling layer is added, making it difficult to generalize a possible subblocks processing.

## Example of subblock processing for EE1.7

Figure 2 Example of subblock processing for EE1.7

Figure 2 shows an example where the decoder decides to process the inputs as 4 subblocks. The size of each subblock needs to be adapted and depends on the model characteristics.

In this example, the model takes as input a 128x128 block which is rearranged into 4 64x64 blocks. The overlapSize is 4 in inputs. Alternatively, for subblock processing, block of size 32x32 are inputs in the network with the correct margin of 4 (overlapSize) or 13 (receptive field size).

# Example of models characteristics

## Frame based - Translation equivariant

A typical example is a frame-based filter where no border is added to the input since an implicit padding process takes place when samples are taken outside the current tensor. In such mode, the working memory for inference can be very large (typically hundreds of times the size of the input frame). In this case the receptive field value is needed to allow a decoder to operate with block-based inputs. The overlap field, as defined in JVET-Z0244, can be used in this case, but it changes the semantic of the syntax element.

In Figure 1, we assume a model which has been designed for a frame-based processing (top figure). If the model is fully translation equivariant, knowing the RF size, a decoder can prepare an input patch as shown in the bottom figure: for a given block, a margin of the same size as the RF is added around the block to process, and the result is cropped accordingly to the original block size. The process can be repeated or done in parallel for all blocks of the frame to get the same results as a frame-based processing.

Figure 3 Translational equivariant NN case without border. Left gray area: input patch, blue border: receptive field, right gray area: output block

## Block based – Translation equivariant

The simplest case occurs when the NN is fully convolutional and translational equivariant: it means that the NN can be applied with any input size and from any location and will give the same result. In practice, it means that the margin size is at least equal to the receptive field of the NN and that the output values do not depend on the input size (for example using only convolution layers without stride).

Figure 4 Translational equivariant NN case. Left gray area: input patch, blue line: receptive field, right gray area: output block.

In Figure 2, we show the case where the model has been designed for a CTU-based processing (middle figures), but the model is translation equivariant. It allows, knowing the RF size, to input patches of any size with the correct margin equal to the RF:

- In top figure, the whole frame is processed at once. No margin is added to the patch since the padding is used for samples outside the frame.

- In bottom figure, the top-left subblock is processed independently: a margin of overlapSize is added on top and left and a margin of RF is added to right and bottom border.

Note: it makes little sense to process a subblock if the RF is larger than the subblock size, so it is assumed that the RF is always smaller than the input patch size.

In the current SEI message, an information on the patch size property (constant or not) is provided (nnpfc_constant_patch_size). It is assumed that when nnpfc_constant_patch_size is equal to 0, the model is translation equivariant. However, the receptive field is not provided for the decoder to choose the correct input margin. The information nnpfc_overlap provides an information on the margin size, but it is currently related to the constant patch size case. Another constraint is that the input is stated to be a multiple of nnpfc_patch_width/height_minus1+1 in the case of non-constant patch size, so the only way to state a translation equivariant NN would be to set these values to 0.

An information on the receptive field is needed for such case. Sample program demonstrating the need for such parameters are provided with this contribution is provided in 1-framebased.py.

## Block based – No full translation equivariance

### Super block processing

Another case arises, when the filter is designed to work on patches, for example CTUs. A decoder might want to process larger area to minimize computation overhead or to minimize the number of processing cycles. In this case, the variable nnpfc_constant_patch_size is set to 1 because the model is not necessarily fully translation equivariant, but it allows processing of larger blocks. An additional signaling may be used to indicate that larger block processing size can be chosen.

Sample program 2-superblock.py demonstrating the need for such parameters are provided with this contribution.

### Sub-block processing

The general case is the one used by already proposed filters in AhG11: the receptive field is smaller than the input border size and the model is not full translation equivariant. Non full translation equivariance happens generally when the network contains subsampling/pooling/shuffling layers.

Figure 5 Non translational equivariant NN case. Left gray area: input patch, circle: receptive field, right gray area: output block. Bottom: details on a sub-blocks-based processing

In such cases, additional parameters are needed to signal how to perform the processing of the sub-blocks as depicted in Figure 3:

- The top-left subblock is defined as before using the overlapSize and RF margins on top/left and bottom/right borders respectively is used, but additional parameters (dL/dT/dR/dB) is needed to indicate potential margin changes.

- Other subblock follows the same logic with specific margins offsets.

#### Example

Sample program 3-subblock.py demonstrating the need for such parameters are provided with this contribution.

The model is a minimal NN with only 3 convolution layers as shown in Figure 6. A subsampling/upsampling is performed through a maxpool and pixel shuffling. In this particular model, for some subblock an offset of 1 is needed on top of the overlap or receptive field margins.

Figure 6 Sample program model and subblock configuration

# Syntax changes

It is proposed to add some additional information (changes in yellow) for a decoder to decide an inference strategy based on the model characteristics.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| nnpfc_purpose | ue(v) |
| if( nnpfc_purpose = = 2 ¦ ¦ nnpfc_purpose = = 4 ) { |  |
| nnpfc_out_sub_width_c_flag | u(1) |
| nnpfc_out_sub_height_c_flag | u(1) |
| } |  |
| if( nnpfc_purpose = = 3 ¦ ¦ nnpfc_purpose = = 4 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| /* input and output formatting */ |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_sample_idc | ue(v) |
| if( nnpfc_inp_sample_idc = = 4 ) |  |
| nnpfc_inp_tensor_bitdepth_minus8 | ue(v) |
| nnpfc_inp_order_idc | ue(v) |
| nnpfc_out_sample_idc | ue(v) |
| if( nnpfc_out_sample_idc = = 4 ) |  |
| nnpfc_out_tensor_bitdepth_minus8 | ue(v) |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_padding_type | ue(v) |
| nnpfc_receptive_field_plus1 | ue(v) |
| if (nnpfc_receptive_field_plus1>0) { |  |
| nnpfc_superblock_possible | u(1) |
| nnpfc_subblock_possible | u(1) |
| if (nnpfc_subblock_possible) |  |
| nnpfc_nb subdivision_minus1 | u(1) |
| for(level=0;level<nnpfc_nb subdivision_minus1+1;level++) { |  |
| for(i=0;i<pow(4, nnpfc_nb subdivision_minus1+1);i++) { |  |
| nnpfc_subblock_dL[level][i] | e(v) |
| nnpfc_subblock_dT[level][i] | e(v) |
| nnpfc_subblock_dR[level][i] | e(v) |
| nnpfc_subblock_dB[level][i] | e(v) |
| } |  |
| } |  |
| } |  |
| nnpfc_complexity_idc | ue(v) |
| if( nnpfc_complexity_idc > 0 ) { |  |
| nnpfc_complexity_element( nnpfc_complexity_idc ) |  |
| } |  |
| } |  |
| /* filter specified or udpated by 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

The semantics of the main added syntax elements is provided below.

nnpfc_receptive_field_plus1 specifies the half size of the receptive field size of the network. It represents a margin size around the input so that changing the values outside the area defined by the block plus the margin in the input gives the same useful output. Receptive field value is in the range 0 to max(nnpfc_patch_width_minus1+1, nnpfc_patch_height_minus1+1)/2. A value of -1 of the receptive field indicates that the model does not have a receptive field. The parameter receptiveField is set equal to nnpfc_receptive_field_plus1-1.

nnpfc_superblock_possible equal to 1 indicates that a decoder is allowed to perform a processing on larger block size than the size specified by nnpfc_patch_height_minus1 and nnpfc_patch_width_minus1. The post-processing filter accepts any patch size that is a positive integer multiple of the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input, similarly to nnpfc_constant_patch_size_flag equal to 0. When nnpfc_superblock_possible is equal to 0, a decoder is not allowed to perform a processing on larger block size. When not present, nnpfc_superblock_possible is inferred to be equal to 0.

nnpfc_possible_subblock equal  to 1 indicates that a decoder is allowed to perform a processing on smaller block size than the size specified by nnpfc_patch_height_minus1 and nnpfc_patch_width_minus1. For example, an input tensor would have the width and height (without the margins). When nnpfc_possible_subblock is equal to 0, a decoder is not allowed to perform a processing on smaller block size. When not present, nnpfc_possible_subblock is inferred to be equal to 0.

nnpfc_nb subdivision_minus1 specifies the number of possible subdivisions nnpfc_nb subdivision. When nnpfc_possible_subblock is set to 1, nnpfc_nb subdivision is set equal to nnpfc_nb subdivision_minus1+1. The value of nnpfc_nb_subdivision_minus1+1 is in the range 1 to floor(log2(min(nnpfc_patch_width_minus1, nnpfc_patch_height_minus1)+1)). When not present, nnpfc_nb subdivision_minus1 is inferred to 0.

In order to process the original block as subblocks, the input patch is changed as follow for each subblock inside the block, given a split chosen at level, with level in the range 1 to nnpfc_nb subdivision:

nbSubblock=pow(4, level)

inpPatchWidth = (nnpfc_patch_width_minus1 + 1)>> level
inpPatchHeight = (nnpfc_patch_height_minus1 + 1)>>level

For each subblock i, with i from 0 to nbSubblock-1 in raster scan order, the margins added around the subblock are given by:

dT[i]= defaultMarginT + nnpfc_subblock_dT[level][i]

dL[i]= defaultMarginL +nnpfc_subblock_dL[level][i]

dB[i]= defaultMarginB+ nnpfc_subblock_dB[level][i]

dR[i]= defaultMarginR+ nnpfc_subblock_dR[level][i]

with dT, dL, DB and dR the top, left,bottom and right margins respectively and:

defaultMarginT is equal to overlapSize for subblocks at the top of the original block, and receptiveField otherwise

defaultMarginL is equal to overlapSize for subblocks at the left of the original block, and receptiveField otherwise

defaultMarginR is equal to overlapSize for subblocks at the right of the original block, and receptiveField otherwise

defaultMarginB is equal to overlapSize for subblocks at the bottom of the original block, and receptiveField otherwise

Output samples results are taken by cropping the output using the margins defined on the input subblocks.

# SEI usage examples

The table below sum-up the different use case of the proposed syntax.

| Case | Decoder choices | SEI configuration | Example | Sample program |
| --- | --- | --- | --- | --- |
| Full translation equivariant | Can choose any input size, from frame to pixel-based processing | nnpfc_constant_patch_size_flag =0 nnpfc_patch_width_minus1=0 nnpfc_patch_height_minus1=0 nnpfc_overlap=0 nnpfc_receptive_field_plus1=RF+1 nnpfc_superblock_possible=1 nnpfc_subblock_possible=0 | e.g. JVET-Z0101, | 1-framebased.py |
| Super-block possible | Can choose block size greater than signaled size (but not necessarily smaller size) | nnpfc_constant_patch_size_flag =1 nnpfc_patch_width_minus1=w-1 nnpfc_patch_height_minus1=h-1 nnpfc_overlap=RF+1 nnpfc_receptive_field_plus1=RF+1 nnpfc_superblock_possible=1 nnpfc_subblock_possible=0 | e.g. JVET-V0092 | 2-superblock.py |
| Sub-block possible | Can choose smaller block size | nnpfc_constant_patch_size_flag =1 nnpfc_patch_width_minus1=w-1 nnpfc_patch_height_minus1=h-1 nnpfc_overlap=margin nnpfc_receptive_field_plus1=RF+1 nnpfc_superblock_possible=0 nnpfc_subblock_possible=1 nnpfc_nb subdivision_minus1=level-1 nnpfc_subblock_dT/B/R/L[level][i] | Models in EE1.5/ EE1.7 | 3-subblock.py |
| Constant block size | Decoder cannot choose the block size | nnpfc_constant_patch_size_flag =1 nnpfc_patch_width_minus1=patchw-1 nnpfc_patch_height_minus1=patchh-1 nnpfc_receiptive_field_plus1=0 nnpfc_overlap=0 nnpfc_padding_type=0 | EE 1.6 |  |
| Frame based no equivariance | Decoder should process the whole frame | nnpfc_constant_patch_size_flag =1 nnpfc_patch_width_minus1=framew-1 nnpfc_patch_height_minus1=frameh-1 nnpfc_receiptive_field_plus1=0 nnpfc_overlap=0 | e.g. JVET-Z0144 |  |

# Conclusion

The added information intends to go beyond the limitations of the current NNR post-filter SEI message, that does not allow to adapt the post filter implementation depending to the target device capabilities. The proposed changes allow supporting a mechanism to operate a filter, with the support of low-complexity decoder. The changes are important for ensuring genericity of the NNR post-filter SEI message.

# Patent rights declaration(s)

InterDigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

In literature, the receptive field often denotes the whole yellow area.