| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0085-v1 |
| --- | --- |

| Title: | EE1-1.1: the Performance of Single-Model Filter Trained on the VTM and ECM Reconstruction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Chang Liqiang Wang Xiaozhong Xu Shan Liu | Email: | renjiechang@tencent.com liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution presents the EE1-1.1 test results based on JVET-Z0093. In this test, a single-model filter trained on VTM or ECM reconstruction is utilized for in-loop filtering, and three tests using different training data and training methods were performed. Based on ECM4.0, the related results are shown in order of RA and AI configurations as follows.

Test 1.1.1 (trained on VTM reconstruction):

{-5.51%, -13.18%, -13.67%}, {-3.90%, -8.51%, -10.40%}

Test 1.1.2 (retrained on ECM reconstruction):

{-5.60%, -12.97%, -12.95%}, {-3.58%, -7.40%, -8.88%}

Test 1.1.3 (fine-tuned on ECM reconstruction):

{-5.91%, -13.69%, -13.72%}, {-4.02%, -8.90%, -10.30%}

Introduction

In JVET-Z0093 [], the dual-model filter [] and single-model filter [] both were integrated into ECM-4.0 to further study the performance of neural network-based in-loop filters on ECM platform. Base on ECM4.0, this EE test is to study the performance of single-model filters trained on different reconstructions, including testing filter trained on VTM reconstruction, testing filter retrained on ECM reconstruction and testing filter fine-tuned on ECM reconstruction.

Proposed method

Network architecture

The network structure of the proposed single-model filter is shown in Fig. 1. Along with the reconstructed image (rec_yuv), the prediction image (pred_yuv) [] is also fed into the network. Besides, the base QP, slice QP and slice type are also fed into the proposed network to enhance the generalization ability.

Fig. 1: The network architecture of single-model filter

Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Tesla V100 32GB |
|  | Framework: | Torch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~1000 |
|  | Batch size: | 64 |
|  | Loss function: | L1 |
|  | Training time: | ~200h |
|  | Training data information: | DIV2K, TVD, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations | 1200 |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Other information: |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
|  | Test items | Test 1.1.1 / Test 1.1.2 / Test 1.1.3 |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.9M |
|  | Total Number of Parameters (All Models) | 1.9M |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 7.60MB / model, 1 model in all |
|  | Multiplay Accumulate (MAC)/pixel | 485K |
| Optional |  |  |
|  | Total Conv. Layers | 101 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Implementation details

- For I slices and B slices, Deblock and SAO are both enabled. The optimal filtered result is decided between the two outputs from the proposed filter and the SAO.

- A scaling operation is carried out to refine the result of NN filter. The scaling factors are signaled in the slice header for each component. The specific process is the same as the scaling process used in []. Additionally, there are 3 fixed weights to blend the results of the proposed filter and the conventional filter. These fixed weights are 1, 0.75 and 0.5, which are similar as in [].

- The proposed filter can be turned on/off at the CTU level and slice level.

Results

In the experiments, ECM-4.0 [][][] is used as the anchor. All the 3 tests are implemented with SADL library [] and the corresponding results are provided as follows.

  - Results of EE1-1.1.1

Table 3. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over ECM-4.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -7.93% | -6.40% | -11.40% | -13.64% | 319% | 224965% |
| Class A2 | -7.46% | -6.13% | -12.48% | -10.44% | 305% | 126930% |
| Class B | -7.47% | -5.29% | -13.66% | -14.35% | 342% | 286432% |
| Class C | -7.20% | -4.65% | -14.45% | -15.25% | 280% | 252143% |
| Class E |  |  |  |  |  |  |
| Overall | -7.49% | -5.51% | -13.18% | -13.67% | 312% | 224172% |
| Class D | -7.62% | -5.37% | -13.15% | -15.54% | 294% | 232465% |
| Class F | -3.61% | -2.07% | -8.50% | -8.01% | 588% | 165258% |

Table 4. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over ECM-4.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -5.12% | -3.95% | -7.10% | -10.18% | 566% | 346645% |
| Class A2 | -4.53% | -3.50% | -8.40% | -6.83% | 375% | 292111% |
| Class B | -4.75% | -3.40% | -7.44% | -10.21% | 342% | 283637% |
| Class C | -5.54% | -3.53% | -10.50% | -12.60% | 245% | 219380% |
| Class E | -6.78% | -5.59% | -9.17% | -11.56% | 413% | 310260% |
| Overall | -5.29% | -3.90% | -8.51% | -10.40% | 362% | 282563% |
| Class D | -5.96% | -3.91% | -10.55% | -13.66% | 232% | 221068% |
| Class F | -3.51% | -2.34% | -6.96% | -7.08% | 266% | 219489% |

  - Results of EE1-1.1.2

Table 5. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over ECM-4.0 |  |  |  |  |  |  |  |  |  |  |
|  | YUV-PSNR |  | Y-PSNR |  | U-PSNR |  | V-PSNR |  | EncT | DecT CPU |  |
| Class A1 | -7.44% |  | -6.14% |  | -10.50% |  | -12.17% |  | 320% | 260380% |  |
| Class A2 | -7.12% |  | -5.95% |  | -11.65% |  | -9.61% |  | 305% | 190477% |  |
| Class B | -7.47% |  | -5.28% |  | -14.03% |  | -14.02% |  | 340% | 294078% |  |
| Class C | -7.65% |  | -5.34% |  | -14.51% |  | -14.71% |  | 278% | 263724% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -7.44% |  | -5.60% |  | -12.97% |  | -12.95% |  | 312% | 255594% |  |
| Class D | -8.44% |  | -6.33% |  | -14.18% |  | -15.34% |  | 293% | 248039% |  |
| Class F | -3.73% |  | -2.14% |  | -8.73% |  | -8.23% |  | 587% | 168544% |  |

Table 6. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over ECM-4.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -4.53% | -3.58% | -6.03% | -8.71% | 565% | 344857% |
| Class A2 | -3.68% | -3.15% | -6.32% | -4.23% | 375% | 291124% |
| Class B | -4.28% | -3.11% | -6.64% | -8.93% | 340% | 282106% |
| Class C | -5.03% | -3.27% | -9.63% | -10.97% | 244% | 218708% |
| Class E | -6.28% | -5.21% | -8.15% | -10.82% | 411% | 310108% |
| Overall | -4.72% | -3.58% | -7.40% | -8.88% | 361% | 281521% |
| Class D | -5.34% | -3.69% | -8.94% | -11.62% | 232% | 220794% |
| Class F | -3.34% | -2.14% | -6.87% | -6.99% | 264% | 224110% |

  - Results of EE1-1.1.3

Table 6. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over ECM-4.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.07% | -6.57% | -11.84% | -13.34% | 319% | 225534% |
| Class A2 | -7.89% | -6.46% | -13.67% | -10.71% | 304% | 127087% |
| Class B | -7.90% | -5.59% | -14.63% | -15.04% | 342% | 287369% |
| Class C | -7.63% | -5.41% | -13.93% | -14.63% | 280% | 261683% |
| Class E |  |  |  |  |  |  |
| Overall | -7.86% | -5.91% | -13.69% | -13.72% | 312% | 226821% |
| Class D | -8.27% | -6.37% | -12.87% | -15.11% | 293% | 245499% |
| Class F | -3.75% | -2.33% | -8.20% | -7.79% | 588% | 162321% |

Table 7. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over ECM-4.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -5.37% | -4.11% | -7.80% | -10.52% | 566% | 346676% |
| Class A2 | -4.10% | -3.62% | -7.59% | -3.48% | 375% | 292191% |
| Class B | -5.01% | -3.47% | -8.16% | -11.10% | 342% | 283766% |
| Class C | -5.70% | -3.65% | -10.86% | -12.88% | 244% | 219138% |
| Class E | -7.07% | -5.75% | -9.90% | -12.12% | 412% | 310072% |
| Overall | -5.41% | -4.02% | -8.90% | -10.30% | 362% | 282518% |
| Class D | -5.95% | -3.94% | -10.03% | -13.93% | 233% | 221131% |
| Class F | -3.64% | -2.43% | -7.08% | -7.41% | 265% | 222228% |

Conclusions

Based on the single-model filter with constrained memory size and lower complexity, the performance of filters trained on VTM or ECM reconstruction is evaluated on ECM4.0. Without any further refinements on network structure, the proposed filters explored on VTM can still achieve a good trade-off on ECM, and the gap between VTM-trained model and ECM-trained model is relatively small. Besides, finetuning can also further improve the model performance compared to retraining.

Reference

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[] R. Chang, L. Wang, X. Xu, S. Liu, F. Galpin. EE1-related: The performance of EE1 Test 1.1 and Test 1.2 on ECM-4.0. JVET-Z0093, 26th Meeting, by teleconference, 20–29 April 2022.

[] L. Wang, X. Xu, S. Liu, F. Galpin. EE1-1.1: neural network based in-loop filter with 2 models. JVET-Z0094, 26th Meeting, by teleconference, 20–29 April 2022.

[] L. Wang, X. Xu, S. Liu, F. Galpin. EE1-1.2: neural network based in-loop filter with a single model. JVET-Z0091, 26th Meeting, by teleconference, 20–29 April 2022.

[] H. Zhu, X. Xu, and S. Liu. Deep learning based in-loop filter with QP and residual distribution information. Doc. AVS-M5654, Aug. 2020.

[] H. Wang, J. Chen, K. Reuze, A. M. Kotra, M. Karczewicz. EE1-1.4: Test on Neural Network-based In-Loop Filter with Large Activation Layer. JVET-W0130, 23rd Meeting, by teleconference, 7–16 July 2021.

[] K. Andersson, J. Ström, D. Liu, R. Sjöberg. EE1-related: Combination of VVC deblocking and NN loop filtering. JVET-Y0098, 25th Meeting, by teleconference, 12–21 January 2022.

[] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-4.0.

[] M. Karczewicz and Y. Ye. Common test conditions and evaluation procedures for enhanced compression tool testing. JVET-Y2017, 25th Meeting, by teleconference, 12–21 January 2022.

[] S. Liu, A. Segall, E. Alshina, R.-L. Liao. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-X2016, 24th Meeting, by teleconference, 6–15 October 2021.

[] F. Galpin, F. Mom, T. Dumas, P. Bordes, P. Nikitin, E. François, AHG11: Small Ad-hoc Deep-Learning Library update, JVET-Y0110, by teleconference, January 2022.