| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0087-v2 |
| --- | --- |

| Title: | EE1-1.4: neural network based in-loop filter with 2 models |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liqiang Wang Sheng Lin Xiaozhong Xu Shan Liu Franck Galpin | Email: | liqiangwang@tencent.com barneylin@tencent.com xiaozhongxu@tencent.com shanl@tencent.com franck.galpin@interdigital.com |
| Source: | Tencent, Interdigital |  |  |

_____________________________

- Abstract

This contribution reports the EE results of JVET-Z0094 []. Compared with the previous method, the models are fine-tuned and the SADL library is updated. Based on the EE1 anchor, the related results are shown in order of RA and AI configurations as follows.

Test 1.4.1 (flt32, Libtorch implementation):

{-9.15%, -19.13%, -19.64%}, {-7.41%, -16.80%, -17.33%}

Test 1.4.2 (int16, SADL implementation):

{-9.17%, -19.92%, -19.80%}, {-7.46%, -16.63%, -17.57%}

Introduction

Based on the network architecture in JVET-W0113 [], more side information is utilized. The inference complexity and memory size are also taken into consideration. Compared with previous method in JVET-Z0094 [1], the models are fine-tuned and the SADL library is updated.

Proposed method

The reconstructed image after LMCS is fed into the network. Then, the output of NN filter is processed by ALF and CCALF. In detail, as shown in Fig. 1, U and V channels are up-sampled and down-sampled in the pre-processing and post-processing, respectively.

Fig. 1 the pre-processing and post-processing units

Network architecture

The network structure of the proposed NN filter is shown in Fig. 2. Along with the reconstructed image (rec_yuv), the prediction image (pred_yuv) [] and the partition image (par_yuv) [] are also fed into the network.

Based on the un-filtered reconstructed image, the partition image is generated by setting each sample value into the average value of the whole partition block. Note that the partition image is only fed for the I slice, and the base QP is fed only for the network of the B slice.

Compared with the implementation in JVET-W0113[2], the ResBlock is also modified as shown in Fig. 3. With the same spirit in JVET-W0113, the number of channels firstly goes up before the activation layer, and then goes down after the activation layer. In addition, and the convolution in the skip-layer connection is removed.

Fig. 2: Network architecture

Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Tesla A100 40GB |
|  | Framework: | Pytorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~1000 |
|  | Batch size: | 64 |
|  | Loss function: | L1 |
|  | Training time: | ~150h |
|  | Training data information: | DIV2K, TVD, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations | 1200 |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Training data update strategy: | Iterative training method |
|  | Other information: |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test items | Test 1.4.1 |  | Test 1.4.2 |
| Mandatory | HW environment: |  |  |  |
|  | GPU Type |  | CPU only |  |
|  | Framework: |  | SADL |  |
|  | Number of GPUs per Task |  | 0 |  |
|  | Number of Parameters (Each Model) |  | 1.54M |  |
|  | Total Number of Parameters (All Models) |  | 3.08M |  |
|  | Parameter Precision (Bits) |  | 32 | 16 |
|  | Memory Parameter (MB) |  | 6.16MB/model, 2 models in all | 3.08MB/model, 2 models in all |
|  | Multiplay Accumulate (MAC)/pixel, worst-case |  | 401K (w/o block extension) 507K (w/ block extension) |  |
|  | Calculation Method |  | block basis |  |
| Optional |  |  |  |  |
|  | Total Conv. Layers |  | 78 |  |
|  | Total FC Layers |  | 0 |  |
|  | Total Memory (MB) |  |  |  |
|  | Batch size: |  | 1 |  |
|  | Patch size |  | 144x144 |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |  |
|  | Peak Memory Usage (Total) |  |  |  |
|  | Peak Memory Usage (per Model) |  |  |  |
|  | Border handling |  |  |  |
|  | Other information: |  |  |  |

Implementation details

- For I slices and B slices, Deblock and SAO are both enabled. The optimal filtered result is decided between the two outputs from the proposed filter and the SAO.

- A scaling operation is carried out to refine the result of NN filter. The scaling factors are signaled in the slice header for each component. The specific process is the same as the scaling process used in []. Additionally, there are 3 fixed weights to blend the results of the proposed filter and the conventional filter. These fixed weights are 1, 0.75 and 0.5, which are similar as those in [].

- The proposed filter can be turned on/off at the CTU level and slice level.

Results

All the test results are provided as follows, which are based on the EE1 anchor [][]. The detailed results, such as LD results, can be found in the attached Excel files.

In specific, the following tests are included into the EE1-1.4 tests.

Test 1.4.1: Optimize the float SADL implementation based on JVET-Z0094. (Table 3 and Table 4)

Test 1.4.2: Optimize the integer SADL implementation based on JVET-Z0094. (Table 7 and Table 8)

For Test 1.4.2, the parameters of the int16 models used in this test can be quantized by the SADL quantization way into int16 format without any loss. Noted the forward inference is still in float precision. Compared with flt32 results, nearly the same performance can be achieved by the int16 model.

In addition to EE1-1.4.1 and EE1-1.4.2, another implementation is designed to reduce the running time or refine the trade-off. The skip connection from the input to the output and PixelShuffle module are revised by performing by C code instead of SADL. The modification of skip connection may be helpful to finish the model quantization more easily on SADL, while the modification of PixelShuffle module may be helpful to reduce the running time.

The additional test results of EE1-1.4.1 are shown in Table 5 and Table 6. It shows an obvious decrease on the running time, especially for the decoder.

The additional test results of EE1-1.4.2 are shown in Table 9 and Table 10. This purpose of this test is to verify a real-int16 implementation. Not only are model parameters quantized to int16 precision, but the forward inference is also conducted in integer precision. Compared with the results of the float implementation (Table 4 and Table 5), the performance can be nearly maintained.

Results of EE1-1.4.1 (flt32, SADL implementation)

Table 3. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -11.01% | -9.38% | -13.69% | -18.12% | 166% | 95053% |
| Class A2 | -11.99% | -10.10% | -19.63% | -15.65% | 160% | 89845% |
| Class B | -11.70% | -8.75% | -20.04% | -21.06% | 166% | 98340% |
| Class C | -12.02% | -8.75% | -21.69% | -21.99% | 147% | 88355% |
| Class E |  |  |  |  |  |  |
| Overall | -11.71% | -9.15% | -19.13% | -19.64% | 160% | 93224% |
| Class D | -12.81% | -9.94% | -20.93% | -21.90% | 150% | 93128% |
| Class F | -5.87% | -3.87% | -12.23% | -11.45% | 208% | 41267% |

Table 4. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.94% | -6.55% | -14.67% | -17.54% | 234% | 68132% |
| Class A2 | -8.89% | -6.53% | -17.49% | -14.49% | 173% | 55906% |
| Class B | -9.15% | -6.56% | -16.89% | -17.00% | 163% | 54682% |
| Class C | -10.40% | -7.78% | -17.27% | -19.27% | 137% | 41534% |
| Class E | -11.97% | -10.06% | -17.48% | -17.92% | 175% | 64357% |
| Overall | -9.82% | -7.41% | -16.80% | -17.33% | 170% | 55032% |
| Class D | -10.21% | -7.77% | -15.97% | -19.05% | 133% | 45020% |
| Class F | -6.07% | -4.42% | -10.77% | -11.26% | 132% | 38780% |

Results of EE1-1.4.1 (flt32, SADL implementation, addition)

Table 5. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |
| Class A2 | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -12.11% | -8.87% | -21.75% | -21.88% | 139% | 74993% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -12.97% | -10.11% | -21.17% | -21.93% | 136% | 68610% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 6. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -10.51% | -7.87% | -17.48% | -19.37% | 131% | 35161% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -10.30% | -7.88% | -16.01% | -19.11% | 125% | 32953% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Results of EE1-1.4.2 (int16, SADL implementation)

Table 7. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -11.23% | -9.27% | -15.64% | -18.53% | 166% | 94678% |
| Class A2 | -11.92% | -10.01% | -19.94% | -15.40% | 160% | 86540% |
| Class B | -11.95% | -8.89% | -20.72% | -21.51% | 166% | 98008% |
| Class C | -12.12% | -8.82% | -22.13% | -21.91% | 146% | 87371% |
| Class E |  |  |  |  |  |  |
| Overall | -11.84% | -9.17% | -19.92% | -19.80% | 159% | 92076% |
| Class D | -12.82% | -9.85% | -21.23% | -22.21% | 150% | 93073% |
| Class F | -6.10% | -3.94% | -13.18% | -11.98% | 209% | 40931% |

Table 8. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.19% | -6.80% | -14.56% | -18.17% | 236% | 69373% |
| Class A2 | -8.83% | -6.60% | -17.02% | -14.00% | 173% | 55906% |
| Class B | -9.29% | -6.64% | -16.89% | -17.57% | 163% | 54812% |
| Class C | -10.40% | -7.69% | -17.35% | -19.74% | 136% | 41531% |
| Class E | -11.86% | -10.05% | -16.90% | -17.63% | 175% | 64445% |
| Overall | -9.87% | -7.46% | -16.63% | -17.57% | 170% | 55246% |
| Class D | -10.17% | -7.61% | -16.14% | -19.53% | 133% | 45054% |
| Class F | -6.59% | -4.54% | -12.95% | -12.47% | 131% | 42095% |

Results of EE1-1.4.2 (real-int16, SADL implementation, addition)

Table 9. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |
| Class A2 | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -12.13% | -8.85% | -22.13% | -21.83% | 126% | 48934% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -12.97% | -10.07% | -21.57% | -21.75% | 123% | 44609% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 10. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -10.43% | -7.86% | -17.17% | -19.10% | 122% | 23637% |
| Class E | -11.89% | -10.02% | -17.13% | -17.86% | 147% | 39452% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -10.27% | -7.88% | -15.91% | -18.98% | 117% | 22065% |
| Class F | 21.23% | 22.19% | 18.61% | 18.09% | #NUM! | #NUM! |

Conclusions

With a constrained memory size and lower complexity, a neural network based in-loop filter is presented in this contribution. Good trade-off between performance and complexity can be achieved by applying the proposed method.

References

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.

[] L. Wang, X. Xu, S. Liu, F. Galpin. EE1-1.1: neural network based in-loop filter with 2 models. JVET- Z0094, 26th Meeting, by teleconference, 20–29 April 2022.

[] L. Wang, W. Jiang, X. Xu, S. Liu. AHG11: neural network based in-loop filter. JVET-W0113, 23rd Meeting, by teleconference, 7–16 July 2021.

[]  H. Zhu, X. Xu, and S. Liu. Deep learning based in-loop filter with QP and residual distribution information. Doc. AVS-M5654, Aug. 2020.

[] W. Lin, X. He, X. Han, D. Liu, J. See, J. Zou, H. Xiong, and F. Wu, “Partition-Aware Adaptive Switching Neural Networks for Post-Processing in HEVC,” IEEE Transactions on Multimedia, vol. 22, no. 11, pp. 2749–2763, Nov. 2020.

[] H. Wang, J. Chen, K. Reuze, A. M. Kotra, M. Karczewicz. EE1-1.4: Test on Neural Network-based In-Loop Filter with Large Activation Layer. JVET-W0130, 23rd Meeting, by teleconference, 7–16 July 2021.

[] K. Andersson, J. Ström, D. Liu, R. Sjöberg. EE1-related: Combination of VVC deblocking and NN loop filtering. JVET-Y0098, 25th Meeting, by teleconference, 12–21 January 2022.

[] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc.

[] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-Z2016, 26th Meeting, by teleconference, 20–29 April 2022.