| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0090-v4 |
| --- | --- |

| Title: | EE1-related: One luma model with IPB and skip for filtering intra and inter luma slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Du Liu, Jacob Ström, Mitra Damghanian, Per Wennersten, Yun Li Torshamnsgatan 23 164 83 Stockholm | Tel: Email: | {du.liu, jacob.strom, mitra.damghanian, per.wennersten, yun.y.li} @ericsson.com |
| Source: | Ericsson |  |  |

# Abstract

This contribution proposes to use one luma model with block type information and block skip information for NN-filtering of intra luma and inter luma slices. It is claimed that the combined model has a worst-case complexity in terms of kMAC/pixel that is 1.7% higher than JVET-Z0112, but that this model makes it possible to reduce the number of models from four to three, and to reduce the total number of parameters from 6.24M to 4.70M. The proposed model uses block type information IPB (intra/uni-predicted/bi-predicted) and block skip information (bypassed or not) as two additional inputs on top of the inter luma model from JVET-Z0112. It is reported that the IPB+skip model gives 0.35% (= 9.99% − 9.64%) higher luma gain for RA than the model without IPB or skip in the tests carried out in the contribution. It is further reported that the proposed IPB+skip model has a loss of 0.1% comparing to using separate intra luma and inter luma models. The IPB+skip model tested on top of JVET-AA0111 EE1-1.6.2 with a combination of deblocking filter and SADL gives a luma gain of 10.27% for RA.

- Test 1: Inter luma model from JVET-Z0112 retrained with only RA data and and with separate intra luma and inter luma models:

BDR-Y -10.09% RA, -7.39% AI, and -8.48% LDB. Worst case kMAC/pixel 649.

- Test 2: Inter luma model from JVET-Z0112 retrained with AI and RA data:

BDR-Y -9.64% RA, -7.48% AI, and -8.47% LDB. Worst case kMAC/pixel 638.

- Test 3: IPB+skip model trained with AI and RA data:

BDR-Y -9.99% RA, -7.44% AI, and -8.47% LDB. Worst case kMAC/pixel 660.

- Test 4: IPB+skip model tested on top of JVET-AA0111 (EE1-1.6.2, float32, combination with deblocking and SADL)

BDR-Y -10.27% RA, -7.32% AI, and xxx LDB. Worst case kMAC/pixel 660

# Introduction

In JVET-Z0112 [1], four CNN models are used for in-loop filtering: One model for intra luma pictures, one model for inter luma pictures, one model for intra chroma pictures, and one model for inter chroma pictures. This contribution proposes to use a single luma model with block type information and block skip information for both intra luma and inter luma pictures. The proposed model has a complexity of 660 kMAC/pixel (worst case, including chroma), which is an increase of 1.7% comparing to the complexity of 649 kMAC/pixel reported in JVET-Z0112. The number of models is reduced from four to three, and the total number of parameters is reduced from 6.24M to 4.70M.

The inputs of the inter luma model in JVET-Z0112 include reconstruction samples, prediction samples, the block boundary strengths, and QP. In this contribution, two extra inputs are used to feed more information to the NN model, i.e., the block type and the block skip mode. The block type (IPB) indicates whether a sample belongs to a block that is intra predicted, uni-predicted, or bi-predicted, thus it has three possible values. The skip mode indicates whether a sample belongs to a block is by-passed, thus it has two values: True or False. It is speculated that it is these two extra inputs that help the model to better fit different block types and to improve the compression performance in the case where the network is used for both intra- and inter- slices.

# Proposed method

## Structure of the proposed luma model

The structure of the proposed model is depicted in Figures 1, 2, and 3. Figure 1 shows the six inputs, which are the reconstructed samples (rec), the prediction samples (pred), the block boundary strength (bs), QP, the block type (IPB), and the skip mode. Figure 2 shows the k-th residual block, where the attention module takes in the aforementioned six inputs. In this contribution, there are eight residual blocks. Figure 3 shows the last part of the network, which is the same as in JVET-Z0112.

Figure 1. Head of network. The inputs are combined to form the input y to the next part of the network.

Figure 2. The k-th residual block (k=0..7). The output y of the head is fed into a first residual block with input z0=y, which also takes the inputs rec, pred, part, bs, qp, IPB, and skip. The output z1 is then fed into another such residual block.

Figure 3. The output of the last residual block is fed into this last part of the network.

## Complexity

The complexity measured in kMAC/sample and the number of parameters were calculated using the thop library [3], version 0.0.31-200524190.

The complexity figures and the number of parameters are listed in Table 1. The original JVET-Z0112 uses four models, and the number of parameters is 6.24M and the total required memory is 24.94×106 bytes. If the JVET-Z0112 inter luma model is retrained and used for both intra luma and inter luma cases, only three models are needed. The total number of parameters becomes 4.67M and the total memory is 18.69×106 bytes. The proposed luma model includes the two additional inputs IPB and skip and three models are used. The total number of parameters is 4.70M and the total memory is 18.79×106 bytes.

Table 1: Summary of complexity and performance for the luma model

| Test/Luma model | Nbr. of luma models | kMAC /sample (Y) | kMAC /pix (YUV) for AI | kMAC/pix (YUV) for RA | kMAC/pix (YUV) worst case (block basis) | Nbr. of param. of the inter luma model (106) | Nbr. of param. in total (106) | Total mem. (106 bytes) | RA BDR-Y |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JVET-Z0112 | 2 | 429 for intra 418 for inter | 649 | 632 | 649 | 1.551 | 6.24 (luma inter + luma intra + chroma inter + chroma intra) | 24.94 | -9.78% |
| Test 1: JVET-Z0112 with inter luma model retrained with only RA data | 2 | 429 for intra 418 for inter | 649 | 632 | 649 | 1.551 | 6.24 (luma inter + luma intra + chroma inter + chroma intra) | 24.94 | -10.09% |
| Test 2: JVET-Z0112 with inter luma model retrained with AI and RA data | 1 | 418 | 638 | 632 | 638 | 1.551 | 4.67 (luma + chroma inter + chroma intra) | 18.69 | -9.64% |
| Test 3: Model extended with IPB+skip | 1 | 440 | 660 | 654 | 660 | 1.576 | 4.70 (luma + chroma inter + chroma intra) | 18.79 | -9.99% |

The intra chroma model in JVET-Z0112 has a complexity of 110 kMAC/sample and 1.566M parameters. The inter chroma model in JVET-Z0112 has a complexity of 107 kMAC/sample and 1.555M parameters. Since the parameter selection in the encoder can choose two different QPs for U and V, the worst case is to run two chroma models, and the worst case therefore becomes 429+110+110=649 in the case of JVET-Z0112, and analogously in the other cases.

Additionally, the proposed IPB+skip model is tested on top of EE1-1.6.2 (JVET-AA0111) [4], which is a combination of deblocking filter and SADL [5]. Table 2 summarizes the complexity and performance.

Table 2: Summary of complexity and performance for the luma model on top of JVET-AA0111 EE1-1.6.2

| Test/Luma model | Nbr. of luma models | kMAC/ sample (Y) | kMAC/pix (YUV) for AI | kMAC/pix (YUV) for RA | kMAC/pix (YUV) worst case (block basis) | Nbr. of param. of the inter luma model (106) | Num. of param. in total (106) | Total mem. (106 bytes) | RA BDR-Y |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JVET-AA0111 EE1-1.6.2 | 2 | 429 for intra 418 for inter | 539 | 525 | 539 | 1.551 | 6.24 (luma inter + luma intra + chroma inter + chroma intra) | 24.94 | -10.19% |
| Test4: IPB+skip on top of EE1-1.6.2 | 1 | 440 | 550 | 547 | 550 | 1.576 | 4.70 (luma + chroma inter + chroma intra) | 18.79 | -10.27% |

The intra chroma model in JVET-Z0112 has a complexity of 110 kMAC/sample and 1.566M parameters. The inter chroma model in JVET-Z0112 has a complexity of 107 kMAC/sample and 1.555M parameters. Since the parameter selection in the encoder can choose two different QPs for U and V, the worst case is to run two chroma models, and the worst case therefore becomes 429+110+110=649 in the case of JVET-Z0112, and analogously in the other cases.

## Training

To train the model to fit to both intra and inter cases, both the DIV2K and the BVI-DVC datasets are used.

| Table 3. Network Information for NN-based video coding tool testing in training stage for the luma model. The intra chroma and inter chroma models are the same as in JVET-Z0112. |  |  |
| --- | --- | --- |
| Network Information in Training Stage |  |  |
| Mandatory | GPU Type | GPU: NVIDIA A100-PCIE-40GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 330 (Test 2 and Test 3) 705 (Test 1) |
|  | Batch size: | 32 (L1) and 16 (L2) for the IPB+skip model (Test 3) 64 (L1) and 32 (L2) for the retrained JVET-Z0112 model (Test 2) 64 (L1) and 16 (L2) for the retrained JVET-Z0112 model with RA data only (Test 1) |
|  | Loss function: | Weighted L1 and L2 |
|  | Training time: | 15 days (Test 3) 12 days (Test 2) 12 days (Test 1) |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, qp {17, 22, 27, 32, 37, 42} |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 256x256 |
|  | Learning rate: | 1e-4 and 1e-5 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Other information: | Since the IPB+skip model is larger, it does not fit in the GPU memory using a batch size of 64, so a batch size of 32 is used for L1 training. However, the same number of patches (and samples) are trained in both cases. |

## Inference

The decoder will extract an input IPB patch the same size as the rec input, e.g., 144x144 samples. An IPB sample will be 0 if the corresponding rec sample belongs to a block that is intra-predicted, 0.5 if the block is uni-predicted and 1 for a bi-predicted block. Likewise, the input skip is a patch the same size as the rec input, and a sample therein will be 0 if it belongs to a block that is not skipped and 1 if it belongs to a block that is skipped.

| Table 4. Network Information for NN-based video coding tool testing in inference stage for the luma model. The intra chroma and inter chroma models are the same as in JVET-Z0112. |  |  |
| --- | --- | --- |
| Network Information in Inference Stage |  |  |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | libtorch |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | See Table 1 |
|  | Parameter Precision (Bits) | fp32 |
|  | Memory Parameter (MB) | See Table 1 |
|  | Multiplay Accumulate (MAC) | See Table 1 |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128, 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

# Experimental result

The proposed model is tested on top of VTM11.0+MCTF. The parameter selection and residue scaling as described in JVET-Z0112 is kept unchanged, where the only change is the NN model and the corresponding inputs. The results are in comparison to the NNVC CTC [2].

The proposed luma model with IPB+skip is trained as described in Sec 2.3. For comparison, the inter luma model (without IPB or skip) from JVET-Z0112 is retrained with the same AI and RA training data and the same number of epochs. The trained model is used for both intra and inter luma slices.

The proposed IPB+skip model gives -9.99% RA and -7.44% AI, whereas the retrained inter luma model from JVET-Z0112 gives -9.64% RA and -7.48% AI. The improvement of the proposed luma model is believed to be given by the extra IPB and skip inputs.

As a reference, the inter luma model from JVET-Z0112 is retrained with only RA data and used only in the inter luma slice, i.e., separate models for intra luma and inter luma. This reference model is trained with the same amount of patches as the other trainings. The retrained reference model gives -10.09% for luma of RA, which indicates that the proposed IPB+skip model has a minor loss of 0.1% comparing to using separate intra luma and inter luma models.

The IPB+skip model is further tested on top of JVET-AA0111, which is a combination of deblocking and SADL. The RA gives a gain of -10.27% for luma.

## The proposed IPB+skip model

The encoding times are not reliable.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -9.15% | -17.18% | -20.07% | 181% | 35123% |
| Class A2 | -10.37% | -20.47% | -17.55% | 176% | 31642% |
| Class B | -9.56% | -23.14% | -22.58% | 176% | 32619% |
| Class C | -10.86% | -22.41% | -23.08% | 149% | 27716% |
| Class E |  |  |  |  |  |
| Overall | -9.99% | -21.22% | -21.21% | 169% | 31506% |
| Class D | -12.93% | -24.05% | -24.84% | 146% | 25167% |
| Class F | -5.49% | -12.58% | -12.52% | 220% | 14212% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6.64% | -16.56% | -17.72% | 158% | 22335% |
| Class A2 | -6.79% | -18.94% | -16.16% | 134% | 18186% |
| Class B | -6.82% | -18.18% | -19.41% | 124% | 17741% |
| Class C | -7.46% | -18.31% | -20.86% | 113% | 11290% |
| Class E | -9.89% | -20.00% | -19.43% | 128% | 18411% |
| Overall | -7.44% | -18.37% | -18.91% | 129% | 16846% |
| Class D | -7.50% | -18.51% | -20.79% | 109% | 10119% |
| Class F | -5.13% | -11.88% | -11.56% | 110% | 14656% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -7.72% | -14.26% | -14.56% | 167% | 33000% |  |
| Class C | -9.51% | -13.67% | -14.25% | 145% | 26616% |  |
| Class E | -8.34% | -16.11% | -14.53% | 258% | 27716% |  |
| Overall | -8.47% | -14.00% | -14.43% | 178% | 29407% |  |
| Class D | -11.28% | -18.87% | -18.56% | 133% | 23009% |  |
| Class F | -5.32% | -8.17% | -6.41% | 210% | 15452% |  |

## The retrained inter luma model from JVET-Z0112

The encoding times are not reliable.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -8.58% | -17.16% | -20.03% | 156% | 32174% |
| Class A2 | -9.94% | -20.35% | -17.54% | 153% | 28285% |
| Class B | -9.31% | -23.07% | -22.42% | 150% | 28785% |
| Class C | -10.63% | -22.36% | -22.97% | 130% | 24586% |
| Class E |  |  |  |  |  |
| Overall | -9.64% | -21.15% | -21.11% | 146% | 28122% |
| Class D | -12.77% | -24.00% | -24.82% | 128% | 23532% |
| Class F | -5.51% | -12.75% | -12.47% | 187% | 13277% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6.68% | -16.19% | -17.63% | 146% | 20778% |
| Class A2 | -6.78% | -18.88% | -16.07% | 118% | 17064% |
| Class B | -6.85% | -17.71% | -19.13% | 110% | 16575% |
| Class C | -7.55% | -18.30% | -20.78% | 105% | 10684% |
| Class E | -9.95% | -19.99% | -19.43% | 119% | 17391% |
| Overall | -7.48% | -18.16% | -18.79% | 117% | 15813% |
| Class D | -7.59% | -18.68% | -20.91% | 99% | 9190% |
| Class F | -5.23% | -11.89% | -11.56% | 102% | 12932% |

|  | Low delay B |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.71% | -14.44% | -14.80% | 164% | 30122% |
| Class C | -9.47% | -14.22% | -14.69% | 141% | 24305% |
| Class E | -8.41% | -15.56% | -13.83% | 250% | 24758% |
| Overall | -8.47% | -14.34% | -14.75% | 173% | 26701% |
| Class D | -11.20% | -18.67% | -18.15% | 134% | 21708% |
| Class F | -5.38% | -7.72% | -6.80% | 209% | 15205% |

## Retrained inter luma model from JVET-Z0112 with only RA data and used only for inter luma slices

The encoding and decoding times are not reliable.

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -9.24% | -17.14% | -20.03% | 181% | 31072% |
| Class A2 | -10.31% | -20.52% | -17.63% | 177% | 27858% |
| Class B | -9.60% | -23.14% | -22.54% | 176% | 28790% |
| Class C | -11.16% | -22.33% | -23.09% | 149% | 26901% |
| Class E |  |  |  |  |  |
| Overall | -10.09% | -21.20% | -21.20% | 169% | 28520% |
| Class D | -13.47% | -24.00% | -24.61% | 147% | 24879% |
| Class F | -5.26% | -12.63% | -12.58% | 217% | 14283% |

|  | All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6.53% | -17.59% | -17.98% | 168% | 21338% |
| Class A2 | -6.63% | -19.47% | -17.09% | 140% | 18228% |
| Class B | -6.68% | -20.43% | -20.46% | 131% | 17056% |
| Class C | -7.61% | -18.40% | -20.71% | 113% | 11061% |
| Class E | -9.86% | -19.99% | -19.43% | 123% | 16859% |
| Overall | -7.39% | -19.28% | -19.37% | 132% | 16228% |
| Class D | -7.64% | -18.69% | -20.90% | 117% | 9771% |
| Class F | -4.16% | -11.84% | -11.53% | 117% | 11688% |

(The result of AI is identical to the result in JVET-Z0112 since the intra models are identical.)

|  | Low delay B |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -7.71% | -14.18% | -14.74% | 180% | 29241% |  |
| Class C | -9.79% | -14.08% | -14.65% | 157% | 24359% |  |
| Class E | -8.02% | -15.60% | -14.14% | 264% | 25254% |  |
| Overall | -8.48% | -14.13% | -14.70% | 189% | 26524% |  |
| Class D | -12.14% | -18.20% | -18.40% | 130% | 20985% |
| Class F | -5.17% | -7.97% | -5.70% | 224% | 14571% |  |

## IPB+skip model tested on top of JVET-AA0111 (combination with deblocking filter and SADL, float32)

The encoding and decoding times are not reliable.

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -9.58% | -17.28% | -19.83% | 305% | 101899% |
| Class A2 | -10.71% | -21.57% | -18.45% | 295% | 91792% |
| Class B | -9.82% | -24.56% | -23.27% | 323% | 83705% |
| Class C | -11.02% | -22.99% | -23.96% | 241% | 94095% |
| Class E |  |  |  |  |  |
| Overall | -10.27% | -22.09% | -21.80% | 290% | 91495% |
| Class D | -13.04% | -24.80% | -25.04% | 243% | 85603% |
| Class F | -5.69% | -12.73% | -11.90% | 436% | 44027% |

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6.54% | -17.95% | -19.80% | 267% | 64719% |
| Class A2 | -6.67% | -20.40% | -17.65% | 191% | 50891% |
| Class B | -6.62% | -21.39% | -21.29% | 181% | 51167% |
| Class C | -7.31% | -18.73% | -21.79% | 153% | 33968% |
| Class E | -9.90% | -20.80% | -20.61% | 208% | 60503% |
| Overall | -7.32% | -19.96% | -20.43% | 192% | 49911% |
| Class D | -7.25% | -19.37% | -21.33% | 155% | 32368% |
| Class F | -5.00% | -11.77% | -10.92% | 137% | 41595% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class C | -9.54% | -15.20% | -15.57% | 248% | 74664% |
| Class E | -9.30% | -18.23% | -19.23% | 588% | 65928% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class D | -11.40% | -19.02% | -18.53% | 235% | 71384% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusions

This contribution proposes to use one luma model with additional IPB and skip inputs for NN-filtering of intra luma and inter luma blocks. It is shown that one luma model trained with AI and RA data can be used for both intra luma and inter luma blocks. It is further demonstrated that with additional IPB and skip inputs, the model gives 0.35% (= 9.99% − 9.64%) higher gain than the model without IPB or skip inputs in the tests carried out in the contribution. It is also shown that the proposed IPB+skip model has a minor loss of 0.1% comparing to using separate intra luma and inter luma models. The IPB+skip model tested on top of JVET-AA0111 EE1-1.6.2 (a combination of deblocking and SADL) gives a luma gain of 10.27% for RA.

The contribution proposes to further study the IPB+skip model for NN loop filters in an exploration experiment and study if we can get the same gain using just IPB or just skip as an input.

# References

[1] Y. Li, K. Zhang, L. Zhang, H. Wang, M. Coban, A. M. Kotra, M. Karczewicz, F. Galpin, “EE1-1.6: Test on Deep In-Loop Filter with Adaptive Parameter Selection and Residual Scaling based on SADL implementation”, JVET-Z0112, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 26th Meeting, by teleconference, 20–29 April 2022

[2] Common Test Conditions and evaluation procedures for neural network-based video coding technology, E. Alshina, R.-L. Liao, S. Liu, A. Segall, JVET-X2016, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 24th Meeting, by teleconference, 6–15 October 2021

[3] https://pypi.org/project/thop/

[4] EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation, JVET-AA0111, Y. Li, K. Zhang, J. Li, L. Zhang, H. Wang, M. Coban, A.M. Kotra, M. Karczewicz, F. Galpin, K. Andersson, J. Ström, D. Liu, R. Sjöberg, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 27th Meeting, by teleconference, 13–22 July 2022

[5] AHG11: Small Ad-hoc Deep-Learning Library (SADL) update, JVET-AA0086, F. Galpin, T. Dumas, P. Bordes, E. François, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 27th Meeting, by teleconference, 13–22 July 2022

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).