| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0095-v1 |
| --- | --- |

| Title: | EE2-5: Adaptive filter shape switch and using samples before deblocking filter for adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121, USA Wenbin Yin Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China Kai Zhang, Li Zhang 8910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | nanh@qti.qualcomm.com yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Qualcomm Incorporated and Bytedance Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2 tests related to adaptive loop filter (ALF) including coding tree block (CTB) level ALF shape switch and longer ALF shape (EE2-5.1), using samples before deblocking filter for ALF (EE2-5.2) and combined tests of EE2-5.1 and EE2-5.2. On top of ECM-5.0 with common test condition, simulation results are reported as below.

EE2-5.1a (CTB-level ALF shape switch)

AI:     -0.04%, 0.00%, 0.00%, 102%, 99%.

RA     -0.11%, 0.02%, 0.03%, 101%, 100%.

LDB:   -0.20%, -0.12%, 0.01%, 103%, 100%.

EE2-5.1b (Long filter length for ALF)

AI:     -0.04%, 0.01%, 0.00%, 100%, 100%.

RA     -0.08%, -0.07%, 0.00%, 100%, 100%.

LDB:   -0.19%, 0.05%, -0.21%, 100%, 100%.

EE2-5.2 (Using samples before deblocking filter for ALF)

AI:     -0.16%, 0.01%, 0.01%, 101%, 101%.

RA     -0.14%, 0.05%, 0.03%, 100%, 101%.

LDB:   -0.08%, 0.08%, 0.24%, 103%, 103%.

EE2-5.3a (Combination of EE2-5.1a and EE2-5.2)

AI:     -0.20%, 0.02%, 0.02%, 104%, 102%.

RA     -0.25%, 0.09%, 0.05%, 102%, 102%.

LDB:   -0.27%, -0.03%, -0.03%, 105%, 102%.

EE2-5.3b (Combination of EE2-5.1b and EE2-5.2)

AI:     -0.19%, 0.02%, 0.01%, 102%, 101%.

RA     -0.23%, 0.08%, -0.02%, 101%, 102%.

LDB:   -0.22%, -0.07%, 0.04%, 101%, 101%.

# Introduction

In last JVET meeting, improved adaptive loop filter (ALF) was proposed and studied in EE2. Firstly, longer filter and coding tree block (CTB) level ALF shape switch were proposed in JVET-Z0149 and tested in EE2.5-1. Secondly, using samples before deblocking filter (DBF) for ALF was proposed in JVET-Z0146 and tested in EE2.5-2. At last, the combination of these two methods were tested in EE2.5-3.

# Proposed methods

## Test 5.1a

In EE2-5.1a, two candidate filter shapes: a diamond shape as shown in Figure 1, which is applied in ECM-5.0 and a cross shape as shown in Figure 2, which is newly introduced, can be adaptively selected by the luma filters in ALF. The number of coefficients of a luma filter is 22 for both the filter shapes. Please note that these 22 taps are constituted with 20 spatial taps and 2 fixed filters based taps in both shapes.

In each adaptation parameter set (APS), a shape index for the derived luma filters is signaled to decoder. Each APS contains the luma filters that are associated with the filter shape index.

For each CTB, an APS index is signaled to indicate which luma filter shape is used to filter the current CTB. When filtering a luma sample, the coefficients and clip indices are also rearranged according to the corresponding filter shape.

Figure 1. The diamond shape of ALF in ECM-5.0.

Figure 2. The newly introduced filter shape for ALF.

## Test 5.1b

In EE2-5.1b, for luma ALF, the diamond shape in ECM-5.0 is replaced by the longer filter shown in Figure 2.

## Test 5.2

In EE2-5.2 samples before deblocking filters are used as additional inputs for ALF. A final ALF sample is derived by weighting the regular ALF and the filter applied to the samples before the deblocking filter. Specifically, a filtered sample is derived as

Rx,y=Rx, y+i=019cifi,0+fi,1+i=2021cigi+i=22Ncihi,0+hi,1

where fi,j  is the clipped difference between a neighboring sample and current sample Rx, y, gi is the clipped difference between an intermediate sample and current sample Rx, y and hi,j  is the clipped difference between a neighboring sample before DBF and current sample Rx, y. The filter coefficients ci, i=0,…24 are signalled. In this test, 3x3 diamond shape is applied to samples before deblocking filter. In an APS, a flag is signalled to indicate whether samples before DBF are used for ALF. In this proposal, this flag is always set as true at encoder.

## Test 5.3 (Combined tests)

Two combined tests are performed in EE2.5-3 as follows:

| # | Test |
| --- | --- |
| 5.3a | Combination of 5.1a and 5.2 (CTB level ALF shape switch and samples before DBF are used for both filter shapes) |
| 5.3b | Combination of 5.1b and 5.2 (Longer ALF shape and samples before DBF are used for this shape) |

# Simulation results

The tests are implemented on top of ECM-5.0 and tested with common test condition described in JVET-Y2017.

## Test 5.1a

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.01% | 0.01% | 104% | 99% |
| Class A2 | -0.10% | 0.00% | 0.00% | 102% | 99% |
| Class B | -0.02% | 0.00% | 0.00% | 102% | 99% |
| Class C | -0.05% | 0.00% | 0.00% | 101% | 100% |
| Class E | -0.02% | 0.01% | 0.01% | 103% | 98% |
| Overall | -0.04% | 0.00% | 0.00% | 102% | 99% |
| Class D | -0.03% | 0.01% | 0.01% | 101% | 99% |
| Class F | -0.04% | 0.00% | 0.00% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.03% | -0.01% | 101% | 100% |
| Class A2 | -0.14% | 0.01% | 0.04% | 101% | 100% |
| Class B | -0.04% | 0.01% | -0.03% | 101% | 100% |
| Class C | -0.20% | 0.03% | 0.11% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | 0.02% | 0.03% | 101% | 100% |
| Class D | -0.32% | -0.13% | -0.08% | 101% | 101% |
| Class F | -0.08% | 0.00% | 0.03% | 105% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.21% | -0.27% | -0.11% | 102% | 101% |
| Class C | -0.21% | 0.02% | 0.04% | 101% | 100% |
| Class E | -0.19% | -0.07% | 0.16% | 105% | 99% |
| Overall | -0.20% | -0.12% | 0.01% | 103% | 100% |
| Class D | -0.22% | 0.78% | 0.52% | 103% | 100% |
| Class F | -0.17% | -0.55% | -0.15% | 106% | 99% |

## Test 5.1b

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.02% | -0.01% | 100% | 100% |
| Class A2 | -0.10% | -0.01% | 0.03% | 100% | 100% |
| Class B | -0.01% | 0.05% | -0.01% | 100% | 99% |
| Class C | -0.05% | -0.01% | -0.02% | 101% | 101% |
| Class E | 0.00% | 0.00% | 0.02% | 100% | 100% |
| Overall | -0.04% | 0.01% | 0.00% | 100% | 100% |
| Class D | -0.02% | 0.03% | 0.01% | 100% | 100% |
| Class F | -0.03% | -0.01% | 0.03% | 102% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.03% | -0.06% | 100% | 100% |
| Class A2 | -0.14% | 0.06% | 0.06% | 100% | 100% |
| Class B | 0.01% | -0.22% | -0.06% | 100% | 100% |
| Class C | -0.18% | 0.00% | 0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.07% | 0.00% | 100% | 100% |
| Class D | -0.31% | -0.26% | 0.22% | 100% | 101% |
| Class F | -0.03% | -0.07% | 0.09% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | -0.01% | 0.09% | 100% | 100% |
| Class C | -0.22% | 0.01% | -0.23% | 100% | 99% |
| Class E | -0.28% | 0.22% | -0.68% | 99% | 101% |
| Overall | -0.19% | 0.05% | -0.21% | 100% | 100% |
| Class D | -0.36% | 0.10% | 0.92% | 99% | 100% |
| Class F | -0.52% | -0.41% | 0.02% | 100% | 99% |

## Test 5.2

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | 0.01% | 0.01% | 103% | 102% |
| Class A2 | -0.20% | 0.02% | 0.02% | 101% | 103% |
| Class B | -0.21% | 0.01% | 0.02% | 100% | 100% |
| Class C | -0.11% | 0.00% | 0.00% | 100% | 100% |
| Class E | -0.09% | 0.02% | 0.02% | 101% | 102% |
| Overall | -0.16% | 0.01% | 0.01% | 101% | 101% |
| Class D | -0.06% | 0.03% | 0.03% | 101% | 102% |
| Class F | -0.18% | 0.04% | 0.04% | 100% | 103% |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.02% | 0.03% | 101% | 101% |
| Class A2 | -0.20% | 0.14% | 0.05% | 101% | 101% |
| Class B | -0.16% | 0.06% | -0.04% | 100% | 100% |
| Class C | -0.08% | 0.03% | 0.11% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | 0.05% | 0.03% | 100% | 101% |
| Class D | -0.07% | -0.06% | 0.22% | 101% | 100% |
| Class F | -0.15% | 0.00% | 0.12% | 101% | 101% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | 0.11% | 0.09% | 104% | 104% |
| Class C | -0.04% | 0.14% | 0.19% | 104% | 104% |
| Class E | -0.17% | -0.05% | 0.57% | 102% | 100% |
| Overall | -0.08% | 0.08% | 0.24% | 103% | 103% |
| Class D | -0.15% | 0.47% | 0.72% | 107% | 117% |
| Class F | -0.43% | -0.09% | 0.22% | 103% | 103% |

## Test 5.3a

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | 0.03% | 0.03% | 107% | 104% |
| Class A2 | -0.30% | 0.02% | 0.02% | 103% | 103% |
| Class B | -0.22% | 0.02% | 0.02% | 103% | 102% |
| Class C | -0.16% | 0.00% | 0.00% | 102% | 101% |
| Class E | -0.11% | 0.03% | 0.03% | 105% | 103% |
| Overall | -0.20% | 0.02% | 0.02% | 104% | 102% |
| Class D | -0.10% | 0.03% | 0.04% | 103% | 101% |
| Class F | -0.21% | 0.04% | 0.04% | 101% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | 0.09% | -0.04% | 102% | 102% |
| Class A2 | -0.35% | 0.14% | 0.13% | 102% | 102% |
| Class B | -0.23% | 0.04% | 0.00% | 102% | 102% |
| Class C | -0.29% | 0.10% | 0.12% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | 0.09% | 0.05% | 102% | 102% |
| Class D | -0.37% | -0.08% | 0.16% | 104% | 101% |
| Class F | -0.22% | 0.06% | 0.13% | 108% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.26% | -0.25% | -0.14% | 103% | 103% |
| Class C | -0.28% | 0.22% | 0.24% | 103% | 101% |
| Class E | -0.26% | 0.00% | -0.21% | 109% | 101% |
| Overall | -0.27% | -0.03% | -0.03% | 105% | 102% |
| Class D | -0.40% | 0.72% | 0.48% | 106% | 101% |
| Class F | -0.40% | -0.57% | -0.21% | 108% | 101% |

## Test 5.3b

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | 0.00% | 0.01% | 103% | 102% |
| Class A2 | -0.30% | 0.01% | 0.05% | 101% | 102% |
| Class B | -0.21% | 0.06% | 0.00% | 102% | 101% |
| Class C | -0.16% | 0.00% | -0.01% | 101% | 101% |
| Class E | -0.09% | 0.02% | 0.04% | 102% | 102% |
| Overall | -0.19% | 0.02% | 0.01% | 102% | 101% |
| Class D | -0.09% | 0.06% | 0.03% | 102% | 101% |
| Class F | -0.20% | 0.03% | 0.07% | 101% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | 0.19% | -0.14% | 101% | 102% |
| Class A2 | -0.35% | 0.13% | 0.11% | 100% | 102% |
| Class B | -0.20% | -0.02% | -0.10% | 100% | 101% |
| Class C | -0.25% | 0.08% | 0.07% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.23% | 0.08% | -0.02% | 101% | 102% |
| Class D | -0.36% | -0.31% | 0.07% | 102% | 102% |
| Class F | -0.19% | 0.08% | 0.25% | 103% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.19% | -0.18% | 0.18% | 101% | 103% |
| Class C | -0.22% | 0.00% | 0.08% | 100% | 100% |
| Class E | -0.28% | 0.00% | -0.23% | 103% | 101% |
| Overall | -0.22% | -0.07% | 0.04% | 101% | 101% |
| Class D | -0.35% | 0.57% | 0.42% | 101% | 100% |
| Class F | -0.38% | -0.65% | -0.90% | 103% | 101% |

# Patent rights declaration(s)

Qualcomm incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).