| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0102-v1 |
| --- | --- |

| Title: | AHG9: SEI processing order SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peng Yin, Sean McCarthy, Walt Husak, Konstantinos Konstantinides, Taoran Lu, Fangjun Pu, , Arjun Arora, Tong Shao | Email: | {sean.mccarthy, pyin}@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

An SEI message for carrying information on the preferred order of processing SEI messages is proposed. An encoder (i.e., content author) could use the proposed SEI message to indicate that SEI messages of a particular payloadType should be processed by the receiving system either before or after SEI messages of another payloadType to enable an adequate user experience. As an example, the SEI message processing order SEI message could be used to indicate that information in a colour transform information SEI message should be processed before neural-network post-filtering using information in NNPF SEI messages. In this case, for example, the processing order could matter because the NNPF was trained using original (unmapped) luma and chroma sample values rather than mapped luma and chroma sample values. The proposed SEI processing order SEI message is modelled on the SEI manifest SEI message.

# Introduction

The proposed SEI processing order SEI message is intended to carry information that can be complimentary to information carried in an SEI manifest SEI message.

An encoder (i.e., content producer) could use an SEI manifest SEI message to indicate that one or more SEI messages is expected (i.e., likely) to be present in the CVS and the degree of necessity of interpretation of the SEI message as "necessary", "unnecessary", or "undetermined" in order to properly process the content and enable an adequate user experience. For example, an encoder could use an SEI manifest SEI message to indicate that colour transform information (CTI) and neural-network post-filter (NNPF) SEI messages are expected to be present in the CVS and that interpretation of both types of SEI messages is "necessary". However, the SEI manifest SEI message does not indicate a degree of necessity of interpretation of SEI messages in a particular order as "necessary", "unnecessary", or "undetermined" in order to properly process the content and enable an adequate user experience

The order of processing information carried in SEI messages can be essential to ensure acceptable video system performance or user experience, as illustrated in Figures 1-3.

Figure 1 – Illustration of the effect that order of processing of information from NNPF and CTI SEI messages for a neural-network post filter that was trained on original (unmapped) rather than mapped luma and chroma sample values.

Figure 1 illustrates the example of a neural-network post filter that was trained on original (unmapped) luma and chroma sample values. For such a neural-network post filter, when luma and chroma sample values in the decoded picture are intended to be remapped to the original (unmapped) domain using information in a CTI SEI message present in the CVS, the remapping process should happen before neural-network post-filtering (Figure 1A), i.e., the intermediate signal YUV1 should be available to the neural network post filter to produce output signal YUV2. Performing the indicated processing in the reverse order (Figure 1B) could result in a different output signal YUV4 and an inadequate user experience.

Figure 2 – Illustration of the effect of order of processing of information from FGC and CTI SEI messages

Figure 2 illustrates the example of film grain synthesis (using information in a film grain characteristics (FGC) SEI message) applied to either luma and chroma sample values remapped to the original (unmapped) domain (using information in a CTI SEI message) or to luma and chroma sample values that have not yet been remapped to the original domain. When luma and chroma sample values in the decoded picture are intended to be remapped to the original (unmapped) domain, a content producer could intend for the remapping process to happen before the film grain synthesis and blending processes (Figure 2A). Performing the indicated processing in the reverse order (Figure 2B) could alter the strength and look of the film grain in a manner that results in an inadequate user experience.

Figure 3 – Illustration of the effect of order of processing of information from NNPF and FGC SEI messages

Figure 3 illustrates the example of film grain synthesis (using information in a FGC SEI message) in combination with changing the width and height of a decoded picture using information in NNPF SEI messages. When a content producer intends the size of the decoded picture to be modified before display (for example, by using a neural-network post filter and information in NNPF SEI messages in the CVS), a content producer could intend for the resizing process to happen before the film grain synthesis and blending processes (Figure 3A). Performing the indicated processing in the reverse order (Figure 2B) could alter the strength and frequency characteristics of the film grain and result in an inadequate user experience.

Other SEI message types for which the order of processing could matter include: user_data_registered_itu_t_t35; user_data_unregistered; frame_packing_arrangement; shutter_interval_info; and omnidirectional video related SEI messages.

# Proposed syntax and semantics

## Proposed additions to the VVC specification

It is proposed to add the following to the VVC specification:

### Payload type

In subclause D.2.1, add the following, yellow-highlighted table row:

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| ... |  |
| else if( payloadType = = 212 ) |  |
| sei_processing_order( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| ... |  |

### Persistence scope

In subclause D.2.2, add the following, yellow-highlighted table row in Table D-1:

Table 4 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| ... |  |
| SEI processing order | The CVS containing the SEI message |

### Syntax

In Annex D, add the following syntax table

D.xx	SEI processing order SEI message syntax

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_num_sei_msg_types | u(16) |
| for( i = 0; i < po_num_sei_msg_types; i++ ) { |  |
| po_sei_payload_type[ i ] | u(16) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| } |  |

### Semantics

In Annex D.xx, add the following semantics

The SEI processing order SEI message carries information indicating the preferred processing order for SEI messages of a particular value of payloadType. Such information may include the following:

–	The indication that certain types of SEI messages are expected (i.e., likely) to be present (although not guaranteed to be present) in the CVS.

–	For each type of SEI message that is indicated as expected (i.e., likely) to be present in the CVS, the preferred processing order for SEI messages of this type, as follows:

–	The preferred order of processing an SEI message type relative to the order of processing an SEI message of a different type.

–	An SEI message is indicated by the encoder (i.e., the content producer) as having a preferred order of processing when the information conveyed by the SEI message is considered as necessary for interpretation by the decoder or receiving system in order to properly process the content and enable an adequate user experience; it does not mean that the bitstream is required to contain the SEI message in order to be a conforming bitstream. It is at the discretion of the encoder to determine which SEI messages are to be considered as having a preferred processing order in a particular CVS. However, it is suggested that some SEI messages, such as the neural-network post-filter activation, colour transform information, and film grain characteristics SEI messages could typically be considered as having a preferred processing order.

The content of an SEI processing order SEI message may, for example, be used by transport-layer or systems-layer processing elements to determine whether the CVS is suitable for delivery to a receiving and decoding system, based on whether the receiving system can properly process the CVS to enable an adequate user experience or whether the CVS satisfies the application needs (as determined in some manner by external means outside the scope of this document).

When an SEI processing order SEI message is present in any access unit of a CVS, an SEI processing order SEI message shall be present in the first access unit of the CVS. The SEI processing order SEI message persists in decoding order from the current access unit until the end of the CVS. When there are multiple SEI processing order SEI messages present in a CVS, they shall have the same content.

An SEI NAL unit containing an SEI processing order SEI message shall not contain any other SEI messages other than an SEI manifest SEI message and SEI prefix indication SEI messages.

D.xx	SEI message processing order SEI message semantics

po_num_sei_msg_types specifies the number of types of SEI messages for which information is provided in the SEI processing order SEI message.

po_sei_payload_type[ i ] specifies the value of payloadType for the i-th SEI message for which information is provided in the SEI processing order SEI message. The values of po_sei_payload_type[ m ] and po_sei_payload_type[ n ] shall not be identical when m is not equal to n.

po_sei_processing_order[ i ] indicates the preferred order of processing any SEI message with payloadType equal to po_sei_payload_type[ i ]. po_sei_processing_order[ m] greater than 0 and less than po_sei_processing_order[ n ] indicates any SEI message with payloadType equal to po_sei_payload_type[ m ], when present, should be processed before any SEI message with payloadType equal to po_sei_payload_type[ n ], when present. po_sei_processing_order[ i ] equal to 0 specifies that the preferred order of processing SEI messages with payloadType equal to po_sei_payload_type[ i ] is unknown or unspecified or determined by external means not specified in this Specification.

In Annex D.8.2 SEI manifest SEI message semantic, the modify the following paragraph as highlighted in yellow:

An SEI NAL unit containing an SEI manifest SEI message shall not contain any other SEI messages other than SEI prefix indication SEI messages and an SEI processing order SEI message. When present in an SEI NAL unit, the SEI manifest SEI message shall be the first SEI message in the SEI NAL unit.

In Annex D.9.2 SEI prefix indication SEI message semantic, the modify the following paragraph as highlighted in yellow:

An SEI NAL unit containing an SEI prefix indication SEI message for a particular value of payloadType shall not contain any other SEI messages other than an SEI manifest SEI message, an SEI processing order SEI message, and SEI prefix indication SEI messages for other values of payloadType.

# Conclusion

An SEI message for carrying information on the preferred order of processing SEI messages is proposed for adoption in VVC. The proposed SEI processing order SEI message can be used to indicate that SEI messages of a particular type should be processed either before or after SEI messages of another type by the decoder or receiving system in order to properly process the content and enable an adequate user experience. The proposed SEI processing order SEI message is modelled on, and designed to complement, the SEI manifest SEI message.

# References

- Y.-K. Wang, T. Stockhammer, D. Singer, A.M. Tourapis “SEI manifest and prefix indication SEI messages” JCTVC-AD0021, Gwangju, KR, Jan 2018

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).