| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0106-v3 |
| --- | --- |

| Title: | EE2-1.7: IBC Adaptation for Camera-Captured Content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu*, Yang Wang# *8910 University Center Lane, San Diego, CA92122, USA #Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This is a test report for the EE2 test 1.7, i.e. IBC adaptation for camera-captured content. Compared with ECM-5.0, the overall BD-rates (Y/U/V) and EncT/DecT for All Intra are –0.90/–0.98/–1.01/130%/101%. When applying the same encoder change to VTM-11.0ecm, the corresponding BD-rate (Y/U/V) and EncT/DecT compared with the VTM11.0ecm anchor are –0.53/–0.51/–0.51/160%/101%.

# Introduction

In JVET-Z0157, it is proposed to adapt the IBC tool for camera-captured content, i.e. class A to E test sequences. It was included in the EE2 as Test 1.7. This document reports the investigation results based on ECM-5.0.

# Proposed methods

## Decoder change

For class A1 and A2, CTU size with 256 is used by default. In the ECM-4.0 software, using CTU size 256 and IBC together menas that 2x256 rows of luma samples above the current CTU row are required to be kept, which is much larger than the memory usages for intraTMP. It is lso much largr than that of applying IBC when CTU size is 128, which is 2x128 lines. In order to keep the memory usage in terms of luma sample lines the same regardless of whether the CTU size is 128 or 256. Reference area shown in Figure 1 is proposed when CTU size is 256. Please note that it is the same design as in JVET-Z0157. Specifically, when CTU size is 256, for CTU (m, n), the reference area includes CTUs with index (m,0)…(m,n) and (m–1,n–1)…(m-1,W). While when CTU size is 128, the reference area is still CTUs with index (m–2,n–2)…(m–2,W),(m–1,0)…(m–1,W),(m,0)…(m,n), where W is the maximum horizntal CTU index.

Figure 1. Reference area for IBC when CTU size is 256. The green and blue blocks denote the reference area while the white blocks denote invalid reference area.

## Encoder change

To reduce the encoding run-time, the local search range for per-sample search is set to [–8,8] horizontally and [–8,8] vertically centered at the first block vector predictor of the current CU. It is noted that such an enoder change is used only for class A to E. For classF or classTGM, the local search range does not change.

# Test results

Table 1 shows the summarized results of the proposed method with IBC=1. Detailed results can be found in the accompanying Excel file JVET-AA0106-v2.xlsm.

Table 1. The summarized results of the test (IBC=1) compared with the ECM-5.0 anchor.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 | -0.13% |  | -0.05% |  | -0.17% | 132% |  | 102% |  |
| Class A2 | -1.33% |  | -1.81% |  | -1.79% | 130% |  | 101% |  |
| Class B | -1.13% |  | -1.23% |  | -1.25% | 130% |  | 101% |  |
| Class C | -0.36% |  | -0.32% |  | -0.25% | 127% |  | 102% |  |
| Class E | -1.55% |  | -1.52% |  | -1.69% | 131% |  | 102% |  |
| Overall | -0.90% |  | -0.98% |  | -1.01% | 130% |  | 101% |  |
| Class D | -0.61% |  | -0.67% |  | -0.77% | 124% |  | 101% |  |
| Class F | 0.00% |  | 0.00% |  | 0.00% | 100% |  | 100% |  |
|  | Random access Main10 |  |  |  |  |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  |  | EncT |  | DecT |
| Class A1 | 0.07% | -0.06% |  | -0.01% |  |  | 104% |  | 99% |
| Class A2 | -0.46% | -0.35% |  | -0.34% |  |  | 103% |  | 100% |
| Class B | -0.46% | -0.73% |  | -0.70% |  |  | 105% |  | 100% |
| Class C | 0.18% | 0.08% |  | -0.04% |  |  | 106% |  | 99% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.18% | -0.30% |  | -0.31% |  |  | 105% |  | 100% |
| Class D | 0.13% | -0.44% |  | 0.11% |  |  | 106% |  | 99% |
| Class F | 0.00% | 0.00% |  | 0.00% |  |  | 100% |  | 100% |
|  |  |  |  | Low delay B Main10 |  |  |  |  |  |
|  |  |  |  | Over ECM-5.0 |  |  |  |  |  |
|  | Y | U |  | V |  |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.12% | -0.73% |  | -0.40% |  |  | 114% |  | 97% |
| Class C | -0.11% | -0.02% |  | 0.24% |  |  | 114% |  | 100% |
| Class E | -0.21% | -0.06% |  | -0.22% |  |  | 109% |  | 98% |
| Overall | -0.14% | -0.32% |  | -0.14% |  |  | 113% |  | 98% |
| Class D | -0.18% | 0.34% |  | 0.68% |  |  | 115% |  | 100% |
| Class F | 0.00% | 0.00% |  | 0.00% |  |  | 99% |  | 100% |

For comparison, table 2 summarized the results of ECM-5.0 with IBC compared with the ECM-5.0 anchor. Detailed results can be found in the accompanying Excel file JVET-AA0106-ECM-5.0_IBC1.xlsm.

Table 2. The summarized results of ECM-5.0 with IBC=1 compared with the ECM-5.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.06% | -0.09% | 265% | 105% |
| Class A2 | -1.50% | -2.02% | -1.95% | 275% | 102% |
| Class B | -1.32% | -1.40% | -1.58% | 179% | 105% |
| Class C | -0.55% | -0.52% | -0.43% | 168% | 107% |
| Class E | -2.02% | -2.04% | -2.08% | 184% | 107% |
| Overall | -1.11% | -1.19% | -1.22% | 203% | 105% |
| Class D | -0.71% | -0.88% | -0.95% | 153% | 104% |

The same encoder change is also tested based on the latest VTM-11.0ecm software, which is available at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC. The AI results are summarized in table 3. Detailed results can be found in the accompanying Excel file JVET-AA0106-VTM-11.0ecm_fast_IBC.xlsm.

Table 3. The summarized results of VTM-11.0ecm with the encoder change and IBC=1 compared with the VTM-11.0ecm anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.12% | 0.11% | 146% | 100% |
| Class A2 | -0.72% | -0.90% | -0.86% | 154% | 99% |
| Class B | -0.69% | -0.63% | -0.71% | 165% | 101% |
| Class C | -0.23% | -0.17% | -0.17% | 165% | 101% |
| Class E | -0.96% | -1.02% | -0.90% | 168% | 101% |
| Overall | -0.53% | -0.51% | -0.51% | 160% | 101% |
| Class D | -0.50% | -0.77% | -0.51% | 156% | 99% |

Table 4 compared the AI performance with different local search range setting for per-sample search, which can be controlled by the macro LOCAL_SEARCH_RANGE in the modified software, as shown in JVET-AA0106.patch, which is also attached with this proposal. It is noted that value 8 corresponds to the results shown in table 1.

Table 4. Performance comparison with different LOCAL_SEARCH_RANGE setting.

| LOCAL_SEARCH_RANGE | BD-rate Y | BD-rate U | BD-rate V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| 8 | -0.90% | -0.98% | -1.01% | 130% | 101% |
| 16 | -0.95% | -1.03% | -1.07% | 130% | 101% |
| 32 | -1.03% | -1.15% | -1.15% | 132% | 101% |
| 64 | -1.09% | -1.21% | -1.19% | 140% | 101% |
| 128 | -1.13% | -1.24% | -1.23% | 169% | 101% |

It seems that value 32 is a good trade-off. Detailed results of LOCAL_SEARCH_RANGE=32 can be found in the accompanying Excel file JVET-AA0106_LS32.xlsm.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).