| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0111-v3 |
| --- | --- |

| Title: | EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Kai Zhang, Junru Li Li Zhang (Bytedance) Hongtao Wang Muhammed Coban Anand Meher Kotra Marta Karczewicz (Qualcomm) Franck Galpin (InterDigital) K. Andersson, J. Ström, D. Liu, R. Sjöberg (Ericsson) | Tel: Email: | yue.li@bytedance.com zhangkai.video@bytedance.com lijunru@bytedance.com lizhang.idm@bytedance.com hongtaow@qti.qualcomm.com Franck.galpin@interdigital.com kenneth.r.andersson @ericsson.com |
| Source: | Bytedance Inc., Qualcomm Inc., InterDigital., Ericsson |  |  |

_____________________________

- Abstract

This contribution presents the EE test results of JVET-Z0113. In EE1-1.6.1, design and performance are identical to JVET-Y-EE1-1.7. The models are the re-trained ones from the cross-checker. In EE1-1.6.2 and EE1-1.6.3, convolutional neural network-based in-loop filters are implemented with SADL using floating point-based and fixed point-based calculations, respectively. BD-rate changes of {Y, Cb, Cr} on top of NNVC anchor are reportedly summarized as below:

EE1-1.6.1:

RA: {-9.79%, -22.33%, -22.82%}, AI: {-7.34%, -20.39%, -20.98%}

EE1-1.6.2 (floating point, SADL):

RA: {-10.19%, -22.10%, -21.82%}, AI: {-7.25%, -20.15%, -20.52%}

EE1-1.6.3 (fixed point, SADL):

RA: {-10.20%, -22.11%, -21.71%}, AI: {-7.26%, -20.14%, -20.56%}

Introduction

In the JVET-Z0113 [1], the deep learning-based in-loop filtering method was proposed to improve the coding efficiency of VVC. Specifically, the deep in-loop filter features an adaptive parameter selection mechanism and external attention-based architecture. Furthermore, it is allowed to combine the outputs of VVC deblocking and NN loop filtering in JVET-Z0113. In this EE test, the deep filter is implemented with a standalone C++ library named SADL [2] supporting both floating point-based and fixed point-based calculations. In addition, two chroma components are designed to share the same model parameters to avoid the duplicate inference of the chroma model at the decoder side. Besides, the EE code is reorganized and cleaned up to improve the quality and readability of the code.

Proposed method

The proposed CNN model takes reconstruction, partitioning, boundary strength and QP as input. In addition, residual scaling is adopted, i.e., the differences (a.k.a. residues) between the input samples and the NN filtered samples may be further scaled by the corresponding scaling factors before being added to input samples. To enable a combination with deblocking, the input samples used in the residual scaling is the output of deblocking filtering. The CNN-architecture is conditioned on QP, leading to a unified model to handle different quality levels. Section 2.1 - 2.3 present the CNN structure, the parameter selection, and the residue scaling, respectively. In addition, section 2.4 presents the inference and section 2.5 describes the training process.

Architecture

The backbone of the proposed CNN filter is shown in Fig. 1. The calculation process in the attention module can be written as:

F_out = F_in × f (Rec, Pred, BS, QP) + F_in

where F_in and F_out denote the input and the output of the attention module, respectively. Rec, Pred, BS, and QP stand for the reconstruction, the prediction, the boundary strengths, and the sequence-level input quantization parameter respectively. f comprises 2 convolutional layers, where an activation function is applied after the first convolutional layer. The objective of f is to generate a spatial attention map from external information, which then recalibrates the feature maps F_in.

The input of the network comprises reconstruction, prediction, boundary strengths, and QP (split is additionally taken as input in intra slice models). The numbers of feature maps and attention residual blocks are set as 96 and 8 in the 8-residual block version, while 96 and 16 in the 16-residual block version.

Fig. 1. (a) Architecture of the proposed CNN filtering method. M denotes the number of feature maps. N stands for the number of samples in one dimension. (b) Construction of Attention Residual Block in (a)

Parameter Selection

In the proposed technique, each slice or block could determine whether to apply the CNN-based filter or not. When the CNN-based filter is determined to be applied to a slice/block, which conditional parameter from a candidate list including three candidates derived from QP could be further decided. Denote the sequence level QP as q, the candidate list includes conditional parameters {q, q - 5, q - 10}. The selection process is based on the rate-distortion cost at the encoder side. Indication of on/off control as well as the conditional parameter index, if needed, are signalled in the bitstream. Other details regarding the parameter selection are provided as below.

- The granularity of the filter determination and the parameter selection is dependent on resolution and QP. In the proposed scheme, given a higher resolution and a larger QP, the determination and selection will be performed in a larger region.

- The candidate parameter list is different across different temporal layers. For a high temporal layer, the third candidate q - 10 is replaced by q + 5.

- For all-intra configuration, parameter selection is disabled while on/off control is still preserved.

- A shared conditional parameter is used for the two chroma components.

Residue Scaling

When a NN filter is being applied to reconstructed pictures, a scaling factor is signaled for each color component in the picture header. The difference between the input samples and the NN filtered samples (residues) are scaled by the scaling factors before being added to input samples. To enable a combination with deblocking, the input samples used in the residual scaling is the output of deblocking filtering. The residual scaling process is shown below, where RNN and RDB refer to the outputs of NN filtering and deblocking filtering respectively.

RRefine = (RNN-RDB)×w + RDB=w×RNN + (1-w)×RDB

Inference

SADL is used for performing the inference of the proposed CNN filters in VTM. Both floating point-based and fixed point-based implementations are studied. In the fixed-point implementation, both weights and feature maps are represented with int16 precision using a static quantization method. The network information in the inference stage is provided in Table 1 as suggested by [3].

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1.56M/model, 4 models in total for all tests |
|  | Parameter Precision (Bits) | EE1-1.6.1: 32 (Float) EE1-1.6.2: 32 (Float) EE1-1.6.3:16 (Int) |
|  | Memory Parameter (MB) | EE1-1.6.1: 6.3MB/model, 4 models EE1-1.6.2: 6.3MB/model, 4 models EE1-1.6.3: 3.1MB/model, 4 models |
|  | Multiply Accumulate (kMAC/pixel) | EE1-1.6.1: 649K/pixel (assuming frame-level input) 821K/pixel (assuming block-level input) EE1-1.6.2: 539K/pixel (assuming frame-level input) 682K/pixel (assuming block-level input) EE1-1.6.3: 539K/pixel (assuming frame-level input) 682K/pixel (assuming block-level input) |
| Optional |  |  |
|  | Total Conv. Layers | 25+16 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

PyTorch is used as the training platform. The DIV2K [4] and BVI-DVC [5] datasets are adopted to train the CNN filters of I slices and B slices, respectively. The network information in the training stage is provided in Table 2 as suggested by [3].

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 2 |
|  | Epoch: | 50 |
|  | Batch size: | 64 |
|  | Training time: | 60h/model |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Experimental results

The proposed CNN-based in-loop filtering method is tested on top of VTM-11.0 + new MCTF [6] according to the common test conditions defined in [3], where the new MCTF patch is from [7]. SAO is disabled while ALF (and CCALF) is placed after the proposed CNN-based filtering. Test results are shown in Table 3 ~ Table 8.

EE1-1.6.1:

Design and performance are identical to JVET-Y-EE1-1.7. The models are re-trained by the cross-checker according to the training scripts released in [8]. More details regarding the crosscheck and the training process can be found in [9] and [10] respectively. It can be observed that the retrained CNN models from the crosschecker could bring on average {9.79%, 22.33%, 22.82%} and {7.34%, 20.39%, 20.98%} BD-rate reductions for {Y, Cb, Cr} under RA and AI configurations.

EE1-1.6.2:

It can be observed that the proposed CNN models of floating point-based implementation could bring on average {10.19%, 22.10%, 21.82%} and {7.25%, 20.15%, 20.52%} BD-rate reductions for {Y, Cb, Cr} under RA and AI configurations.

EE1-1.6.3

It can be observed that the proposed CNN models of fixed point-based implementation could bring on average {10.20%, 22.11%, 21.71%} and {7.26%, 20.14%, 20.56%} BD-rate reductions for {Y, Cb, Cr} under RA and AI configurations.

Table 3. RA performance of EE1-1.6.1

|  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -12.07% | -9.47% | -17.90% | -20.01% | 197% | 46749% |
| Class A2 | -13.15% | -10.56% | -21.56% | -22.21% | 189% | 43719% |
| Class B | -12.58% | -9.38% | -24.84% | -23.92% | 194% | 44389% |
| Class C | -12.74% | -9.96% | -23.08% | -24.01% | 167% | 39290% |
| Class E |  |  |  |  |  |  |
| Overall | -12.64% | -9.79% | -22.33% | -22.82% | 186% | 43283% |
| Class D | -14.31% | -11.48% | -24.88% | -26.01% | 164% | 35357% |
| Class F | -6.41% | -4.53% | -13.38% | -13.44% | 252% | 20445% |

Table 4. AI Performance of EE1-1.6.1

|  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -9.95% | -6.56% | -18.49% | -18.93% | 180% | 33276% |
| Class A2 | -9.25% | -6.62% | -20.72% | -18.92% | 148% | 26660% |
| Class B | -9.34% | -6.57% | -21.27% | -21.80% | 143% | 25317% |
| Class C | -10.31% | -7.52% | -19.92% | -22.76% | 128% | 16998% |
| Class E | -12.32% | -9.90% | -21.15% | -21.37% | 145% | 28669% |
| Overall | -10.14% | -7.34% | -20.39% | -20.98% | 146% | 24974% |
| Class D | -10.04% | -7.45% | -20.14% | -22.13% | 125% | 14386% |
| Class F | -5.81% | -3.99% | -13.30% | -12.91% | 123% | 17757% |

Table 5. RA performance of EE1-1.6.2

|  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -12.17% | -9.76% | -17.43% | -19.93% | 267% | 81881% |
| Class A2 | -13.05% | -10.99% | -21.54% | -18.42% | 253% | 78046% |
| Class B | -12.80% | -9.80% | -24.48% | -23.30% | 259% | 81675% |
| Class C | -13.07% | -10.39% | -23.06% | -23.93% | 216% | 72349% |
| Class E |  |  |  |  |  |  |
| Overall | -12.80% | -10.19% | -22.10% | -21.82% | 247% | 78401% |
| Class D | -14.49% | -11.90% | -24.69% | -24.88% | 213% | 66729% |
| Class F | -6.62% | -5.11% | -12.69% | -11.85% | 358% | 37862% |

Table 6. AI Performance of EE1-1.6.2

|  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -9.82% | -6.42% | -17.97% | -19.74% | 242% | 66678% |
| Class A2 | -8.99% | -6.48% | -20.48% | -17.81% | 184% | 53437% |
| Class B | -9.33% | -6.49% | -21.90% | -21.61% | 169% | 51253% |
| Class C | -10.00% | -7.42% | -18.84% | -21.71% | 146% | 35225% |
| Class E | -12.17% | -9.87% | -20.81% | -20.61% | 180% | 58423% |
| Overall | -9.98% | -7.25% | -20.15% | -20.52% | 178% | 50707% |
| Class D | -9.79% | -7.35% | -19.43% | -21.33% | 141% | 29786% |
| Class F | -5.53% | -4.08% | -11.73% | -10.89% | 137% | 34932% |

Table 7. RA performance of EE1-1.6.3

|  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -12.14% | -9.82% | -17.26% | -19.62% | 216% | 54472% |
| Class A2 | -13.03% | -10.98% | -21.45% | -18.38% | 206% | 51754% |
| Class B | -12.82% | -9.82% | -24.61% | -23.24% | 209% | 53872% |
| Class C | -13.06% | -10.38% | -23.14% | -23.86% | 178% | 48331% |
| Class E |  |  |  |  |  |  |
| Overall | -12.79% | -10.20% | -22.11% | -21.71% | 201% | 52032% |
| Class D | -14.49% | -11.88% | -24.79% | -24.96% | 172% | 43913% |
| Class F | -6.67% | -5.15% | -12.75% | -11.91% | 279% | 25148% |

Table 8. AI Performance of EE1-1.6.3

|  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -9.86% | -6.45% | -17.96% | -19.88% | 200% | 43661% |
| Class A2 | -9.00% | -6.50% | -20.46% | -17.82% | 159% | 35104% |
| Class B | -9.36% | -6.52% | -21.87% | -21.59% | 151% | 33814% |
| Class C | -9.99% | -7.40% | -18.86% | -21.76% | 133% | 23565% |
| Class E | -12.18% | -9.87% | -20.85% | -20.66% | 155% | 38562% |
| Overall | -9.99% | -7.26% | -20.14% | -20.56% | 156% | 33494% |
| Class D | -9.78% | -7.32% | -19.50% | -21.39% | 130% | 19936% |
| Class F | -5.52% | -4.06% | -11.76% | -10.93% | 128% | 23010% |

Conclusions

This contribution presents the test result of a CNN-based in-loop filtering method. The proposed CNN-based filtering method shows favorable trade-off in terms of the coding performance and complexity. It is suggested to adopt the fixed point implementation-based version EE1-1.6.3 as the software base for NN-based tool evaluations in the future.

References

- Y. Li, K. Zhang, L. Zhang, H. Wang, M. Coban, A.M. Kotra, M. Karczewicz, F. Galpin, K. Andersson, J. Ström, D. Liu, R. Sjöberg, EE1-1.7: Combined Test of EE1-1.6 and EE1-1.3, JVET-Z0113, by teleconference, April 2022

- F. Galpin, T. Dumas, P. Bordes, E. François, AHG11: SADL update, JVET-Z0161, by teleconference, April 2022.

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, by teleconference, April 2022.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-11.0.

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master/Software%20Patches

- https://vcgit.hhi.fraunhofer.de/jvet-z-ee1/VVCSoftware_VTM/-/tree/EE1-1.6

- K. Lin, C. Jia, S. Wang, Cross-check of JVET-AA0111 (EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation), by teleconference, July 2022.

- Y. Li, K. Zhang, L. Zhang, iDAM: Iteratively Trained Deep In-Loop Filter with Adaptive Model Selection. ACM Transactions on Multimedia Computing, Communications, and Applications (TOMM).

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).