| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0122-v1 |
| --- | --- |

| Title: | EE1-1.3: On BaseQP adjustment in CNNLF |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution reports the EE results of JVET-Z0128[1]. A QP-adjustment method was proposed to improve the coding performance of JVET-Z0091[2] and JVET-Z0094[3]. In those two contributions, the QP parameters were part of the input to the network at the inference stage and these QP parameters include BaseQP and SliceQP. JVET-Z0128 proposed to add an offset to the BaseQP at the slice-level and use the final adjusted BaseQP as the input of the network. On top of those two proposals, it is reported that the proposed method can further improve the coding efficiency, {Y, U, V, EncT, DecT}, shown as follows:

EE1-1.3.1:

RA: Y -0.31%, U -0.14%, V -0.14%, EncT 105%, DecT 100%;

LDB: Y -0.43%, U -0.38%, V -0.21%, EncT 100%, DecT 96%.

EE1-1.3.2:

RA: Y -0.32%, U -0.60%, V -0.43%, EncT 105%, DecT 101%;

LDB: Y -0.44%, U -0.15%, V 0.55%, EncT 107%, DecT 111%

# Introduction

In JVET-Z0094[1], a neural network-based in-loop filter with constrained storage and low complexity was proposed, which was included in EE1 in the last meeting and named EE1-1.4 for further study. Two models were used in EE1-1.4, one for I-Slice frames and the other for B-Slice frames. The specific network framework is provided as shown in the following Figure 1 and the network information in inference stage is provided as shown in Table 1.

Figure 1. NN structure of JVET-Z0094.

Table 1: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Torch v1.9.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.54M |
|  | Total Number of Parameters (All Models) | 3.08M |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 6.16MB/model, 2 models in all |
|  | Multiplay Accumulate (MAC)/pixel, worst-case | 401K |
| Optional |  |  |
|  | Total Conv. Layers | 78 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

For saving memory size, a single-model method was proposed in JVET-Z0091. For different base QPs and different color components, there was only one model used in the whole filtering process. The partition image was not used in the proposed filter, and the slice type was fed into the network to identify the coded type of the input. The specific network framework was provided as shown in the following Figure 2 and the network information in inference stage is provided as shown in Table 2.

Figure 2. NN structure of JVET-Z0091.

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Libtorch v1.9.0 or SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.9M |
|  | Total Number of Parameters (All Models) | 1.9M |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 7.6MB/model, 1 model in all |
|  | Multiplay Accumulate (MAC)/pixel, worst-case | 485K |
| Optional |  |  |
|  | Total Conv. Layers | 101 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

# Proposed method

This contribution proposes that, instead of using the BaseQP directly, a slice-level adjustment can be made to the BaseQP and the adjusted BaseQP is used as the input to the B-Slice model. In this contribution, the adjustment is done by simply adding a small offset to the BaseQP, and the offset candidates used in this contribution are -5 or 5.

For example, supposing the offset {5} is chosen to add to the BaseQP, the input to the model will be (BaseQP + 5) for each and every CTU in the current slice.

A slice-level flag is signalled to indicate whether BaseQP offset is enabled. If this flag is on, another slice-level flag will be signalled to indicate what value of the offset is used, and every CTU in the current slice will take the same offset for BaseQP.

Note that this adaptive QP adjustment method was originally proposed in JVET-Z0128[1].

## Encoder optimization 1

In this optimization scheme, the encoder firstly uses the default BaseQP as input of network, filters all the CTUs in the current slice using the neural network-based loop filter, outputs the reconstructed picture, and calculates the distortion from the original picture, marks it as cost0. Secondly, the encoder adds the first offset candidate {-5} to BaseQP to get the finalBaseQP as the input of network, then completes the same process as step 1 to get the distortion cost1.

If cost1 is smaller than cost0, the encoder will skip testing the second offset candidate {5}.

## Encoder optimization 2

In a contrast to the optimization scheme described above, where the encoder always filters all the CTUs in the slice and accumulates every CTU’s distortion for both the default BaseQP and the finalBaseQPj, this optimization scheme 2 proposes to only filter one out of every four CTUs, which skips three CTU’s inference and saves more encoding time, during the process of selecting the best QP offset. As shown in Figure 3, only the shaded CTUs are filtered and used in calculating the distortion from the original CTUs. The main process is described below:

Firstly, the encoder filters only one out of every four CTUs by using the different BaseQP candidates {BaseQP, BaseQP-5, BaseQP+5} as input of network;

Meanwhile, the encoder calculates and accumulates every filtered CTU’s distortion from the original picture, obtains the costs from the different BaseQP candidates, and selects the best candidate with the smallest cost;

Finally, the encoder filters the rest CTUs, which are not shaded in Figure 3, using the best QP candidate as input of network.

Figure 3. Encoder optimization 2.

# Simulation results

The proposed method is implemented on top of both JVET-Z0091(single mode) and JVET-Z0094(two models). The simulations were performed following the NNVC common test condition (CTC)[4]. Because this method is only applied to B slice, only Random access and Low delay B are tested.

## EE1-1.3.1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over JVET-Z0094 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | 0.28% | -0.42% | 105% | 100% |
| Class A2 | -0.40% | -0.12% | 0.19% | 107% | 98% |
| Class B | -0.28% | -0.26% | -0.18% | 105% | 101% |
| Class C | -0.36% | -0.31% | -0.11% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -0.14% | -0.14% | 105% | 100% |
| Class D | -0.20% | -0.35% | 0.05% | 105% | 102% |
| Class F | -0.06% | 0.14% | 0.17% | 104% | 104% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over JVET-Z0094 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.39% | -0.13% | -0.30% | 100% | 95% |
| Class C | -0.51% | -0.71% | -0.09% | 98% | 90% |
| Class E | -0.38% | 1.39% | 0.20% | 101% | 107% |
| Overall | -0.43% | -0.38% | -0.21% | 100% | 96% |
| Class D | -0.42% | -0.35% | 0.98% | 100% | 92% |
| Class F | 0.01% | -0.03% | 0.31% | 100% | 95% |

## EE1-1.3.2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over JVET-Z0091 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.32% | -0.43% | 105% | 100% |
| Class A2 | -0.36% | -0.41% | -0.25% | 105% | 100% |
| Class B | -0.27% | -0.61% | -0.34% | 106% | 102% |
| Class C | -0.44% | -0.93% | -0.68% | 105% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | -0.60% | -0.43% | 105% | 101% |
| Class D | -0.39% | -0.72% | -0.38% | 105% | 104% |
| Class F | -0.12% | -0.12% | 0.05% | 106% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over JVET-Z0091 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.37% | 0.13% | 0.65% | 108% | 109% |
| Class C | -0.63% | -0.49% | 0.43% | 103% | 104% |
| Class E | -0.31% | 1.72% | 0.51% | 109% | 128% |
| Overall | -0.44% | -0.15% | 0.55% | 107% | 111% |
| Class D | -0.59% | -0.24% | -0.24% | 105% | 100% |
| Class F | -0.19% | -0.31% | 0.59% | 110% | 105% |

# Conclusion

This contribution proposes an adaptive QP adjustment method for JVET-Z0091(further study in AA-EE1-1.5) and JVET-Z0094(further study in AA-EE1-1.4) to further improve the coding performance. This method is implemented on top of those two methods and it is reported that the simulations showed the following Y, U, V BD-rate coding results :

Over single model:

RA: Y -0.32%, U -0.60%, V -0.43%;

LDB: Y -0.47%, U -0.30%, V -0.07%.

Over two models:

RA: Y -0.31%, U -0.14%, V -0.14%;

LDB: Y -0.44%, U -0.15%, V 0.55%.

This method can provide good performance with a small impact on the encoding time and no increase in the decoding time. It is suggested to include this method in EE1-1.4 and EE1-1.5.

# Reference

[1] Zhihuang Xie, et al., “EE1-related: on BaseQP parameter in EE1-1.1”, JVET-Z0128, April. 2022.

[2] Liqiang Wang, et al., “EE1-1.2: neural network based in-loop filter with a single model”, JVET-Z0091, April. 2022.

[3] Liqiang Wang, et al., “EE1-1.1: neural network based in-loop filter with 2 models”, JVET-Z0094, April. 2022.

[4] Shan Liu, et al., “JVET common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-Z2016, April. 2022.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).