| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0126-v2 |
| --- | --- |

| Title: | EE2-1.1c, 1.3a and 1.3b: Combined tests of EE2-1.1a, 1.1b and 1.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pekka Astola Jani Lainema Ramin G. Youvalari Alireza Aminlou Krit Panusopone Che-Wei Kuo, Hong-Jheng Jhu, Xiaoyu Xiu, Ning Yan, Wei Chen, Xianglin Wang | Email: | firstname.lastname@nokia.com {cheweikuo, jhuhong-jheng, xiaoyuxiu, yanning03, chenwei06, xianglinwang}@kwai.com |
| Source: | Nokia, Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports results of EE2 Test-1.1c, 1.3a and 1.3b. Convolutional cross-component model (CCCM) is proposed in JVET-Z0064 and studied in Test-1.1a. Filter-based linear model (FLM)/gradient linear model (GLM) are proposed in JVET-Z0140 and studied in Test-1.1b and Test-1.2. Test-1.1c is the combination of CCCM and FLM, Test-1.3a is the combination of CCCM and GLM, and Test-1.3b is the combination of CCCM, FLM and GLM.

Compared to ECM-5.0 anchors, the BD-rate impact is summarized as below.

| Class A to E |  | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.1c | CCCM+FLM | -1.41% | -3.60% | -3.49% | 104% | 100% | -0.78% | -2.44% | -2.88% | 102% | 99% |
| 1.3a | CCCM+GLM | -1.38% | -3.02% | -2.97% | 106% | 100% | -0.78% | -2.33% | -2.63% | 103% | 99% |
| 1.3b | CCCM+FLM+GLM | -1.40% | -3.75% | -3.60% | 110% | 100% | -0.79% | -2.51% | -2.89% | 104% | 99% |

| Class F |  | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.1c | CCCM+FLM | -0.86% | -3.61% | -3.38% | 103% | 100% | -1.20% | -3.07% | -3.19% | 102% | 100% |
| 1.3a | CCCM+GLM | -1.05% | -4.46% | -3.97% | 104% | 100% | -1.62% | -3.78% | -4.09% | 103% | 100% |
| 1.3b | CCCM+FLM+GLM | -0.97% | -4.88% | -4.56% | 106% | 100% | -1.60% | -3.86% | -4.29% | 103% | 99% |

| Class TGM |  | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.1c | CCCM+FLM | -2.92% | -5.70% | -5.57% | 103% | 99% | -1.64% | -2.65% | -2.59% | 102% | 99% |
| 1.3a | CCCM+GLM | -4.16% | -10.34% | -10.13% | 104% | 98% | -2.65% | -5.03% | -4.98% | 103% | 100% |
| 1.3b | CCCM+FLM+GLM | -4.21% | -10.49% | -10.31% | 107% | 97% | -2.64% | -5.11% | -5.07% | 104% | 98% |

# Proposal

Details of the proposed CCCM can be found in the reports provided for Test-1.1a [1].

Details of the proposed FLM and GLM can be found in Test-1.1b and Test-1.2 [2]. Additionally, when being combined with the CCCM, the following modifications are additionally included to the FLM and GLM designs.

- For the FLM, based on the four filter shapes in Test-1.1b, one additional nonlinear term is introduced into the FLM filters. The definition of the non-linear term is exactly the same as that of the CCCM in Test-1.1a, which is represented as a power of two of the center luma sample.

- Only four out of sixteen gradient filters are enabled for the GLM, as illustrated in Figure 1. Meanwhile, the combination of gradient filters with the existing CCLM down-sampling filter is disabled.

Figure 1 Four Sobel based gradient patterns for GLM.

# Simulation results

The proposed techniques were implemented on ECM-5.0 reference software [3] and tested by following the common test condition (CTC) as specified in [4].

## Test1.1c

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.23% | -3.63% | -5.99% | 103% | 101% |
| Class A2 | -5.69% | -5.19% | -8.10% | 104% | 100% |
| Class B | -0.60% | -4.24% | -2.22% | 103% | 99% |
| Class C | -0.13% | -1.03% | -0.90% | 105% | 101% |
| Class E | -0.36% | -4.32% | -1.97% | 103% | 100% |
| Overall | -1.41% | -3.60% | -3.49% | 104% | 100% |
| Class D | -0.09% | -0.83% | -0.73% | 104% | 99% |
| Class F | -0.86% | -3.61% | -3.38% | 103% | 100% |
| Class TGM | -2.92% | -5.70% | -5.57% | 103% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.15% | -2.35% | -4.70% | 103% | 100% |
| Class A2 | -2.11% | -2.56% | -4.33% | 101% | 98% |
| Class B | -0.33% | -3.88% | -2.66% | 101% | 99% |
| Class C | -0.07% | -0.62% | -0.70% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.78% | -2.44% | -2.88% | 102% | 99% |
| Class D | -0.06% | -0.56% | -0.69% | 102% | 100% |
| Class F | -1.20% | -3.07% | -3.19% | 102% | 100% |
| Class TGM | -1.64% | -2.65% | -2.59% | 102% | 99% |

## Test1.3a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.21% | -2.70% | -4.81% | 104% | 101% |
| Class A2 | -5.57% | -4.72% | -7.60% | 108% | 100% |
| Class B | -0.61% | -3.65% | -1.90% | 105% | 100% |
| Class C | -0.12% | -0.88% | -0.75% | 108% | 102% |
| Class E | -0.35% | -3.44% | -1.25% | 106% | 100% |
| Overall | -1.38% | -3.02% | -2.97% | 106% | 100% |
| Class D | -0.07% | -0.15% | -0.63% | 108% | 102% |
| Class F | -1.05% | -4.46% | -3.97% | 104% | 100% |
| Class TGM | -4.16% | -10.34% | -10.13% | 104% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.13% | -2.22% | -4.13% | 102% | 99% |
| Class A2 | -2.09% | -2.49% | -4.25% | 101% | 97% |
| Class B | -0.37% | -3.60% | -2.46% | 103% | 99% |
| Class C | -0.04% | -0.70% | -0.52% | 104% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.78% | -2.33% | -2.63% | 103% | 99% |
| Class D | -0.02% | -0.74% | -0.41% | 104% | 102% |
| Class F | -1.62% | -3.78% | -4.09% | 103% | 100% |
| Class TGM | -2.65% | -5.03% | -4.98% | 103% | 100% |

## Test1.3b

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.23% | -3.78% | -6.00% | 109% | 100% |
| Class A2 | -5.66% | -5.62% | -8.40% | 113% | 99% |
| Class B | -0.61% | -4.45% | -2.47% | 108% | 100% |
| Class C | -0.11% | -1.02% | -0.91% | 111% | 101% |
| Class E | -0.36% | -4.34% | -1.84% | 110% | 100% |
| Overall | -1.40% | -3.75% | -3.60% | 110% | 100% |
| Class D | -0.07% | -0.19% | -0.60% | 111% | 103% |
| Class F | -0.97% | -4.88% | -4.56% | 106% | 100% |
| Class TGM | -4.21% | -10.49% | -10.31% | 107% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.16% | -2.37% | -4.59% | 105% | 100% |
| Class A2 | -2.12% | -2.92% | -4.42% | 103% | 98% |
| Class B | -0.36% | -3.96% | -2.72% | 103% | 99% |
| Class C | -0.05% | -0.50% | -0.68% | 104% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.79% | -2.51% | -2.89% | 104% | 99% |  |
| Class D | -0.02% | -0.54% | -0.35% | 103% | 101% |
| Class F | -1.60% | -3.86% | -4.29% | 103% | 99% |
| Class TGM | -2.64% | -5.11% | -5.07% | 104% | 98% |

# References

- P. Astola, J. Lainema, R. G. Youvalari, A. Aminlou, K. Panusopone, “ EE2-1.1a: Convolutional cross-component intra prediction model”, JVET-AA0057, Jul. 2022, Teleconference.

- C.-W. Kuo, H.-J. Jhu, X. Xiu, N. Yan, W. Chen, X. Wang, “ EE2-1.1b and 1.2: Filter-based linear model and gradient linear model”, JVET-AA0125, Jul. 2022, Teleconference.

- ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Apr. 2022.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).