| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0127-v1 |
| --- | --- |

| Title: | EE2-1 related: Encoder optimization of EE2-1.2 and 1.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Che-Wei Kuo, Hong-Jheng Jhu, Xiaoyu Xiu, Ning Yan, Wei Chen, Xianglin Wang | Email: | {cheweikuo, jhuhong-jheng, xiaoyuxiu, yanning03, chenwei06, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, encoder optimizations are proposed for EE2-1.2 (gradient linear model) and 1.3a (combination of convolutional cross-component model and gradient linear model). Simulation results show that for EE2-1.2, the encoding runtime is reduced from xxx% to xxx% and xxx% to xxx% for AI and RA, respectively. For EE2-1.3a, the corresponding encoding runtime is reduced from xxx% to xxx% and xxx% to xxx% for AI and RA, respectively. For both two EE tests, the BD-rate impacts of the proposed encoder optimizations are negligible.

Compared to ECM-5.0 anchors, the performance of EE2-1.2 and 1.3a with the proposed encoder optimizations are summarized as below.

| Class A to E |  | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.2 | GLM |  |  |  |  |  |  |  |  |  |  |
| 1.3a | CCCM+GLM |  |  |  |  |  |  |  |  |  |  |

| Class F |  | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.2 | GLM |  |  |  |  |  |  |  |  |  |  |
| 1.3a | CCCM+GLM |  |  |  |  |  |  |  |  |  |  |

| Class TGM |  | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.2 | GLM |  |  |  |  |  |  |  |  |  |  |
| 1.3a | CCCM+GLM |  |  |  |  |  |  |  |  |  |  |

# Proposal

Compared with EE2-1.2 and 1.3a software, the following two encoder optimizations are proposed.

- Software clean-up: removing redundant operations in the gradient calculation process of the GLM at encoder.

- Early R-D termination: bypassing the SATD checking of the combination of the GLM with multiple directional linear model (MDLM) if the stand-alone SATD cost of the GLM is larger than that of CCLM.

# Simulation results

The proposed techniques were implemented on ECM-5.0 reference software [1] and tested by following the common test condition (CTC) as specified in [2].

## Test1.2 with encoder optimization

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.12% | -0.23% | 0.42% | 103% | 99% |
| Class C | 0.02% | 0.02% | 0.10% | 104% | 102% |
| Class E | -0.03% | -0.29% | 0.17% | 103% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | 0.70% | 0.14% | 104% | 105% |
| Class F | -0.46% | -2.21% | -2.05% | 103% | 100% |
| Class TGM | -3.44% | -9.38% | -9.35% | 103% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |

## Test1.3a with encoder optimization

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.60% | -3.48% | -1.75% | 103% | 102% |
| Class C | -0.12% | -0.80% | -0.69% | 105% | 102% |
| Class E | -0.35% | -3.35% | -1.25% | 103% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.09% | -0.09% | -0.44% | 105% | 106% |
| Class F | -1.02% | -4.21% | -3.73% | 103% | 101% |
| Class TGM | -4.11% | -10.02% | -9.71% | 103% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# References

- ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Apr. 2022.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).