| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0129-v1 |
| --- | --- |

| Title: | Non-EE2/AHG10: Improved inter hash RDO considering OBMC off in ECM5.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, C.-W. Kuo, Hong-Jheng Jhu, Wei Chen, Ning Yan, Xianglin Wang | Email: | xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This document presents one encoder optimization for inter-hash-based motion estimation by enabling the R-D cost calculation of disabling OBMC at CU-level. For TGM sequences, the proposed encoder improvement reportedly shows 0.99% and 1.45% BD-rate reduction over ECM5.0 anchor for RA and LDB configurations, respectively, without noticeable impacts on both encoding and decoding run-time.

# Introduction

In ECM5.0 [1], one hash-based block matching scheme is applied to the motion estimation process for screen video contents. Specifically, for each reference picture, multiple hash tables are built for various block sizes. For one inter CU, hash-based motion search is applied at the beginning of the motion estimation process such that when a hash-matched block is found the conventional motion search can be early bypassed.

Overlapped block motion compensation (OBMC) is one inter coding tool that is applied in ECM5.0 [1]. The OBMC can reduce the blocking artifacts at the motion compensation stage by combing the prediction signal of the current CU with the prediction signals generated using the motion vectors (MVs) of spatial neighboring blocks. Additionally, for each non-merge inter CU (which contains more than 32 luma samples), one control flag is signaled to indicate whether the OBMC is enabled for the CU or not. In the ECM5.0, both the rate-distortion costs of enabling the OBMC (i.e., set the OBMC flag to 1) and disabling the OBMC (i.e., set the OBMC flag to 0) are evaluated for conventional motion estimation process. However, for inter-hash-based motion estimation, only the R-D cost of enabling the OBMC is calculated.

# Proposal

Compared to natural camera-captured contents, screen content show very different characteristics. Specifically, due to the nature of camera lens, camera-captured videos usually present smooth transitions across the boundaries of different objects while screen content presents shape edges. Therefore, for screen content videos, the OBMC may blur the meaningful high-frequency information of the motion compensated prediction samples.

To improve the inter coding efficiency of screen content, it is proposed to enable the R-D cost evaluation of disabling the OBMC (i.e., set the OBMC flag to 0) for inter-hash-based motion estimation. Specifically, the existing function xCheckRDCostInterWoOBMC() that is used for the OBMC-OFF R-D checking for conventional motion estimation is reused for the RDO of the inter-hash-based motion estimation. Additionally, to ensure not to increase the encoder run-time, some existing early-skip logics in ECM5.0 are reused to conditionally skip the additional OBMC-OFF R-D process.

# Experimental results

The proposed encoder optimization scheme was implemented on top of the ECM5.0 [1] and tested using the common test conditions as defined in [2].

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.12% | -0.14% | -0.09% | 101% | 100% |
| Class TGM | -0.99% | -0.54% | -0.54% | 99% | 99% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.34% | -0.59% | -0.41% | 100% | 100% |
| Class TGM | -1.45% | -1.08% | -1.12% | 98% | 97% |

# References

- ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Apr. 2022.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).