| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0132-v2 |
| --- | --- |

| Title: | AHG6: ECM software configuration parameters for template matching tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Chun-Chi Chen, 2Han Huang, 3Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 USA | Email: | 1chunchic@qti.qualcomm.com 2hanhuang@qti.qualcomm.com 3vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution provides a software package on top of ECM5 that adds configuration parameters to turn on and off template matching tools to facilitate evaluating coding tool performance without the need of touching default macro settings of template matching tools and re-compiling the software. It is reported that some of the template matching tools (e.g., TM-OBMC, TM-MMVD) in ECM are not configurable using configuration parameters solely instead of macros since such configuration option is not implemented in the software. The provided software package fixes the aforementioned software issues and is capable of handling template matching tools through configuration parameters without the need of software re-compilation and macro changing. Experimental result shows comparison between macro-off and config-off settings for template matching tools. It is reported that both settings provide identical PSNR results.

# Introduction

In ECM5, the way to evaluate template matching off result is to turn off the setting of the macro ENABLE_INTER_TEMPLATE_MATCHING in the software and software re-compilation is required. The ECM5 software provides a configuration option (i.e., --DMVD=0) to turn off decoder-side methods without the need of touching default macro settings; however, it handles only some of the template matching tools while some others are controlled by other configuration options, such as --AML=0 for disabling ARMC and –ARL=0 for disabling reference picture reordering. There are still some of template matching tools configurable only at macro-level setting, such as TM-OBMC and TM-MMVD, and there is no configuration parameter implemented for them. Besides, the configuration option (i.e., --DMVD=0) also disables non-template matching tools, such as multi-pass DMVR. There is no way except for turning the macro ENABLE_INTER_TEMPLATE_MATCHING off to disable template matching tools in ECM software through setting encoder configuration parameters.

To facilitate evaluating coding tool performance with or without template matching tools, this contribution provides a software package on top of ECM5 (d406af21) to add configuration parameters for switching template matching tools without the need of touching default macro settings and re-compiling the software. Experiments were conducted with that the anchor disables all macros of template matching tools and the test keeps default macro settings with all configuration parameters of template matching tools setting equal to off. It is reported that both settings provide identical PSNR results.

# Configuration parameters of template matching tools

The template matching tools supported in ECM5 is listed in the table below, and the corresponding configuration parameters are also listed. When all configuration parameters are off, the codec runs as if the macro ENABLE_INTER_TEMPLATE_MATCHING were setting off.

- Configuration parameters to switch on/off template matching tools.

| Template matching tool | Configuration parameter | Added or ECM5 |
| --- | --- | --- |
| TM-AMVP | TMAmvp | Added |
| TM-MRG | TMMrg | Added |
| TM-GPM | GPMTM | Added |
| TM-CIIP | CIIPTM | Added |
| ARMC | AML | ECM5 |
| TMVP and Non-adjacent MVP reordering | TmvpNmvpAML | Added |
| MVD sign prediction | MVSD | ECM5 |
| TM-MMVD (for MMVD for Affine MMVD) | TMMMVD | Added |
| GPM split mode reordering | AltGPMSplitModeCode | ECM5 |
| Reference picture reordering | ARL | ECM5 |
| TM-OBMC | OBMCTM | Added |

# Test results

The proposed method was implemented on top of the ECM5 software (d406af21) and tested based on the common test condition as specified in the Exploration Experiments 2 document (JVET-Z2024). The anchor disables template matching tool macros, as shown below:

- #define ENABLE_INTER_TEMPLATE_MATCHING  0

The test keeps the default setting of template matching macro but turns off template matching tools using configuration parameters, as follows:

| - TMAmvp | : | 0 |
| --- | --- | --- |
| - TMMrg | : | 0 |
| - GPMTM | : | 0 |
| - CIIPTM | : | 0 |
| - AML | : | 0 |
| - TmvpNmvpAML | : | 0 |
| - MVSD | : | 0 |
| - TMMMVD | : | 0 |
| - AltGPMSplitModeCode | : | 0 |
| - ARL | : | 0 |
| - OBMCTM | : | 0 |

It is reported that both settings provide identical PSNR results.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).