| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0132-v3 |
| --- | --- |

| Title: | AHG6: ECM software configuration parameters for template matching tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Chun-Chi Chen, 2Han Huang, 3Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 USA | Email: | 1chunchic@qti.qualcomm.com 2hanhuang@qti.qualcomm.com 3vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution provides a software package to make ECM5 have the ability to switch each template matching tool using respective configuration parameter to facilitate evaluating coding tool performance without the need of macro settings and software re-compilation. It is reported that some of the template matching tools (e.g., TM-OBMC, TM-MMVD) in ECM5 are not configurable using configuration parameters solely without changing macro settings, since such configuration option is not implemented in the software. Such inconsistency design among template matching tools may limit the configuration-wise flexibility to switch individual template matching tool, not to mention combinations of them. The provided software package fixes the aforementioned software issues and makes ECM5 capable of handling each template matching tool through respective configuration parameter instead of macro changing and software re-compilation. Experimental result shows the comparison between the macro-off and config-off settings for template matching tools. It is reported that both settings provide identical PSNR results and the average BD-rate is 0.0% in each color component, where the anchor is with macro-off setting and the test is with config-off setting.

# Introduction

In ECM5, the way to evaluate template matching off result is to turn off the setting of the macro ENABLE_INTER_TEMPLATE_MATCHING in the software and software re-compilation is required. The ECM5 software provides a configuration option (i.e., --DMVD=0) to turn off decoder-side methods without the need of touching default macro settings; however, it handles only some of the template matching tools while another options control the rest, such as --AML=0 for disabling ARMC and --ARL=0 for disabling reference picture reordering. There are still some of template matching tools configurable only at macro-level setting, such as TM-OBMC and TM-MMVD, and there is no configuration parameter implemented for them. Such inconsistency design among template matching tools (in which some are configurable while others are not) limits the configuration-wise flexibility and readability to switch individual template matching tool, not to mention combinations of them. Besides, the configuration option (i.e., --DMVD=0) is not dedicated to template matching tools. It also disables other decoder-side motion refinement methods, such as multi-pass DMVR. There is no way except for turning the macro ENABLE_INTER_TEMPLATE_MATCHING off to disable template matching tools in ECM software through setting encoder configuration parameters.

To facilitate evaluating coding tool performance with or without template matching tools, this contribution provides a software package on top of ECM5 (d406af21) to add configuration parameters for switching template matching tools without the need of touching default macro settings and re-compiling the software. In addition, these five merge requests (MR180/185/188/189/190) are also integrated into the software package to fix template matching related bugs. Experiments were conducted with that the anchor disables all macros of template matching tools and the test keeps default macro settings with all configuration parameters of template matching tools setting equal to off. It is reported that both settings provide identical PSNR results.

# Configuration parameters of template matching tools

The template matching tools supported in ECM5 is listed in the table below, and the corresponding configuration parameters are also listed. When all configuration parameters are off, the codec runs as if the macro ENABLE_INTER_TEMPLATE_MATCHING were set equal to 0.

- Configuration parameters to switch on/off template matching tools.

| Template matching tool | Configuration parameter | Added or ECM5 |
| --- | --- | --- |
| TM-AMVP | TMAmvp | Added |
| TM-MRG | TMMrg | Added |
| TM-GPM | GPMTM | Added |
| TM-CIIP | CIIPTM | Added |
| ARMC | AML | ECM5 |
| TMVP and Non-adjacent MVP reordering | TmvpNmvpAML | Added |
| MVD sign prediction | MVSD | ECM5 |
| TM-MMVD (for MMVD for Affine MMVD) | TMMMVD | Added |
| GPM split mode reordering | AltGPMSplitModeCode | ECM5 |
| Reference picture reordering | ARL | ECM5 |
| TM-OBMC | OBMCTM | Added |

# Test results

The proposed method was implemented on top of a newer software revision of ECM5 (d406af21) with five merge requests (MR180/185/188/189/190) integrated and tested based on the common test condition as specified in the Exploration Experiments 2 document (JVET-Z2024) except that template matching tools are disabled.

The anchor is with the macro-off setting that disables the macros of template matching tools, as shown below:

- #define ENABLE_INTER_TEMPLATE_MATCHING  0

The test is with the config-off setting which keeps the default macro setting of template matching tools but turns the template matching tools off alternatively using configuration parameters instead of macros, as follows:

| - TMAmvp | : | 0 |
| --- | --- | --- |
| - TMMrg | : | 0 |
| - GPMTM | : | 0 |
| - CIIPTM | : | 0 |
| - AML | : | 0 |
| - TmvpNmvpAML | : | 0 |
| - MVSD | : | 0 |
| - TMMMVD | : | 0 |
| - AltGPMSplitModeCode | : | 0 |
| - ARL | : | 0 |
| - OBMCTM | : | 0 |

It is reported that both settings provide identical PSNR results. The only difference in bitstream is the presence of template matching related sps flags, since the macro-off setting disables all template matching tools. In other words, these sps flags are present only in config-off setting. Reportedly, the average BD-rate is 0.0%.

- Performance results of config-off setting relative to macro-off setting.

|  | Random Access Main 10 |  |  |  |  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% | x | x | x | x | x |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 101% | x | x | x | x | x |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | x | x | x | x | x | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 99% | 99% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).