| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0135-v1 |
| --- | --- |

| Title: | EE2-1.5: Chroma intra modes derived from collocated luma blocks and neighbouring chroma blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Keming Cao, Bappaditya Ray, Vadim Seregin, Marta Karczewicz | Email: | yjchang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to replace chroma DIMD mode by adding multiple DM modes. The chroma mode list is constructed in the following order: 5 DM modes, 5 neighbouring chroma modes, derived modes by adding offset +1 or -1 to the first two intra modes in the list, and default modes. The proposed changes were implemented on top of ECM-5.0, and the experimental results are summarized as follows::

AI: -0.01 % (Y), -0.18 % (U), -0.15 % (V), 99% (EncT), 98% (DecT)

RA: -0.01 % (Y), -0.02 % (U), -0.08 % (V), 99% (EncT), 99% (DecT)

# Introduction

In ECM [1], there are 12 intra prediction modes for chroma intra coding, which are listed as follows:

- 6 cross-component linear model (CCLM) modes: LM, MMLM, LM_L, LM_T, MMLM_L, and MMLM_T.

- 6 non-CCLM modes: direct mode (DM), DIMD chroma mode, and 4 default modes

DM mode is set to be the intra prediction mode of the collocated luma block covering the center position of the current chroma block as shown in Figure 1.

(a) Luma QTBT structure               (b) Chroma QTBT structure

Figure 1. Separate block partitioning structure for luma and chroma components in I slices, where the luma CU covering the position c is used to derive DM mode.

The DIMD chroma mode derived the histogram of gradients (HoGs) from the neighboring reconstructed Y, Cb and Cr samples as shown in Figure 2, and sort HoGs in a descending order. DIMD chroma mode is set as the intra mode with the largest HoG. If the intra mode with the largest HoG is the same as DM, the DIMD chroma mode is set to be the intra mode with the 2nd largest HoG.

Figure 2. Neighbouring reconstructed samples used for DIMD derivation in chroma

# Test in EE2-1.5

In the proposal, chroma DIMD mode is replaced by using multiple DM modes and the proposed non-CCLM chroma intra mode list is constructed in the following order, where the first 9 unique modes are added into the non-CCLM chroma mode list:

- 5 DM modes in the order of C, TL, TR, BL and BR as shown in Figure 3(a).

- 5 neighbouring chroma modes from the neighboring positions shown as blocks 0, 1, 2, 3, and 4 in Figure 3(b).

- The derived chroma modes by offsetting +1 or -1 to the first two intra modes in the mode list.

- Default modes: PLANAR_IDX, VER_IDX, HOR_IDX, DC_IDX, VDIA_IDX, VER_IDX - 4, VER_IDX + 4, HOR_IDX - 4, HOR_IDX + 4.

A truncated unary code is used to code an index of one of the nine chroma intra modes. The number of RD checks is kept the same as in the current ECM. Table 1 summarizes the difference of non-CCLM mode list construction between ECM-5.0 and the proposed method.

(a) Luma QTBT structure                         (b) Chroma QTBT structure

Figure 3. MDMs derived from the co-located luma CUs corresponding to different positions in current chroma CU

Table 1. Comparison table between current ECM-5.0 and the proposed method in EE2-1.5

|  | Non-CCLM mode list | DIMD derivation in chroma block | Signaling |
| --- | --- | --- | --- |
| ECM-5.0 | DM, DIMD, 4 default modes | Y/Cb/Cr neighbouring samples for gradients, histogram, and sorting | 0 (DM), 10 (DIMD), 11 + fix-lengh coding (4 default modes) |
| EE2-1.5 | 9 modes based on the proposed chroma mode list construction | No DIMD | 0, 10, 110, …, 11111110, 11111111 for 9 mode indexes |

# Experimental results

The EE experiment is tested on top of ECM-5.0 under the common test conditions [2]. Table 2 shows the performance of EE2-1.5.

Table 2. Performance of Test 1.5

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | -0.13% | 97% | 93% | 0.00% | 0.09% | -0.13% | 100% | 100% |
| Class A2 | -0.02% | -0.13% | -0.12% | 98% | 97% | -0.03% | 0.01% | -0.01% | 99% | 98% |
| Class B | -0.01% | -0.18% | -0.18% | 100% | 101% | 0.01% | -0.21% | -0.14% | 99% | 100% |
| Class C | 0.00% | -0.19% | -0.15% | 99% | 98% | -0.01% | 0.10% | 0.00% | 98% | 98% |
| Class E | -0.04% | -0.45% | -0.14% | 100% | 99% |  |  |  |  |  |
| Overall | -0.01% | -0.18% | -0.15% | 99% | 98% | -0.01% | -0.02% | -0.08% | 99% | 99% |
| Class D | 0.02% | 0.00% | -0.21% | 100% | 100% | 0.01% | -0.22% | 0.05% | 100% | 99% |
| Class F | -0.01% | -0.06% | -0.29% | 99% | 100% | -0.03% | -0.03% | 0.04% | 98% | 98% |

# References

- M. Coban, F. Le Léannec, M. Sarwer, and J. Ström, “Algorithm description of Enhanced Compression Model 5 (ECM 5),” JVET-Z2025, Jul. 2022.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).