| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0136-v2 |
| --- | --- |

| Title: | Non-EE2: On CCCM improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Vadim Seregin, Marta Karczewicz | Email: | yjchang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In ECM, there are three types of templates for CCLM to derive the linear modes: full template, top-only template, and left-only template. This template selection scheme is extended to CCCM in this contribution. In ECM, the predictors of non-CCLM modes can be fused with the predictors of MMLM. In this contribution, MMLM fusion is replaced with multi-model CCCM fusion. On top of ECM-5.0, the experimental results are summarized as follows:

AI: -0.05 % (Y), -1.14 % (U), -1.09 % (V), 103 % (EncT), 101 % (DecT)

RA: -0.03 % (Y), -0.79 % (U), -0.9 % (V), 100 % (EncT), 98 % (DecT)

# Introduction

In ECM-5.0, there are three types of templates for CCLM to derive the linear models: full template, top-only template, and left-only template. As shown in Figure 1, full template is composed of the reconstructed samples in top-only template and the reconstructed samples in left-only template. Moreover, the predictors of non-CCLM intra chroma modes, i.e., DM, DIMD, and 4 default modes, can be fused with the predictors of MMLM.

Figure 1. Luma and chroma samples used for the derivation of linear model

Convolutional cross-component mode (CCCM) is tested in EE2-1.1a (JVET-AA0057). It uses a 7-tap filter to predict chroma samples from reconstructed luma samples in a similar spirit as done by the current CCLM modes. The CCCM uses only both left and top templates to derive the model.

# Proposed method

There are two proposed changes:

- Extend the CCLM template selection to CCCM to use left, top or top and left templates for model derivation.

- MMLM fusion is replaced with multi-model CCCM fusion. More specifically, the predictors of a non-CCLM chroma mode can be fused with the predictors of multi-model CCCM mode. The derivation of fusion weights is kept unchanged.

# Experimental results

The proposed method is tested on top of EE2-1.1a software (CCCM over ECM-5.0) under the common test conditions [2].

Table 1. Performance of the proposed method over EE2-1.1a

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -1.45% | -1.08% | 105% | 103% | -0.07% | -0.55% | -0.69% | 100% | 96% |
| Class A2 | -0.01% | -1.74% | -2.18% | 105% | 102% | -0.01% | -1.28% | -1.36% | 98% | 98% |
| Class B | -0.07% | -1.33% | -1.27% | 104% | 103% | -0.02% | -1.06% | -1.19% | 101% | 99% |
| Class C | -0.04% | -0.31% | -0.45% | 103% | 101% | -0.03% | -0.26% | -0.36% | 101% | 99% |
| Class E | -0.07% | -1.04% | -0.60% | 100% | 98% |  |  |  |  |  |
| Overall | -0.05% | -1.14% | -1.09% | 103% | 101% | -0.03% | -0.79% | -0.90% | 100% | 98% |
| Class D | -0.02% | -0.61% | -0.42% | 102% | 95% | -0.01% | -0.45% | -0.56% | 97% | 97% |
| Class F | -0.04% | -0.55% | -0.75% |  |  | -0.16% | -0.75% | -0.93% | 98% | 97% |
| Class TGM | -0.66% | -1.71% | -1.73% |  |  | -0.23% | -0.71% | -0.69% |  |  |

# Conclusion

This contribution proposes an extension on template selection for CCCM and using CCCM in chroma predictor fusion. The results show that the gain of CCCM can be further improved. It is suggested to adopt this contribution into ECM.

# References

- M. Coban, F. Le Léannec, M. Sarwer, and J. Ström, “Algorithm description of Enhanced Compression Model 5 (ECM 5),” JVET-Z2025, Jul. 2022.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).