| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0140-v2 |
| --- | --- |

| Title: | EE2-related: Self-Aware Filter Estimation for CCLM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Zhipin Deng 8910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents a self-aware filter estimation CCLM (SAFE-CCLM) method to ECM. With SAFE-CCLM, N candidate luma down-sampling filters are predefined. When SAFE-CCLM is applied, a linear model between luma and chroma component is derived in the same way as that in ECM for each candidate filter first. Second, prediction values are calculated with each linear model in a testing region including one-column left neighbouring samples and one-row above neighbouring samples. Third, a SAD cost between the reconstructed samples and their corresponding prediction values in the testing region is computed for each filter candidate. Finally, the filter candidate with the least SAD cost is selected as the down-sampling filter to perform the CCLM prediction for the current block. When CCLM is indicated to be used for a block, a SAFE-CCLM flag is signaled to further indicate whether SAFE-CCLM is applied. On top of ECM-5.0, simulation results of the proposed method are reported as below:

AI: {-0.80%, -0.03%, -0.23%, 103%, 103%}; RA:{-0.38%, 0.03%, -0.13%, 100%, 100%}.

Simulation results on TGM category are reported as below:

AI: {-3.31%, -6.73%, -6.45%, 102%, 104%}; RA:{-2.11%, -3.40%, -3.23%, 102%, 100%}.

# Introduction

In VVC and ECM, the cross component linear model (CCLM) approach is applied to predict chroma samples with corresponding luma samples. To generate a luma sample “corresponding” to a chroma sample, a fixed down-sampling filter is used for color format 4:2:0 or 4:2:2 in VVC and ECM.

Since a fixed down-sampling filter may not always be the optimal one, multiple filter CCLM (MF-CCLM) prediction is proposed [1], where several candidate down-sampling filters are predefined and the selected one is signaled. Although multiple candidate filters can bring benefits, MF-CCLM cannot afford more filters due to the overheads bits to represent the filter index.

# Proposed methods

A self-aware filter estimation CCLM (SAFE-CCLM) method is proposed to explore the advantage of multiple down-sampling filters without signaling the filter index.

With SAFE-CCLM, N candidate luma down-sampling filters are predefined. As shown in Fig.1, the down-sampling filtering process for 4:2:0 color format is performed as

,

where RecL(x, y) and Rec’L(x, y) are reconstructed luma samples before and after the down-sampling filtering at position (x, y), and {c0, c1, c2, c3, c4, c5} are filtering coefficients. It should be noted that {c0, c1, c2, c3, c4, c5} is set to {1, 2, 1, 1, 2, 1} in the current ECM. N candidate luma sampling filter with different coefficients are predefined by SAFE-CCLM.

Fig.2 shows a flowchart of SAFE-CCLM. First, when SAFE-CCLM is applied, a linear model between luma and chroma component is derived in the same way as that in ECM for each candidate filter. Second, prediction values are calculated with each linear model in a testing region including one-column left neighbouring samples and one-row above neighbouring samples. Third, a SAD cost between the reconstructed chroma samples and their corresponding prediction values in the testing region is computed for each filter candidate. Finally, the filter candidate with the least SAD cost is selected as the down-sampling filter to perform the CCLM prediction for the current block.

When CCLM is indicated to be used for a block, a SAFE-CCLM flag is signaled to further indicate whether SAFE-CCLM is applied.

Fig. 1 Luma sample down-sampling.

Fig. 2 A flowchart of SAFE-CCLM.

In this contribution, N = 32, and the filter candidate set is tabulated in Table 1.

Table 1: Filter candidate set when N = 32

| F0 | F1 | F2 | F3 | F4 | F5 | F6 | F7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 14011011 | 12011000 | 12001001 | 12000011 | 1201-101-1 | 14121-1-2-1 | 120100-10 | 1200100-1 |
| F8 | F9 | F10 | F11 | F12 | F13 | F14 | F15 |
| 010000 | 001000 | 000010 | 000001 | 100000 | 000100 | 14121000 | 14000121 |
| F16 | F17 | F18 | F19 | F20 | F21 | F22 | F23 |
| 12-110-110 | 12-101-101 | 1201-101-1 | 14363-1-2-1 | 1201000-1 | 120010-10 | 12010-100 | 121000-10 |
| F24 | F25 | F26 | F27 | F28 | F29 | F30 | F31 |
| -12-1-12-1 | 001-100 | 10000-1 | 14 101101 | 1101-10 | 0110-11 | 101010 | 010101 |

# Experimental results

The proposed method is implemented on top of ECM-5.0 [2] and tested following the common test conditions [3]. The simulation results are summarized in Table 2.

Table 2: Coding performance

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | 0.51% | 0.13% | 103% | 104% | -0.22% | 0.19% | -0.07% | 102% | 102% |
| Class A2 | -4.41% | -0.48% | -3.20% | 103% | 106% | -1.61% | 0.26% | -1.54% | 101% | 100% |
| Class B | -0.11% | 0.13% | 0.87% | 102% | 104% | -0.05% | -0.21% | 0.53% | 100% | 99% |
| Class C | 0.02% | 0.06% | 0.23% | 103% | 100% | 0.01% | 0.06% | 0.07% | 99% | 98% |
| Class E | -0.05% | -0.49% | -0.05% | 103% | 102% |  |  |  |  |  |
| Overall | -0.80% | -0.03% | -0.23% | 103% | 103% | -0.38% | 0.03% | -0.13% | 100% | 100% |
| Class D | 0.06% | 0.22% | 0.10% | 103% | 99% | 0.03% | 0.17% | 0.25% | 99% | 99% |
| Class F | -0.61% | -2.05% | -1.66% | 101% | 102% | -1.03% | -2.20% | -1.83% | 99% | 99% |
| Class TGM | -3.31% | -6.73% | -6.45% | 102% | 104% | -2.11% | -3.40% | -3.23% | 102% | 100% |

# Conclusions

This contribution presents a method of SAFE-CCLM. The proposed method contributes promising coding gains with acceptable computational complexity. Considering a good trade-off between coding performance and complexity, it is recommended to adopt SAFE-CCLM into ECM.

# References

- K. Zhang, J. Chen, L. Zhang, X. Li and M. Karczewicz, "Enhanced Cross-Component Linear Model for Chroma Intra-Prediction in Video Coding," in IEEE Transactions on Image Processing, vol. 27, no. 8, pp. 3983-3997, Aug. 2018, doi: 10.1109/TIP.2018.2830640.

- ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-5.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).