| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0145-v1 |
| --- | --- |

| Title: | AHG9: On decoupling neural-network post-filter activation SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Boon Teo Jingying Gao Chong Soon Lim Kiyofumi Abe Virginie Drugeon | Tel: Email: | +65-6550-5417 hanboon.teo@sg.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution proposes to decouple the neural-network post-filter activation SEI message in the working draft of VSEI amendment from the current picture by introducing a picture identifier to the SEI message. Two variants of the solution are proposed. Three implementations of the solution are suggested.

# Introduction

Neural-network post-filter characteristics SEI message is adopted into versatile supplemental enhancement information messages for coded video bitstreams (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7) in JVET-Z2006 [1]. The amendment intends to facilitate the neural network-based post-processing filtering step by indicating the activation of a particular neural network for current picture. Each post-processing filter is identified using nnpfc_id, whose value is in the range of 0 to 232 − 2, inclusive. Furthermore, nnpfc_mode_idc equal to 1 specifies that the post-processing filter associated with the nnpfc_id value is a neural network represented by the ISO/IEC 15938-17 bitstream (Multimedia content description interface – Part 17: Compression of neural networks for multimedia content description and analysis, also known as Neural Network Representation, NNR) contained in the SEI message.

# Problem Statement

The neural-network post-filter activation SEI message is tied to current picture. In some application scenarios, the NNR bitstream (in nn_post_filter_characteristics SEI messages) may be stored in different areas instead of the video bitstream layer due to its large size. When nnpfc_id is changed for current picture, this may result in a critical timing delay issue when new NNR bitstream is to be fetched and loaded for current picture filtering.

For example, in file format case the NNR bitstream maybe stored at the beginning of a file and placed in a location different from the video data.

In another example, for streaming protocols such as MPEG-DASH, the NNR bitstream may be stored in the common segment of the server (e.g. the DASH initialization segment) that is different from the video data segments.

In such cases, the decoder has limited amount of time to get the neural-network ready for filtering current picture. It is preferable that the decoder has advance notice of the use of neural-network to prefetch and load the new neural-network parameters (in NNR bitstream) before filtering of the target picture.

# Solution

The proposed solution is to add a picture identifier to the neural-network post-filter activation SEI message.

## Proposed text changes variant 1

Supports one SEI message to activation of one picture. Multiple SEI messages are needed for activation of multiple pictures.

[Ed.: Add the highlighted parts to subclauses 8.29.1 and 8.29.2 in [1]]

8.29.1	Neural-network post-filter activation SEI message syntax

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_id | ue(v) |
| nnpfa_pics_in_output_order | ue(v) |
| } |  |

8.29.2	Neural-network post-filter activation SEI message syntax

This SEI message specifies the neural-network post-processing filter that may be used for post-processing filtering for the current picture target picture.

The neural-network post-processing filter activation SEI message persists only for the current picture target picture.

NOTE – There may be several neural-network post-processing filter activation SEI messages present for the same picture, for example, when the post-processing filters are meant for different purposes or filter different colour components.

nnpfa_id specifies that the neural-network post-processing filter specified by one or more neural-network post-processing filter characteristics SEI messages that pertain to the current picture target picture and have nnpfc_id equal to nnfpa_id may be used for post-processing filtering for the current picture target picture.

nnpfa_pics_in_output_order specifies the number of picture, in output order, from the picture containing this SEI message, until the target picture that uses the neural-network post-processing filter specified by one or more neural-network post-processing filter characteristics SEI messages that have nnpfc_id equal to nnpfa_id.

## Proposed text changes variant 2

Supports one SEI message to activation of multiple pictures.

[Ed.: Add the highlighted parts to subclauses 8.29.1 and 8.29.2 in [1]]

8.29.1	Neural-network post-filter activation SEI message syntax

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_cnt_minus1 | ue(v) |
| for( i = 0; nnpfa_cnt_minus1+1; i++ ) { |  |
| nnpfa_id[ i ] | ue(v) |
| nnpfa_pics_in_output_order[ i ] | ue(v) |
| } |  |
| } |  |

8.29.2	Neural-network post-filter activation SEI message syntax

This SEI message specifies the neural-network post-processing filter that may be used for post-processing filtering for the current picture target picture.

The neural-network post-processing filter activation SEI message persists only for the current picture target picture.

NOTE – There may be several neural-network post-processing filter activation SEI messages present for the same picture, for example, when the post-processing filters are meant for different purposes or filter different colour components.

nnpfa_cnt_minus1 plus 1 specifies the number of nnpfa_id and nnpfa_delta_poc that may be present in the SEI message.

nnpfa_id[ i ] specifies that the neural-network post-processing filter specified by one or more neural-network post-processing filter characteristics SEI messages that pertain to the current picture target picture and have nnpfc_id equal to nnfpa_id may be used for post-processing filtering for the current picture target picture. nnpfa_pics_in_output_order[ i ] specifies the number of picture, in output order, from the picture containing this SEI message, until the target picture that uses the neural-network post-processing filter specified by one or more neural-network post-processing filter characteristics SEI messages that have nnpfc_id equal to nnpfa_id[ i ].

## Proposed text changes common to variants 1 and 2

[Ed.: In clauses 8.1, Table 4, replace the row for Neural-network post-filter activation with the highlighted parts]

| SEI message | Persistence scope |
| --- | --- |
| … | … |
| Neural-network post-filter activation | Specified by the syntax of the SEI message |

# Implementations

There are a few options to implement the solution.

## Option 1

Signal neural-network post-filter activation SEI message ahead at every picture. Only the first picture cannot be filtered by neural-network post-processing filter as illustrated in Figure 1. It is suggested to use variant 1 with this option.

Figure 1

## Option 2

Signal neural-network post-filter activation SEI message ahead at every picture, including current picture as illustrated in Figure 2. Variant 1 or 2 can be used with this option.

Figure 2

## Option 3

Signal neural-network post-filter activation SEI message only at IRAP picture as illustrated in Figure 3. It is suggested to use variant 2 with this option.

Figure 3

# Conclusion

This proposal highlights the possible application scenarios and challenges for implementing the neural-network post-processing filter. It is suggested to decouple the neural-network post-filter activation SEI message from current picture by introducing a picture identifier to the SEI message. Two variants of the solution are proposed. Three possible implementations of the solution are suggested. It is suggested to adopt either variant 1 or variant 2 of the proposal.

# References

- S. McCarthy, T. Chujoh, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang, “Additional SEI messages for VSEI (Draft 1)”, JVET-Z2006, 26th Meeting, by teleconference, 20–29 April 2022

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).