| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 |  |  |  | Document: JVET- AA0222 |
| --- | --- | --- | --- | --- |
| Title: | Motion Vector limits for AVC/H.264 levels 6.x |  |  |  |  |
| Status: | Input document of JVET |  |  |  |  |
| Purpose: | Proposal |  |  |  |  |
| Author(s) or Contact(s): | A. Tourapis | Email: | atourapis@apple.com |  |  |
| Source: | Apple Inc |  |  |  |  |

# Abstract

The 24th version of the ITU-T Rec. H.264 | ISO/IEC 14496-10 document (11th edition of the H.264 specification) introduced, among other things, several additional levels for decoder capability, i.e., levels 6, 6.1, and 6.2, mostly to support larger picture sizes and higher frame rates. One of the introduced changes to support such functionality included the increase of the motion vector range limits that can be supported compared to lower levels. In particular, the vertical motion vector component limit MaxVmvR was increased from 512 to 8192, while the horizontal motion vector range for luma motion vectors was changed from [−2048, 2047.75] to [−8192, 8191.75]. This change was seen as a potentially desired change so as to better handle larger resolution images that may be characterized by higher range motion and to better align with more modern coding specifications such as HEVC and VVC. However, during the introduction of those levels, no concerns were raised about existing HW implementations.

It was recently drawn to our attention that there exist several HW AVC decoder implementations that although can support the resolutions that correspond to the 6.x levels, cannot unfortunately claim complete conformance to these levels because of this difference in motion vector range. This is because such implementations are mostly based on earlier AVC decoder designs that were limited to 5.x levels. Ideally, for properly supporting these new levels a low level change in the design of these decoders is necessary given the higher bitdepth precision needed for the new motion vector range. Such change is, unfortunately, seen as costly and undesirable. suporting. In some cases this would mean a complete replacement of existing HW devices that may have such capabilities.

This document requests an “amendment” to the capabilities of these levels so as to properly enable such decoders. A few possible solutions are suggested.

# Introduction

It was recently identified to us that several HW AVC decoder implementations that could theoretically support level 6.x decoding were incapable or limited in doing so because of the increased motion vector range requirements expected for these decoders compared to decoders of lower capabilities. It is highly desirable that such decoders were properly certified as being able to decode video resolutions that correspond to such levels without, however, having to conform to the currently specified motion vector range. We should highlight that we are not aware of any HW decoder that is currently capable of handing such motion vector ranges, at least in consumer applications. However, it is quite possible that such may exist. Furthermore, video bitstreams could be encoded while including bitstream restrictions on the motion vector range, i.e., by setting log2_max_mv_length_horizontal and log2_max_mv_length_vertical to 11 and 9, respectively. That would result in valid decoding for such bitstreams on such decoders, but there is currently no guarantee that bitstreams that exceed these ranges could no be encountered that may result in some form of decoding corruption/failure.

In the next section a few methods are proposed that could allow these decoders to claim some form of conformance.

# Proposed methods

The following solutions to this issue were identified.

- Update the motion vector range of the existing levels 6.x to reflect the desired change regardless of profile. 
We understand that this solution may not be desirable especially if there are bitstreams already encoded using these levels.

- Update the motion vector range of the existing levels 6.x of certain profiles to reflect the desired change. This would impact the following profiles:

  - Constrained Baseline Profile

  - Baseline Profile

  - Extended Profile

  - Main Profile

  - High Profile

  - Progressive High Profile

  - Constrained High Profile

  - Multiview High Profile

Given that this is limited around certain profiles, this likely limits the impact of invalidating some existing bitstreams, but does not eliminate this probability.

- Specify that the profiles in (b) need to also indicate explicitly the log2_max_mv_length_horizontal and log2_max_mv_length_vertical values and be constrained with values 11 and 9, respectively. This does not necessarily invalidate any previously generated bitstreams although these are currently not exactly conforming to any particular level. However, one could argue that these bitstreams could be acknowledged by such devices and could be decoded in a best effort decoding mode.

- Define new levels that would have the same capabilities as levels 6.x other than the motion vector range. This could be done by either introducing new level numbers or augmenting the existing level numbers, i.e., creating new levels 6b, 6.1b, and 6.2b, with a constraint set flag or by imposing the presence of the mv limits in VUI.

Our preference would be to consider option d, preferably with a constraint flag, and if that is not possible option c. In either case, we would afterwards encourage encoder manufacturers to utilize these new levels when encoding high resolution bitstreams that require levels beyond 5.x to maximize decoder support.

# Patent rights declaration(s)

Apple Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).