| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0229-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AA0089 (EE1-related: More refinements on EE1-1.4 and EE1-1.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Du Liu | Email: | du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for JVET-AA0089, which presents a neural network-based in-loop filter. Two crosscheck tests were carried out, one for testing two NN models and one for testing one NN model. Both tests include Class C and Class D under AI and RA configurations. It is reported that the crosscheck results match exactly with the results in JVET-AA0089.

# Introduction

The contribution JVET-AA0089 proposes to use one or two models from EE1-1.4 [2] and EE1-1.5 [3] for in-loop filtering and enables an encoder optimization tool [4]. There are six tests in JVET-AA0089, and the proponents requested to crosscheck two of them under AI and RA configurations for Class C and Class D:

Test 1: Test 3.3 in JVET-AA0089: Filter 1, int16, two models, new models, EncDbOpt

Test 2: Test 3.6 in JVET-AA0089: Filter 2, int16, one model, new model, EncDbOpt

Libtorch v1.9.0+cpu was used.

# Results

The simulation follows NNVC-CTC [5]. The results are shown in the following tables.

## Test of two models (Test 3.3 in JVET-AA0089)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -9.41% | -23.50% | -23.52% | 155% | 31314% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -10.21% | -22.41% | -23.61% | 156% | 32909% |
| Class F |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -7.63% | -17.60% | -20.27% | 157% | 13567% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -7.46% | -16.68% | -19.80% | 154% | 14531% |
| Class F |  |  |  |  |  |

## Test of one model (Test 3.6 in JVET-AA0089)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -9.21% | -20.92% | -22.34% | 164% | 36012% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -10.23% | -20.20% | -21.60% | 165% | 38228% |
| Class F |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -6.42% | -15.91% | -18.54% | 162% | 15197% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -6.35% | -14.04% | -17.46% | 163% | 16252% |
| Class F |  |  |  |  |  |

# References

[1] L. Wang, S. Lin, X. Xu, S. Liu, Z. Xie, Y. Yu, H. Yu, D. Wang, “EE1-related: More refinements on EE1-1.4 and EE1-1.5”, JVET-AA0089, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 27th Meeting, by teleconference, 13-22 July 2022

[2] L. Wang, S. Lin, X. Xu, S. Liu, F. Galpin, “EE1-1.4: neural network based in-loop filter with 2 models”, JVET-AA0087, 27th Meeting, by teleconference, 13–22 July 2022.

[3] L. Wang, S. Lin, X. Xu, S. Liu, F. Galpin, “EE1-1.5: neural network based in-loop filter with a single model”, JVET-AA0088, 27th Meeting, by teleconference, 13–22 July 2022.

[4] N. Hu, V. Seregin, W.-J. Chien, M. Karczewicz, “Encoder optimization with deblocking filter”, JVET-M0428, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

[5] E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common Test Conditions and evaluation procedures for neural network-based video coding technology”, JVET-X2016, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 24th Meeting, by teleconference, 6–15 October 2022