| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET- AA0234-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AA0137 (Non-EE2: Intra Prediction Fusion) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Luhang Xu | Email: | xuluhang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-AA0137 Non-EE2: Intra Prediction Fusion. It is reported that the crosscheck results exactly match with the results provided by the proponent.

# Introduction

The contribution JVET-AA0137 proposes to derive intra prediction by weighting intra predictions obtained from multiple reference lines represented as pfusion=w0pline+w1pline+1, where pline is the intra prediction from the default reference line and pline+1 is the prediction from the line above the default reference line. In the experiment, the weights are set as w0=3/4 and w1=1/4. The proposed intra prediction fusion is applied to luma blocks when angular intra mode has non-integer slope and the block size is greater than 16, it is used with MRL and not applied for ISP coded blocks.

# Experimental Results

The crosscheck has been performed in All Intra and Random Access configuration. The simulations were conducted on an Intel Xeon cluster with Linux OS. So far, only partial results are available (attached to this file) and they are matched with the results provided by the proponent.

Table 1. Crosscheck results

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.09% | -0.05% | 101% | 92% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.15% | -0.14% | -0.07% | 103% | 97% |
| Class C | -0.14% | -0.11% | -0.07% | 103% | 103% |
| Class E | -0.11% | 0.06% | -0.02% | 105% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.12% | 0.06% | -0.10% | 101% | 96% |
| Class F | 0.01% | 0.01% | 0.02% | 99% | 91% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.02% | -0.11% | -0.23% | 98% | 96% |
| Class C | -0.05% | 0.00% | -0.07% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | -0.11% | 0.07% | 101% | 103% |
| Class F | 0.09% | -0.05% | 0.11% | 103% | 103% |