| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET- AA2020-v1 |
| --- | --- |

| Title: | Film grain synthesis technology for video applications (Draft 2) |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | D. Grois Y. He W. Husak P. de Lagrange A. Norkin M. Radosavljević A. Tourapis W. Wan | Email: | Dan_Grois@comcast.com yonghe@qti.qualcomm.com wjh@dolby.com philippe.delagrange@interdigital.com anorkin@netflix.com milosr@xiaomi.com atourapis@apple.com wade.wan@broadcom.com |
| Source: | Editors |  |  |

_____________________________

Summary This draft technical report provides guidance on the use of film grain synthesis technology for video applications. Such technology may be used in conjunction with metadata signalling mechanisms, such as the supplemental enhancement information messages available in several video coding standards. The purpose of this document is to provide a publicly referenceable overview of end-to-end processing steps for film grain synthesis, that may include content analysis, noise/film grain removal and film grain model parameter estimation, parameter encoding, encapsulation, and decoding, and film grain synthesis and blending for consumer distribution applications.

Keywords Film Grain, Synthesis, Analysis, Modelling, Metadata.

- Table of Contents

1	Scope	4

2	References	4

3	Definitions	5

4	Abbreviations and acronyms	5

5	Conventions	5

5.1	General	5

5.2	Arithmetic operators	5

5.3	Bit-wise operators	6

5.4	Assignment operators	6

5.5	Relational, logical and other operators	7

5.6	Mathematical functions	7

5.7	Order of operations	7

6	Overview of film grain technologies	8

6.1	General	8

6.2	Film grain	9

6.3	Film grain modelling	10

6.4	Use cases and applications	12

7	Film grain synthesis	12

7.1	General	12

7.2	General description of pattern-based film grain synthesis	13

7.2.1	General	13

7.2.2	Pattern generation	14

7.2.3	Randomization	14

7.2.4	Local adaptation	17

7.2.5	Deblocking	18

7.2.6	Blending	18

7.3	Frequency filtering model for film grain synthesis	18

7.3.1	Overview of the generalized frequency filtering model	18

7.3.2	Fixed-point example of the frequency filtering model for film grain synthesis	20

7.3.3	Fixed-point example of the frequency filtering model conforming to SMPTE RDD-5	24

7.3.4	Fixed-point example of resolution-adaptive frequency filtering model for film grain synthesis	24

7.3.5	Fixed-point example of resolution-adaptive frequency filtering model for film grain synthesis without a line buffer	25

7.4	Autoregressive models for film grain synthesis	25

7.4.1	Overview of the generalized autoregressive model	25

7.4.2	Fixed-point example of film grain synthesis supporting both frequency filtering and autoregressive models	26

7.4.3	Example of the autoregressive model for film grain synthesis	26

7.4.4	Use of an FCS SEI message with the example autoregressive model for film grain synthesis	27

8	Film grain analysis	27

8.1	General	27

8.2	Denoising and image analysis	28

8.2.1	Denoising	28

8.2.2	Edge and texture analysis	29

8.3	Frequency filtering model parameter estimation	29

8.3.1	Determination of intensity intervals and grain variance	29

8.3.2	An example of FGC SEI message scaling factor estimation	30

8.3.3	Determination of cut-off frequencies	33

8.3.4	An example of FGC SEI message cut-off frequency estimation	34

8.4	Autoregressive model parameter estimation	34

9	Film grain metadata: syntax and semantics	35

9.1	General	35

9.2	Film grain characteristics SEI message	35

9.2.1	General	35

9.2.2	Interpretation of FGC SEI message syntax	35

9.3	Alternative film grain metadata	38

9.3.1	General	38

Annex A Example implementations of film grain synthesis technologies	39

A.1	FGC SEI message example implementations	39

A.2	Film grain analysis example implementations	40

A.3	Film grain synthesis example implementations	40

Film Grain Synthesis Technology for Video Applications

[WORKING DRAFT]

# Scope

This report provides guidance on the use of film grain synthesis technology in video applications, including for use in the ITU-T H.264/AVC (MPEG-4 Part 10), ITU-T H.265/HEVC (MPEG-H Part 2), and ITU-T H.266/VVC (MPEG-I Part 3) standards. Such technology can provide subjective quality benefits for certain video applications and thus can effectively achieve improved video compression. This can be achieved by commonly removing or reducing the amount of noise that may be present in a video or image signal prior to compression, and by resynthesizing and adding back an approximation of the removed noise, given a set of signalled parameters, during decoding. Such parameters may be signalled using appropriate mechanisms, such as the supplemental enhancement information messages that are supported by several video coding standards. The purpose of this document is to provide a publicly referenceable overview of the end-to-end processing steps for film grain and sensor noise removal, estimation, parameterization, synthesis, and blending for consumer distribution applications. This document includes examples of encoder-side processing steps and post-decoding processing steps for grain blending for some of the currently defined technologies.

# References

- Rec. ITU-T H.266 | ISO/IEC 23090-3, Versatile Video Coding.

- Rec. ITU-T H.274 | ISO/IEC 23002-7, Versatile supplemental enhancement information messages for coded video bitstreams

- Rec. ITU-T H.265 | ISO/IEC 23008-2, High efficiency video coding.

- Rec. ITU-T H.264 | ISO/IEC 14496-10, Advanced video coding for generic audiovisual services.

- SMPTE RDD 5-2006, Film grain technology specifications for H.264 | MPEG-4 AVC bitstreams

- C. Gomila and A. Kobilansky, “SEI message for film grain encoding.” document JVT-H022, May 2003.

- T. Chen, "Elimination of Subband-Coding Artifacts Using the Dithering Technique", Proceeding of International Conference on Image Processing (ICIP), pp. 874-877, 1994.

- X. Jin, S. Goto and K. N. Ngan, "Composite Model-Based DC Dithering for Suppressing Contour Artifacts in Decompressed Video," in IEEE Transactions on Image Processing, vol. 20, no. 8, pp. 2110-2121, Aug. 2011, doi: 10.1109/TIP.2011.2114356.

- J. M. Boyce and A. M. Tourapis, "Comfort noise for compressed video," International Conference on Consumer Electronics (ICCE), 2005, pp. 323-324, doi: 10.1109/ICCE.2005.1429848.

- Enhorn, J., Sjöberg, R., & Wennersten, P. (2020, October). A temporal pre-filter for video coding based on bilateral filtering. In 2020 IEEE International Conference on Image Processing (ICIP) (pp. 1161-1165). IEEE.

- Dabov, K., Foi, A., Katkovnik, V., & Egiazarian, K. (2007). Image denoising by sparse 3-D transform-domain collaborative filtering. IEEE Transactions on image processing, 16(8), 2080-2095.

- Ameur, Z., Hamidouche, W., François, E., Radosavljević, M., Menard, D. and Demarty, C.H., 2022. Deep-based Film Grain Removal and Synthesis. arXiv preprint arXiv:2206.07411.

- Brooks, T., Mildenhall, B., Xue, T., Chen, J., Sharlet, D., & Barron, J. T. (2019). Unprocessing images for learned raw denoising. In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition (pp. 11036-11045).

- Tian, C., Fei, L., Zheng, W., Xu, Y., Zuo, W., & Lin, C. W. (2020). Deep learning on image denoising: An overview. Neural Networks, 131, 251-275.

- Cheong, H. Y., Tourapis, A. M., Llach, J., & Boyce, J. (2004, October). Adaptive spatio-temporal filtering for video denoising. In 2004 International Conference on Image Processing, 2004. ICIP'04. (Vol. 2, pp. 965-968). IEEE.

- Kokaram, A., Kelly, D., Denman, H., & Crawford, A. (2012, September). Measuring noise correlation for improved video denoising. In 2012 19th IEEE International Conference on Image Processing (pp. 1201-1204). IEEE.

[Ed. Note: Any additional IEEE, JVT or other references to be added here.]

# Definitions

For the purpose of this document, the definitions given in [1] [2] and [3] apply.

# Abbreviations and acronyms

For the purpose of this document, the abbreviations and acronyms given in [1] [2] [3] and the following apply.

FGS		Film Grain Synthesis

FGC		Film Grain Characteristics

SEI		Supplemental Enhancement Information

# Conventions

## General

The mathematical operators used in this document are similar to those used in the C programming language. However, the results of integer division and arithmetic shift operations are defined more precisely and additional operations are defined, such as exponentiation and real-valued division. Numbering and counting conventions generally begin from 0, e.g., "the first" is equivalent to the 0-th, "the second" is equivalent to the 1-th, etc.

## Arithmetic operators

The following arithmetic operators are defined as follows:

|  | Addition |
| --- | --- |
| − | Subtraction (as a two-argument operator) or negation (as a unary prefix operator) |
| * | Multiplication, including matrix multiplication |
| xy | Exponentiation. Denotes x to the power of y. In other contexts, such notation is used for superscripting not intended for interpretation as exponentiation. |
| / | Integer division with truncation of the result towards zero. For example, 7 / 4 and (−7) / (−4) are truncated to 1 and (−7) / 4 and 7 / (−4) are truncated to −1. |
| ÷ | Used to denote division in mathematical formulae where no truncation or rounding is intended. |
| xy | Used to denote division in mathematical formulae where no truncation or rounding is intended. |
| i=xyf(i) | The summation of f( i ) with i taking all integer values from x up to and including y. |
| x % y | Modulus. Remainder of x divided by y, defined only for integers x and y with x >= 0 and y > 0. |

## Bit-wise operators

The following bit-wise operators are defined as follows:

| & | Bit-wise "and". When operating on integer arguments, operates on a two's complement representation of the integer value. When operating on a binary argument that contains fewer bits than another argument, the shorter argument is extended by adding more significant bits equal to 0. |
| --- | --- |
| ¦ | Bit-wise "or". When operating on integer arguments, operates on a two's complement representation of the integer value. When operating on a binary argument that contains fewer bits than another argument, the shorter argument is extended by adding more significant bits equal to 0. |
| ^ | Bit-wise "exclusive or". When operating on integer arguments, operates on a two's complement representation of the integer value. When operating on a binary argument that contains fewer bits than another argument, the shorter argument is extended by adding more significant bits equal to 0. |
| x >> y | Arithmetic right shift of a two's complement integer representation of x by y binary digits. This function is defined only for non-negative integer values of y. Bits shifted into the MSBs as a result of the right shift have a value equal to the MSB of x prior to the shift operation. |
| x << y | Arithmetic left shift of a two's complement integer representation of x by y binary digits. This function is defined only for non-negative integer values of y. Bits shifted into the LSBs as a result of the left shift have a value equal to 0. |

## Assignment operators

The following assignment operators are defined as follows:

| = | Assignment operator |
| --- | --- |
| ++ | Increment, i.e., x  is equivalent to x  x  1; when used in an array index, evaluates to the value of the variable prior to the increment operation. |
| − − | Decrement, i.e., x− − is equivalent to x  x − 1; when used in an array index, evaluates to the value of the variable prior to the decrement operation. |
| += | Increment by amount given, i.e., x += 3 is equivalent to x = x + 3, and x += (−3) is equivalent to x = x + (−3). |
| −= | Decrement by amount given, i.e., x −= 3 is equivalent to x = x − 3, and x −= (−3) is equivalent to x = x − (−3). |

## Relational, logical and other operators

The following operators are defined as follows:

| == | Equality operator |
| --- | --- |
| != | Not equal to operator |
| !x | Logical negation "not" |
| > | Larger than operator |
| < | Smaller than operator |
| ≥ | Larger than or equal to operator |
| ≤ | Smaller than or equal to operator |
| && | Conditional/logical "and" operator. Performs a logical "and" of its Boolean operators, but only evaluates the second operand if necessary. |
| ¦ ¦ | Conditional/logical "or" operator. Performs a logical "or" of its Boolean operators, but only evaluates the second operand if necessary. |
| a ? b : c | Ternary conditional. If condition a is true, then the result is equal to b; otherwise the result is equal to c. |

## Mathematical functions

The following mathematical functions are defined as follows:

Absx=x;x≥0-x;x<0

Ceilx	the smallest integer greater than or equal to x.

Clip3( x, y, z ) =  x;z<xy;z>yz;otherwise

Minx,y= x;x<yy;otherwise

## Order of operations

When order of precedence in an expression is not indicated explicitly by use of parentheses, the following rules apply:

–	Operations of a higher precedence are evaluated before any operation of a lower precedence.

–	Operations of the same precedence are evaluated sequentially from left to right.

Table 1 specifies the precedence of operations from highest to lowest; a higher position in the table indicates a higher precedence.

NOTE – For those operators that are also used in the C programming language, the order of precedence used in this document is the same as that used in the C programming language.

Table 1 – Operation precedence from highest (at top of table) to lowest (at bottom of table)

Operations (with operands x, y and z)

"x++", "x− −"

"!x", "−x" (as a unary prefix operator)

"xy"

"x * y", "x / y", "x  y"" xy xy ", "x % y"

"x + y", "x − y" (as a two-argument operator), " i=xyf(i) i=xyf(i) "

"x << y", "x >> y"

"x < y", "x <= y", "x > y", "x >= y"

"x = = y", "x != y"

"x & y"

"x ¦ y"

"x && y"

"x ¦ ¦ y"

"x ? y : z"

"x..y"

"x = y", "x += y", "x −= y"

# Overview of film grain technologies

## General

This section provides an overview of film grain technologies, including historical information on the development of such technologies (Sub-clause 6.2), a high level description of film grain modeling (6.3), and information on use cases and applications for such technologies (6.4). [Ed. Note: The earlier text included a circular reference and needs to be addressed.] More details are provided in subsequent clauses as follows:

- Clause 7 describes some of the already defined film grain synthesis technologies, and in particular the frequency filtering and autoregressive models.

- Clause 8 provides examples of technologies that can be used for film grain analysis, including techniques for video denoising, edge and complex texture detection, film grain characteristic analysis, and model parameters estimation.

- Clause 9 describes some of the film grain metadata that have already been defined in current video coding standards and specifications and how each metadata is interpreted, if appropriate, in the context of the frequency filtering and autoregressive models. Example implementations of such technologies are then described in Annex A.

## Film grain

Film grain is considered a primary contributor to what is commonly referred to as the film look or cinematic look. [Ed. Note: Maybe odd way to start this section. Why not maybe start talking about movies and video capture first and say that it has gone though many advancements, from film to digital, then mention that for film one of the characteristics of film was a form of noise, that is referred to as film grain, and then discuss its impact. Also then mention digital movies and the addition of noise. It seems that this introduction starts somewhat in the opposite way, which is not maybe the best.] Motion picture film consists of silver-halide crystals dispersed in photographic emulsion. Images are then formed by exposure and the development of these crystals. In color images, where the silver is chemically removed after development, dye clouds (like soft tiny colored grains) are formed on the sites where the silver crystals have been exposed. Grains are randomly distributed in the resulting image because of the random formation of silver crystals in the original emulsion. The naked eye cannot distinguish individual grains, which are about 0.002 mm down to about a tenth of that in size. Instead, the eye resolves groups of grains in an image, that an observer identifies as a grainy look that is commonly called just “film grain”. This is illustrated in Figure 1. Another example is shown in Figure 2, with a different color image formation process called “autochrome”. This is one of the first color image techniques invented by Lumiere brothers, where a classical black and white photo emulsion is exposed through a color filter made of a fine dust of potato starch dyed with different colors. [Ed. Note: Need a statement on the fact that film grain is not temporally correlated here]

The larger the image resolution, the higher the perception of the film grain. Film grain is clearly noticeable in cinema and HD images; it progressively loses importance in SDTV and becomes imperceptible in smaller formats [6].

10 μm

10 μm

1 mm

1 mm

Figure 1: Fuji S 400 film (left: 4000 dpi scan of 2x2mm area; right: raw negative 500x microscope view of 0.1x0.1mm area)

Figure 2: 24x24mm and 2.2x2.2mm crops of a 1916 autochrome picture (13x18cm glass plate) from A. Samama, courtesy of Ministère de la Culture / RMN-GP (France)

Perception of moderate grain texture is a desirable, often sought after, feature in motion picture. Although the exact effect of the grain is not clear, it is considered a requirement in motion picture industry to preserve the grainy appearance of images throughout the image processing and delivery chain. Intuitively, the presence of film grain helps to differentiate 'real-world' images from 'computer-generated' material with no film grain. Furthermore, it is possible that film grain provides some visual cues that facilitate the correct perception of depth in two-dimensional pictures [6]. Even when movies are captured with digital cameras, artificial film grain is added at the post-processing stage to create a specific look, that artists qualify as “soft”, “organic”, “living”.

Film grain preservation during video distribution can be a challenge because of two main reasons. First, the compression gains related to temporal prediction cannot be fully exploited because of the random nature of the grain. [Ed. Note: the statement about temporal prediction was too strong; there are ways you can still exploit temporal prediction without removing noise at the end. Btw, this is also not an issue for high end applications, like for digital cinema or archiving, so this should be stated. This is clearly for consumer applications that require strong compression]. Second, the grain appears at high frequencies in the DCT domain, and it is typically filtered with other noise by in-loop deblocking filters or for a wide range of QP values during quantization [6]. This challenge is more severe with recent codecs, as bitrates gains have come along with noise elimination. In addition, introduction of pre-filtering in the video distribution chain can potentially remove film grain prior to compression. [Ed. Note: Some statements are needed here about quantization matrices. That is a technique that permits the preservation of the noise up to certain bitrate better and was a conclusion also during AVC days. It seems not appropriate not saying anything about that. Note also that the significance of FGS increased due to adaptive streaming applications and the need to achieve lower bitrates. Applications such as Blu-ray can operate at higher bitrates and for those having the “exact” noise is maybe important and it is less of an issue of not using FGS. However, for broadcast and adaptive streaming applications that is no more true. Potentially showing a RD curve and images of how noise deteriorates with increased compression may be of interest here. ]

## Film grain modelling

[Ed. Note: This does not seem to provide a good flow to the document. I think it is better to first discuss what the intent is and then talk about film grain modelling. For example, say that although some compression can be achieved by coding a video sequence directly, it was determined that filtering the content, compressing, and providing information that enables the regeneration of the film grain, evem if that is just an approximation, can result in more efficient coding performance and a better visual outcome. This is called film grain modelling.]  Film grain modelling technologies provide a means of optionally removing noise prior to or during the encoding process in order to improve compression efficiency and, subsequently, to reconstruct an approximation of the film grain during or after the decoding process. Film grain modelling technologies may also be used to add visual noise to decoded video to mask or attenuate the visibility of compression artefacts, such as blockiness, or bit-depth-related artefacts, such as banding or contouring. [Ed. Note: This should be tied to other dithering methods presented in the past as well as dithering used on TVs or monitors to deal with reduced bit depth.]

Use of light-dependent film grain model parameters may be useful, particularly for simulation of photographic film grain, as photographic film grain is intrinsically intensity dependent. First, variation of film opacity is the result of a variation of grain density, as seen in Figure 1, which has an impact on perceived grain. Also, film is organized in several layers (usually 3) for each color component, with various light sensitivities to reach to its full dynamic range. Light-sensitive crystals have a distribution of sizes. Larger crystals capture more photons and are more likely to be exposed than smaller crystals, particularly in darker region. As a result, darker regions tend to have larger and more noticeable film grain. This characteristic results in a frequency dependence of the noise with brightness. An example with both grain size and strength variation is shown in Figure 3.

Figure 3: Kodak Vision 250D film, 1500 dpi scan of 5.5x5.5mm areas of the same picture

Use of film grain modelling technologies to denoise source video by using a preprocessor is illustrated in Figure 4. Source video is input to a denoising process that outputs a video sequence from which noise or film grain is attenuated or removed. A film grain parameterization process then compares the source and denoised videos to determine film grain model parameter values, which relate to the variance, spatial frequency characteristics, colour correlation, and other statistical characteristics of the film grain. The process of denoising followed by the film grain model parameter estimation is commonly referred to as the film grain analysis process. Such process will be further discussed in Clause 8.

After these processes are performed, the denoised video is then encoded and the film grain model parameter values are either signalled in the coded bit stream or provided to the decoder by some external means.

Figure 4: Use of film grain modelling technologies with a denoising pre-processing.

[Ed. Note : Update figure. The top block could be moved to the center resulting in a straight line from the denoiser output. ]

An alternative use of film grain modelling technologies to denoise source video is also illustrated in Figure 5; but, in this case, the encoder itself acts as the denoising process. . [Ed. Note : Is this an alternative use or an alternative implementation ? ]

Figure 5: Use of film grain modelling technologies with an encoder to denoise source video.

The use of film grain modelling technologies with pre-determined film grain parameter values is illustrated in Figure 6. Note that film grain parameter values may, for example, have been pre-determined during post-production or when the statistical characteristics of the film grain are otherwise known a priori. The film grain parameters could be adjusted by the encoder depending on coding parameters, or a default film grain configuration could be selected to mask coding artifacts.

Figure 6: Use of film grain modelling technologies with pre-determined film grain model parameter values.

Figure 7 presents a decoding process, along with the film grain synthesis post-processing, for each of the examples provided in Figure 4 to Figure 6. At the decoder, the film grain model parameter values are parsed and input to a film grain synthesis process that generates simulated film grain and blends the grain with the decoded video to output decoded video with simulated film grain.

Figure 7: Decoding process along with the film grain synthesis post-processing.

## Use cases and applications

[Ed. Note: Should this section be above 6.3? It also seems to repeat some concepts

The use of film grain modelling technologies can be beneficial in the context of image and video compression by providing improved subjective quality at a lower bitrate for certain types of video content. For example, these technologies can potentially provide benefits to video content that contains noise, such as film grain noise or image sensor noise. In some cases, film grain noise is a result of the physical process of photographically developing light-exposed silver-halide crystals in the emulsion on the film negative used in the capture process. In other cases, noise may be synthetic film grain that was added in post-production to create a particular artistic look. In yet other cases, noise in digitally captured content may be the result of image sensor noise. In each case, film grain and sensor noise tend to be temporally and spatially unpredictable, which significantly impairs the compressibility of video material containing such noise.

The first use case of film grain synthesis is artistic intent: recreate the film grain at the decoder side, that was unavoidably lost by compression involved in content distribution at practical bitrates. In this case, the film grain is considered to be a significant aspect of the video, and the content provider wants it to be part of the user experience.

Preserving film grain through video compression would require unacceptably high bit rates, while removing it allows using the full potential of video compression while requiring film grain synthesis after decoding. [Ed. Note: what is defined here as unacceptable? Blu-ray, for example, still does it while even some grain is maintained for broadcast/cable applications. This seems needing some toning down]

The second use case, which can also complement the first one, is the masking of compression impairments, like blocking, banding, mosquito noise, etc. If there is no artistic intent, then the constraints on film grain model accuracy may be relaxed. For this use case, the encoder can adjust film grain parameters to fit coding parameters, so that defect masking is effective (e.g., adjust noise amplitude based on quantization step). [Ed. Note: Dithering has been shown to provide subjective benefits on compressed signals. Here, this is a form of “guided” dithering, that is controlled by the encoder or some intermediate dithering ingestion stage that analyzes the content and could exploit existing HW implementations to achieve a consistent dithering look instead of relying on differing manufacturer implementations of similar but blind dithering technologies]

# Film grain synthesis

## General

Film grain synthesis [Ed Note: considered here? methods described in this document?] is commonly based on gaussian noise generation, with spatial correlation modeled either by frequency limits or autoregressive parameters, and local adaptation to target image intensity levels that consists of adjusting grain amplitude, and potentially also spatial correlation.

Gaussian noise can be generated with a random generator and a gaussian distribution table. The gaussian noise generator can be run for every pixel in the picture, along with spatial correlation methods, or can be run for a limited area (e.g., 64x64 pixels), further called as the “pattern”, which is then randomized to generate the full picture. Several such patterns may need to be generated when spatial correlation varies across intensity intervals.

Pattern randomization (i.e., extension to the full picture) can be performed by dividing the picture into blocks significantly smaller than the pattern (e.g., 16x16 compared to 64x64 pattern), and by choosing a pseudo-random offset within the pattern space for each of those smaller blocks. A pseudo-random sign inversion can be added to the random offset to improve randomization. When such process is performed, deblocking is needed across randomization blocks, especially when spatial correlation within the pattern is significant (in other words: when the grain is large).

Working with patterns avoids running the gaussian generator and correlation process for the full picture, which can be costly, not only because of more computations, but also because storing neighboring lines (for spatial correlation) can be problematic in hardware. In contrast, using random offsets does not involve line storage, but just reading pre-computed patterns at specific locations.

Local adaptation may be based on pixel intensity or a local average: a scaling factor and a specific pattern is selected depending on intensity. Using a local (sub-block) average requires deblocking on sub-block borders and involves hardware cost by requiring storage of neighboring lines. On the other hand, using pixel-based pattern adaptation requires patterns to be correlated with each other (e.g., based on the same underlying pseudo-random pattern and incrementally low-pass filtered) so that transition from one pattern to another does not break spatial correlation.

## General description of pattern-based film grain synthesis

### General

[Ed. Note (PL): this section is inspired from JVET-Z0115; addresses both FF and AR methods and should mostly replace 7.3.1 and 7.4.1]

This section describes the general process of pattern-based film grain synthesis, including pattern generation, block-based randomization, and local adaptation, as illustrated in Figure 8.

Figure 8: Pattern-based film grain synthesis workflow

[Ed. Note: It might be good if in this section we provide cross references to the subsequent sections and explicitly mention each block]

### Pattern generation

#### General

The first step in the synthesis process is to generate film grain patterns (small patches, typically 64x64 pixels) according to the model parameters that are received by the decoder. When the model parameters specify different grain characteristics for different pixel intensities (which is possible when using the FGC SEI), then as many patterns should be generated. Some implementations may support a limited number of patterns, by potentially merging the characteristics for different intensity intervals.

This step is executed every time the model parameters are updated, which is at most once per picture. It is expected to be run by software or firmware, while the subsequent steps (randomization, local adaptation, etc) are potentially run by hardware.

#### Frequency filtering model

In the FGC SEI based frequency filtering mode [Ed Note (PL): broaden scope beyond FGC SEI], the film grain characteristics are specified by horizontal and vertical spatial cut-off frequencies. The grain pattern for a given set of cut-off frequencies can be generated as follows:

- Generate a 2-dimensional array of random-value elements having a normalized Gaussian distribution. The 2-dimensional array represents discrete cosine transform (DCT2) coefficients. The column and row indexes of the array represent horizontal and vertical frequencies, respectively. (Note - The example method described in the FGC SEI message [2] uses 16x16 arrays. The methods specified in [5] and subclause 7.2.2 of this document use 64x64 arrays.)

- Set the value of all elements of the random-value DCT2 array to 0 when the corresponding column and row indexes are not within the corresponding high and low horizontal and vertical cut-off frequencies signalled in the FGC SEI message for the intensity interval. Also set the DC element to zero.

- Calculate the inverse discrete cosine transform (IDCT2) of the array produced in step2.

#### Autoregressive model

[Ed Note (PL): move 7.4.1.1 here, as least the generic/functional part]

### Randomization

#### General

Randomization consists of extending a film grain pattern (or a set of them), generated according to the model parameters, to the full picture.

It essentially consists in dividing the picture into blocks smaller than the pattern, and for each block, selecting a random region of the pattern, as illustrated in Figure 9. Methods considered here can bring additional diversity by randomly inverting the sign of the pattern for each block, which does not change the statistical properties since the gaussian noise is centered around zero. Other known methods of randomization are horizontal and vertical flip, which could be used if the film grain model is symmetrical, and rotations if the model is isotropic, but are not further described here.

Figure 9: image blocks each taking a random window from pattern.

Selecting a random region (or window) within the pattern(s) for a given block can be performed by generating a random x/y offset with the correct range, considering the size of the pattern and the size of the window. To give good results, neighboring blocks should select as little common pixels from the pattern as possible (the windows should not overlap too much), and the same sequence of offsets should not repeat.

#### Block size

The window (matching block size) should be significantly smaller than the pattern, otherwise the probability of overlapping the center of the pattern, and a large portion of it (because the window is large), would be too high (see Figure 10). As an example, a window ¼ the size of the pattern in each dimension gives acceptable results, while a window ½ the size of the pattern in each dimension can cause visible repetitions: the eye can recognize the center of the pattern and identify displacements, as illustrated in Figure 11.

However, the window should be large enough compared to the size of the grains, so that the statistics within the window are still representative of the pattern, and ultimately, the model parameters. In practice, 1/8th the size of the pattern in each dimension can be too small when using large grain, causing too much blocking artefacts. This leaves ¼ as a good compromise (e.g., 16x16 blocks for 64x64 patterns).

Figure 10: probability of (N x N) pattern pixels to be part of a random (n x n) window,
with n/N equal to 1/8 (left), 1/4 (center), 1/2 (right)

Figure 11: visual comparison of randomization using 16x16 or 32x32 blocks, and a common 64x64 pattern

#### Offset randomness

In addition to block size, randomization quality also depends on x/y offset randomness: adjacent blocks should avoid selecting similar offsets, which would mean a similar region of the pattern, thus visible repetitions. Similarly, repeating sequences of offsets would cause repetition of groups of blocks, which would be visible and should be avoided.

To reach these goals:

- The pseudo-random generator should be large enough, to be random enough, so that repetition of sequences is not likely. Figure 12 illustrates an example with a 32-bit LFSR (linear feedback shift register) using a 31-order polynomial, as in [5], that has been tested successfully. [Ed. Note: Maybe we can write better text on what we mean “large enough”. The wording is also not very good here.]

- The successive offsets should be as far as possible from each other. Depending on how offsets are derived from the pseudo-random generator, the method can differ (examples are given below).

- Reducing the choice to a limited number of evenly spaced positions (e.g., one out of 4) both ensures a minimal spacing and better scrambling, because the (finite) pseudo-random possibilities are spread over a small number of large displacements instead of many small (and potentially close) displacements. This has the additional benefit of enabling aligned reads of pattern memory in a practical implementation.

Figure 12: LFSR with 31st order polynomial (x31 + x3 + 1)

<figure: a few, evenly spaced window positions?>

#### Derivation of x/y offsets from the pseudo-random generator

The range of random values needed for the x and y offsets depends on the block size and offset alignment, as seen in previous sections.

Going from values of p with a uniform distribution in the range of [0 .. 2P[ (typical of a pseudo-random generator, or a bit field extracted from it), to a variable x (random offsets) with uniform distribution in a different range [0 .. N[, where N is not necessarily a power of two, can be achieved with different methods, which have different implications. [Ed. Note: is the ending “[“intending to show that this number is excluded? This is not defined in section 5 and it is better to either add -1 in the range or state the range verbosely without using the word “inclusive”]

The method used in [5], where P is 16 and N is 52 or 56, is to take a modulo: x = p % N. This splits the range of p into Ceil(2P ÷ N) bins of the same size, except the last one which can be smaller [to do: illustration], and x is basically an offset within the current bin. Since the number of bins is typically high and they all have the same size (except the last and unlikely one), the resulting distribution uniformity is typically very good. However, modulo is equivalent to a division, and when N is not a power of two, this can be costly for hardware implementations.

<figure: conversion to range N using modulo>

To avoid a non-power-of-two modulo, another method, used in [VFGS] [Ed. Note (YH): VFGS is JVET-Z0115 implementation software, may be referred in the Annex instead], is to multiply by N, then take the integer division by a power of two (bit masking, or right shift): x = ( p * N ) / 2P. This divides the range of p into N bins, with x the zero-based index of the bin [to do: illustration]. The bins do not have the same size: some can contain 2P/N values while others 2P/N + 1, which makes the distribution uniform up to N ÷ 2P (relative to the probability of one bin). The hardware cost of the multiplication by N is optimized by selecting N with a simple decomposition into powers of two (to minimize the number of adders for the multiplication), and a low number for P (to minimize the size of the adders), while keeping the uniformity error at an acceptable level. In [VFGS], N is either 12 (8+4: one adder) or 13 (8+4+1: two adders), with P=10 (extracting 10-bit fields of the random generator). [Ed. Note: The above text uses both a lower case p and an upper case P, which can make things confusing.]

<figure: conversion to range N using multiplication and shift>

Another consideration to optimize spacing between offsets relates to how the random generator is implemented and what bit field is used. The example given is an LFSR. Such generator could shift left and feed the new (pseudo-random) bit from the right (LSB), as in Figure 12; then two successive values of the LFSR would be related by a multiplication by 2, and potentially +1. The same holds for a continuous bitfield extracted from the LFSR (in the same order), for example the 16 lower bits. For low values, this could lead to successive offsets that are similar, with the eye potentially identifying the duplication/shift. Another option is that the LFSR shifts right and feeds the new bit from the left (MSB), as in Figure 13; that way, two successive values are related by a division by two, and potentially +half range, which means that successive offsets are less likely to be close.

Figure 13: Right-shifting LFSR with polynomial (x31 + x3 + 1)

About the random generator size, a 32-bit size is interesting in two aspects: first, compared to a 16-bit one, it provides a much longer repetition period. Second, several bit fields (e.g., 10 bits) can be extracted to generate different offsets and sign flips at the same time; those bit fields can partially overlap, but this overlap can be minimized so that extracted variables still look independent. An example is given below.

<figure : example of bit fields extraction from 32-bit LFSR for x/y offsets and sign flips for three colour components>

### Local adaptation

#### General

As explained in the introduction, film grain characteristics vary significantly depending on exposure, both in terms of amplitude and spatial correlation, with the amplitude variation being the most obvious. These variations are inherent to the image formation process at the physical film level. Replicating them on synthetic grain makes it look natural by anchoring it into the image, while a fixed overlay would likely not be visually pleasing.

Here, local adaptation consists of selecting the relevant grain pattern and grain amplitude according to pixel values and model parameters. For example, the FGC SEI message defines a set of intensity intervals for each color component, and for each interval, specific grain correlation parameters (either frequency-based or autoregressive coefficients) and the grain amplitude.

Local adaptation can be performed per-pixel or on a sub-block basis (based on local intensity average), with different implications, which will be detailed in the following sections.

#### Adaptation of grain shape

Adaptation of grain shape consists of selecting a specific grain pattern depending on local intensity, according to the model. This requires the initial stage to generate as many patterns as required to fit the model within practical implementation limits, as described in section 7.2.1.

Pattern selection can be applied either per-pixel or on a sub-block basis. This question arises because, since grain is spatially correlated, changing pattern within a block may destroy this spatial correlation, even though the random window and potential sign flip (see section 7.2.2) is kept constant for the whole block whatever the pattern selected:

- Selecting pattern on a sub-block basis [5] guarantees that the grain characteristics are preserved within the block as long as the grain size is significantly smaller than the sub-block. However, that requires computing a local intensity average to select the relevant pattern. Computing an average over a block requires line buffers, which may be too costly in hardware implementations. Also, changing pattern on sub-block boundaries requires deblocking so that the transition is not visible.

- Selecting a pattern on a pixel basis [as in VFGS] does not require line buffers nor deblocking, but requires solving the problem of spatial correlation preservation. This can be done at pattern generation time, by ensuring they are correlated with each other, making transitions smooth. For example, patterns can be the result of filtering the same underlying gaussian noise pattern with different frequency cut-offs or autoregressive coefficients. In implementations supporting a very limited number of patterns for storage cost reasons, transition smoothness could be further improved by pattern interpolation.

<Figure: illustrations>

#### Adaptation of grain amplitude

Adaptation of grain amplitude consists of scaling the amplitude of the pattern selected in the previous step by a scaling factor that depends on local intensity, according to the model.

There is no benefit in performing this adaptation on a sub-block basis. A pixel-based approach is more similar to the physical process, more closely follows the image, and avoids transitions on a fixed grid, which could potentially be visible in a video sequence.

<Figure: illustration? >

### Deblocking

Deblocking is required because of block-based randomization (see section 7.2.2), especially when grain is large.

When local adaptation is performed on a sub-block basis, deblocking may be needed on sub-block borders too.

Deblocking traditionally involves low-pass filtering block borders. However, for horizontal borders, this means vertical filtering, requiring line buffers (typically two, when using 3 coefficients).

To avoid the cost of such line buffers in hardware implementation, it may be preferable to use an overlapping process instead of a filter: for example [VFGS & AOM], on the first two lines, the samples of the current pattern matching two different random windows are blended together, the window of the current block and the block above. No line storage is required, just reading two places in the pattern (the address of these places can be computed on the fly). [Ed. Note: Revise paragraph to improve wording]

Another option is to attenuate grain amplitude on horizontal borders [5].

### Blending

The final stage is to blend the synthesized film grain with the picture. Though the FGC SEI message specifies either multiplicative or additive blending, practical implementations [listed here] use additive blending, followed by appropriate clipping.

[Ed Note (PL): end of generic description based on JVET-Z0115… to be merged with “overview of the generalized frequency filtering model” below][Ed. Note: please note that you need to add “message” after SEI] [Ed. Note: Does this mean that only additive blending is commonly used? Or does it mean that even when multiplicative blending is signaled. i.e., blending_mode_id = 1, that is ignored? That would be weird but that is somehow suggested here. Clarify?]

## Frequency filtering model for film grain synthesis

### Overview of the generalized frequency filtering model

[Ed Note (PL): this whole section is now mostly redundant with 7.2. To be pruned]

#### General

This subclause describes grain synthesis using the frequency filtering model. The overall process is illustrated in Figure 9 and consists of the following steps:

- Pre-compute a grain pattern database using a transform T

- Calculate an average luma value for each BlockSize x BlockSize block in the decoded picture to determine the intensity interval(s) to which the block belongs.

- Calculate pseudo-random offsets to be used to select grain blocks from the pre-computed grain pattern database.

- Deblock vertical edges across adjacent grain blocks.

- Blend grain blocks and the decoded picture.

Figure 9 Frequency filtering model FGS workflow

#### Generate a grain pattern database

In general, a database of grain patterns corresponding to each unique set of component model values signalled in an FGC SEI message can be determined as follows:

- Generate a 2-dimensional array of random-value elements having a normalized Gaussian distribution. The 2-dimensional array represents discrete cosine transform (DCT2) coefficients. The row and column indexes of the array represent horizontal and vertical frequencies, respectively. (Note - The example method described in the FGC SEI message [2] uses 16x16 arrays. The methods specified in [5] and subclause 7.2.2 of this document use 64x64 arrays.)

- Set the value of all elements of the random-value DCT2 array to 0 when the corresponding row and column indexes are not within the corresponding high and low horizontal and vertical cut-off frequencies signalled in the FGC SEI message for the intensity interval.

- Calculate the inverse discrete cosine transform (IDCT2) of the array produced in step2.

- Repeat steps 1 through 3 for each intensity interval signalled in the FGC SEI message.

Note that the database of grain patterns can be pre-computed and stored.

A fixed-point example of generating a grain pattern database is provided in subclause 7.2.2.

#### Determine applicable intensity intervals

For the frequency filtering model, applicable intensity intervals for each sample for each applicable colour component can be determined as follows:

- Assign each sample of the decoded image to a unique block of neighbouring sample values

- Calculate the average value of the sample values in the block

- Associate all samples within the block with the index of each interval value signalled in the FGC SEI message when the block average is within the upper and lower bounds of the intensity interval. Note that more than one or no intensity interval may be applicable to each block of samples.

A fixed-point example of determining applicable intensity intervals is provided in subclause 7.2.2.

#### Synthesize and blend film grain with a decoded picture

Synthesized grain can be constructed as follows:

- For each block of samples of the decoded picture having the same applicable intensity interval, select a block of the same size from the grain pattern for the corresponding intensity interval.

- Scale the values of the selected grain pattern block by the standard deviation applicable to the intensity interval.

- When correlation between consecutive colour components is signalled in the SEI message, add scaled sample values of the grain pattern for colour component (c – 1) to calculate the grain pattern for colour component c when the value of c is greater than 0.

Optionally, apply deblocking filters across block boundaries. [Ed. Note: should expected benefits for deblocking be mentioned? Also, this section is very vague, with no description of the deblocking filters applied. Maybe it could also be said that what is provided are only the parameters. How the parameters are interpreted and/or used may differ from implementation to implementation and to achieve different complexity/performance trade-offs. Should we have some reference to the deblocking process?]

A more detailed example of synthesizing and blending film grain with the decoded picture is provided in subclause 7.2.2.

### Fixed-point example of the frequency filtering model for film grain synthesis

#### General

The following describes a constrained fixed-point example of the generalized frequency filtering synthesis process described above. The fixed-point grain synthesis process can be used to produce numerically identical cropped decoded output pictures when the following apply:

–	4:2:0 chroma format

–	Frequency filtering film grain model with additive blending mode

–	The color space for film grain simulation is the same as for encoding.

–	No overlap between consecutive intensity intervals of the same color component.

–	No band-pass filtering and cross-color correlation.

Inputs to the process are the decoded picture sample arrays before grain blending decPictureL, decPictureCb, and decPictureCr.

Outputs of this process are the modified decoded picture sample arrays after grain blending blendPictureL, blendPictureCb, and blendPictureCr.

[Ed. note: what happens for mono? Also, what is the resolution of the pictures? These seem missing from the above description. Add more info.]

Each sample of synthesized grain, G[ c ][ x ][ y ], is added to the collocated sample of the decoded picture, Idecoded[ c ][ x ][ y ], for each applicable colour component followed by clipping to prevent out of range sample values.

The grain synthesis process is derived using the following ordered steps:

–	The grain pattern database generation is invoked in clause 7.2.2.2 with a 13x13x64x64 array GrainDb as the output.

–	The grain selection and blending process as specified in clause 7.2.2.3 is invoked with decPictureL, decPictureCb, and decPictureCr. as inputs, and blendPictureL, blendPictureCb, and blendPictureCr as outputs.

#### Grain pattern database generation

[Ed. Note: Some introduction should be provided here]

Output of this process is a 13x13x64x64 grain pattern database array GrainDb.

The variable prngVal is a value produced by a uniform pseudo-random number generator.

The variable gaussianLUT[ m ] is a LUT of 2,048 Gaussian random numbers stored in 2’s complement form and ranging from -127 to 127, inclusive.

The variable fgCutFreqH[ h ] is set equal to ( ( h + 3 ) << 2) – 1, with h being in the range of 0 to 12, inclusive.

The variable fgCutFreqV[ v ] is set equal to ( ( v + 3 ) << 2) – 1, with v being in the range of 0 to 12, inclusive.

The variable R64 is an IDCT2 64x64 transform matrix.

The variable fgCoeffs[ h ][ v ][ i ][ j ] is initially set equal to 0, with h = 0..12, v = 0..12, i = 0..63, j = 0..63, and is derived as follows:

for(h = 0; h < 13 , h++ ) {	7-1
	for(v = 0; v < 13 , v++ ) {
		prngVal = SeedLUT[ h + 13 * v ] 
		for( j = 0; j <= fgCutFreqV[ v ] ; j++ ) {
			for( i = 0; i <= fgCutFreqH[ h ] ; i + = 4 ) {
				for( k = 0; k < 4; k++ )
					fgCoeffs[ h ][ v ][ i + k ][ j ] = gaussianLUT[ ( prngVal + k ) % 2048 ]
				prngVal = Prng( prngVal )
			}
		}
		fgCoeffs[ h ][ v ][ 0 ][ 0 ] = 0
	}
}

Where SeedLUT[ i  ] with i = 0..255 is the pseudo-random number generator array.

For given h, and v, with h in the range of 0 to 12, inclusive, v in the range of 0 to 12, inclusive, the 64x64 array GrainDb[ h ][ v ][ x][ y ] is derived as follows:

bh[v]=((R64T*fgCoeff[h][v]+128) ≫8)*R64+128≫8 	7-2

GrainDb[ h ][ v ][ x ][ y ] = Clip3( -127, 127 , b[ h ][ v ][ x ][ y ] )

[Ed Note (PL): the following is specific to RDD5-based implementations (is it the case for the whole 7.2.2 section?). It describes a vertical pre-deblocking technique by attenuation baked into the patterns, that matches the specific randomization and local adaptation of RDD5: random vertical offset constrained to a multiple of 8, and local adaptation based on 8x8 sub-blocks. Maybe move that to a section related to deblocking, since alternatives exist? E.g., while using mostly the same frequency-limited DCT-based pattern generation as above, Z0115 proposes alternate randomization and local adaptation, with less frequent deblocking and less visual drawbacks (less softening)].

The grain pattern database GrainDb is further refined as follows, where the value of BlockSize is equal to 8:

deblockFactor[ v ] = { 64, 71, 77, 84, 90, 96, 103, 109, 116, 122, 128, 128, 128 } 	7-3
for( j = 0; j < 64 ; j += BlockSize ) {
	for( i = 0; i < 64 ; i++ ) {
		GrainDb[ h ][ v ][ i ][ j ] = ( GrainDb[ h ][ v ][ i ][ j ] * deblockFactor[ v ] )  >>  7
		GrainDb[ h ][ v ][ i ][ j+ BlockSize – 1  ] = 
			( GrainDb[ h ][ v ][ i ][ j+ BlockSize – 1 ] * deblockFactor[ v ] )  >>  7
		}
	}
}

#### Grain selection and blending process

##### General

[Ed. Note: Some introduction should be provided here]

Input to this process is the decoded picture prior to grain blending, i.e., the arrays decPictureL, decPictureCb, and decPictureCr.

Output of this process is the modified decoded picture after grain blending, i.e., the array blendPictureL, blendPictureCb, and blendPictureCr.

The variable BlockSize indicates the BlockSize x BlockSize size of grain blocks.

The variable prngArray is an array of values produced by a uniform pseudo-random number generator.

For cIdx = 0..2, the following apply:

- If cIdx is equal to 0, decSamples, PicWidth, PicHeight, PicWidthInBlock, and PicHeightInBlock are derived as follows:

  - decSamples corresponds to the decoded picture sample array decPictureL and blendSamples corresponds to the blended sample array blendPictureL

  - PicWidthInLumaSamples is the width of the decoded picture in units of luma samples

  - PicHeightInLumaSamples is the height of the decoded picture in units of luma samples

  - PicWidth is set equal to PicWidthInLumaSamples

  - PicHeight is set equal to PicHeightInLumaSamples

  - PicWidthInBlock is set equal to Ceil( PicWidthInLumaSamples ÷ BlockSize )

  - PicHeightInBlock is set equal to Ceil( PicHeightInLumaSamples ÷ BlockSize )

- Otherwise (cIdx is not equal to 0), decSamples, blendSamples, PicWidth, PicHeight, PicWidthInBlock, and PicHeightInBlock are derived as follows:

  - If cIdx is equal to 1, decSamples corresponds to the decoded picture sample array decPictureCb and blendSamples corresponds to the blended sample array blendPictureCb

  - Otherwise (cIdx is not equal to 1), decSamples corresponds to the decoded picture sample array decPictureCr and blendSamples corresponds to the blended sample array blendPictureCr.

  - PicWidth is set equal to PicWidthInLumaSamples / 2

  - PicHeight is set equal to PicHeightInLumaSamples / 2

  - PicWidthInBlock is set equal to Ceil( PicWidthInLumaSamples ÷ 2 ÷ BlockSize )

  - PicHeightInBlock is set equal to Ceil( PicHeightInLumaSamples ÷ 2 ÷ BlockSize )

- If film mode is not modelled for the cIdx-th color component, blendSamples[ i ][ j ] is set equal to decSamples[ i ][ j ], with i=0.. PicWidth, j=0..PicHeight

##### Film grain block blending process

[Ed. Note: Some introduction should be provided here?]

Inputs to this process are:

- a location ( xCurr, yCurr ) specifying the top-left sample of the current block relative to the top-left sample of the current picture component,

- a variable cIdx specifying the colour component of the current block,

- a block index ( xIdx, yIdx ) specifying the block index of the current block,

- a picture seed array prngArray,

- a decoded sample array decSamples.

- a variable numIntensityIntervals [ c ] specifying the number of intensity intervals of c-th color component

- A variable intensityIntervalLowBound [ c ][ i ] specifying the lower bound of the i-th intensity interval of c-th color component

- A variable intensityIntervalUpperBound [ c ][ i ] specifying the upper bound of the i-th intensity interval of c-th color component

- A variable bitDepth specifying the bit depth for the luma component of the film grain

- A variable modelValue[ c ][ i ][ 0 ] specifies the standard deviation of i-th intensity interval of c-th color component

- A variable modelValue[ c ][ i ][ 1 ] specifies the horizontal high cut frequency

- A variable modelValue[ c ][ i ][ 2 ] specifies the vertical high cut frequency

Output to this process is the blended sample array blendSamples.

The variable picWidth is set equal to ( cIdx  = =  0 ) ? PicWidthInLumaSamples : PicWidthInLumaSamples / 2.

The variable picHeight is set equal to ( cIdx  = =  0 ) ? PicHeightInLumaSamples : PicHeightInLumaSamples / 2.

The variable intensityIntevalIdx for the current block is initialized to be – 1 and is derived as follows:

sumBlock = 0 	7-4
avgRatio = Log2( BlockSize / 8)
for( i = 0; i < BlockSize ; i++ )
	for( j = 0; j < BlockSize ; j++ )
		sumBlock += decSamples[ Min( picWidth − 1, xCurr + i ) ][ Min( picHeight − 1, yCurr + j ) ] 
bavg = Clip3( 0, 255, ( sumBlock + ( 1  <<  bitDepth[ cIdx ]  ) )  >>  ( bitDepth[ cIdx ] + 2 * avgRatio − 2 )

for( i = 0; i  <  numIntensityIntervals[ cIdx ]; i++ )
	if( bavg  >=  intensityIntervalLowBound[ cIdx ][ i ]  &&
			bavg  <=  intensityIntervalUpperBound[ cIdx ][ i ] ) {
		intensityIntervalIdx = i
		break
}

The derivation of decSamples for the current block is as follows: [Ed. Note (YH): the following derivation is RDD5 implementation, either use VSEI general text or moved following text to next clause 7.2.3]

- If intensityIntervalIdx is equal to –1, blendSamples[ xCurr + i ][ yCurr + j ] is set equal to decSamples[ xCurr + i ][ yCurr + j ], for i=0..Min(picWidth – xCurr – 1, BlockSize – 1), j=0.. Min(picHeight – yCurr – 1, BlockSize – 1 );

- Otherwise (intensityIntervalIdx is not equal to –1), the following applies:

  - The variable scaleFactor is set equal to modelValue[ cIdx ][ intensityIntervalIdx ][ 0 ] * ( 1 – 2 * BIT0( prngArray[ xIdx ][ yIdx ] )

  - The variable hFreq is derived as follows:

hFreq = modelValue[ cIdx ][ intensityIntervalIdx ][ 1 ]

hFreq = (cIdx  = =  0) ? hFreq : Clip3( 2, 14, ( hFreq  <<  1 ) )

hFreq = hFreq – 2

  - The variable vFreq is derived as follows:

vFreq = modelValue[ cIdx ][ intensityIntervalIdx][ 2 ]

vFreq = (cIdx  = =  0) ? vFreq : Clip3( 2, 14, ( vFreq  <<  1 ) )

vFreq = vFreq – 2

  - The variable horzDenom = (BlockSize  = =  32)? 36: 52

  - The variable vertDenom = (BlockSize  = =  32)? 40: 56

  - The variable horzOffset is derived as follows:

horzOffset = ( MSB16( prngArray[ xIdx ][ yIdx ] )  %  horzDenom )

horzOffset  & =  0xFFFC

horzOffset  + =  ( BlockSize  = =  8 ) ? ( xIdx & 0x0008 ) : 0

  - The variable vertOffset is derived as follows:

vertOffset = ( LSB16( prngArray[ xIdx ][ yIdx ] )  %  vertDenom )

vertOffset  & =  0xFFF8

vertOffset  + =  ( BlockSize  = =  8 ) ? ( yIdx & 0x0008) : 0

  - grainSamples is derived as follows, for i = 0..Min( picWidth – xCurr – 1, BlockSize – 1 ), j = 0.. Min( picHeight – yCurr – 1, BlockSize – 1 ):

grainSamples[ xCurr + i ][ yCurr + j ] = 
		( scaleFactor * GrainDb[ hFreq ][ vFreq ][ i + horzOffset ][ j + vertOffset] )  >>  ( fg_log2_scale_factor + 6 )

- if xCurr > 0, the following applies:

for( k = 0; k < BlockSize; k++ ) {
  	l1 = grainSamples[ xCurr – 2 ][ k ]
  	l0 = grainSamples[ xCurr – 1 ][ k ]
  	r0 = grainSamples[ xCurr ][ k ]
  	r1 = grainSamples[ xCurr + 1 ][ k ]
  grainSamples[ xCurr ][ k ] = ( ( l0 + ( r0  <<  1 ) + r1 )  >>  2 )
  grainSamples[ xCurr – 1 ][ k ] = ( ( l1 + ( l0  <<  1 ) + r0 )  >>  2 )
}

  - blendSamples[ xCurr + i ][ yCurr + j ] is derived as follows, for i = 0..Min( picWidth – xCurr – 1, BlockSize – 1 ), j = 0.. Min( picHeight – yCurr – 1, BlockSize – 1 )

blendSamples[ xCurr + i ][ yCurr + j ] = Clip3( 0, ( 1  <<  fgBitDepth[ cIdx ] ) − 1,	
 decodedSamples[ xCurr + i ][ yCurr + j ] + 
( grainSamples[ xCurr + i ][ yCurr + j ]  < < (fgBitDepth[ cIdx ] – 8 ) ) )

### Fixed-point example of the frequency filtering model conforming to SMPTE RDD-5

SMPTE RDD-5 [5] specifies a constrained version of the process described in subclause 7.2.2. An RDD5 implementation shall apply the following constraints:

–	variable prngVal is specified in [5] subclause 1.5.2.

–	The variable gaussianLUT[ m ] is specified in [5] subclause 2.2.1.

–	The variable R64 is the IDCT2 transform specified in [5] subclause 2.2.3.

–	The variable BlockSize is equal to 8.

–	The variable prngArray is determined as specified in [5] subclause 1.5.3.3

### Fixed-point example of resolution-adaptive frequency filtering model for film grain synthesis

The following describes a resolution-adaptive version of the process described in subclause 7.2.2 for use cases in which it may be beneficial to adjust the grain block size in proportion to common picture formats, e.g., HD, 4K UHD, and 8K. The resolution-adaptive fixed-point grain synthesis process can be used to produce numerically identical cropped decoded output pictures when applying the following constraints:

–	Let PicSizeInLumaSamples specify the total size in terms of luma samples of the entire picture.

–	The variable prngVal is specified in [5] subclause 1.5.2.

–	The variable gaussianLUT[ m ] is specified in [5] subclause 2.2.1.

–	The variable R64 is the IDCT2 transform specified in [1] clause 8.7.4.4.

–	BlockSize is determined as follows:

if PicSizeInLumaSamples <= (1920*1080)
		BlockSize = 8
	elseif PicSizeInLumaSamples <= (3840*2160)
		BlockSize = 16
	else
		BlockSize = 32

–	The variable prngArray is determined as specified in [5] subclause 1.5.3.3.

[Ed. Note: Variables such as PicSizeInLumaSamples need to be defined. Also can we reference variables in external documents?]

### Fixed-point example of resolution-adaptive frequency filtering model for film grain synthesis without a line buffer

The following describes a resolution-adaptive version of the process described in subclause 7.2.2 for use cases in which it may be beneficial to eliminate the need of line buffers to compute the block average described in 7.2.1. The resolution-adaptive fixed-point grain synthesis process can be used to produce numerically identical cropped decoded output pictures when applying the following constraints:

–	variable prngVal is specified in [5] subclause 1.5.2.

–	The variable gaussianLUT[ m ] is specified in [5] subclause 2.2.1.

–	The block size is indicated by BlockWidth x 1 and determined as follows:

if PicSizeInLumaSamples <= (1920*1080)
		BlockWidth = 8
	elseif PicSizeInLumaSamples <= (3840*2160)
		BlockWidth = 16
	else
		BlockWidth = 32

–	The variable prngArray is determined as specified in [5] subclause 1.5.3.3

## Autoregressive models for film grain synthesis

### Overview of the generalized autoregressive model

This subclause describes grain synthesis using the autoregressive model. The overall process consists of the following steps:

- Generate a grain pattern using an autoregressive filter applied to a series of Gaussian variables. The grain pattern may be pre-computed.

- Calculate the intensity or intensity interval(s) to which each sample in the decoded picture belongs.

- Select a grain sample from the grain pattern for each sample location in the decoded picture.

- Blend grain samples and the decoded picture samples.

#### Grain pattern generation

A grain pattern can be generated using an autoregressive filter as follows:

[Ed. Note (PL): To be moved to “General description of pattern-based film grain synthesis” ]

[Ed. Note: The indices in the following should be reviewed.]

G(x,y,c) = a0,0n+i=-L-1a0,jG(x+i,y,c)+i=-LLj=-L-1ai,jG(x+i,y+j,c)+k<cbkG(x,y,k) 	7-5

Where n is a zero-mean Gaussian variable, ai,j are autoregressive coefficients, bk are colour correlation coefficients, L is a lag parameter, c is the colour component index, and G(x,y,c) is the grain value for colour component, c, at the sample location x,y. An autoregressive filter with lag parameter value equal to 2 is illustrated in Figure 10.

Figure 10 Autoregressive filter with lag equal to 2

For the FGC autoregressive model, the lag parameter is not signalled explicitly. Instead it is implicitly specified to be in the range of 0 to 2, inclusive, depending on the number of model values present for each intensity interval in which the film grain has been modelled. The coefficients allowed for the FGC autoregressive model are constrained as illustrated in

Figure 11 where the variable, A, is the pixel aspect ratio of the film grain. The film grain pixel aspect ratio could be useful in cases in which a picture is stretched in resizing or was captured with isomorphic optics [6]. [Ed. Note (PL): isomorphic or anamorphic?]

Figure 11: FGC autoregressive model

[Ed. Note (PL): There is a problem (in SEI spec) with aspect ratio & a-1,-1 & a1,-1. They should both be multiplied by A (or none of them)]

For a hypothetical example extended version of such a film grain synthesis model, the lag parameter, L, could have a larger range, such as 0 to 3, inclusive.

### Fixed-point example of film grain synthesis supporting both frequency filtering and autoregressive models

[Ed. Note (PL): See JVET-Z0115/VFGS. Most of the material is already in section “General description of pattern-based film grain synthesis”. Maybe move/copy some specific examples and implementation details here ?]

### Example of the autoregressive model for film grain synthesis

[Ed. Note (PL): we could reuse the Z0115 description here (only pattern generation changes -see 7.2.2.1), and maybe state the existence of AOM stuff with reduced capability (single pattern)]

The following describes an example of an autoregressive model for film grain synthesis when the following properties apply:

–	4:2:0 chroma format

–	Auto-regression model with additive blending mode

–	The color space for film grain simulation is the same as for encoding.

–	lag value is less than or equal to 2

–	all colour correlation coefficients are equal to 0

–	pixel aspect ratio for film grain is equal to 1

[TO BE COMPLETED]

#### Grain pattern database generation

The variable ar_prngArray is an 82x73 array of values produced by a pseudo-random number generator

The variable ar_coeff[ ar_lag + dx ][ ar_lag + dy ] is equal to adx,dy

for ( y = 3; y < 73; y++) { 	7-6
	for ( x = -3; x <= 82; x++) {
		for ( dy = -ar_lag; dy <= 0; dy++) {
			grainDb[ x ][ y ] = 0
			for ( dx = -ar_lag; dx <= ar_lag; dx++) {
				if( dy == 0 && dx == 0)
					break
				grainDb[ x ][ y ] + = 
					ar_coeff[ ar_lag + dx ][ ar+lag + dy ] * ar_prngArray[ x + dx ][ y + dy ]
			}
		}
	}
}

[ADD COLOUR PROCESS]

[TO BE COMPLETED]

#### Grain pattern scaling table determination

[TO BE COMPLETED]

#### Grain strip construction

[TO BE COMPLETED]

      - Grain blending

Inputs to this process are w, h, subX, and subY

[TO BE COMPLETED]

### Use of an FCS SEI message with the example autoregressive model for film grain synthesis

The following describes the use of the FGC SEI message with the example process described in 7.3.2. [TO BE COMPLETED]

[Ed. Note (PL): Z0115/VFGS: no problem, that’s supported; AOM: need either SEI restriction to have common AR coefficients for all intensity intervals, or take the ones of the first interval... or the most significant, etc, etc]

# Film grain analysis

## General

[Ed. Note (PL): describing analysis after synthesis may be helpful to understand model parameters before learning how to estimate them; also, analysis may be optional, as parameters could be presets, or upstream info, potentially adjusted to coding parameters]

[Ed. Note: possibly describe how film grain can be removed from input video.]

[Ed. Note: General overview of common pre-processing steps: filtering, noise estimation, edge detection, etc.]

The film grain analysis process is applied on the encoder side and is a non-normative process regardless of the synthesis method and standard in use. It can be implemented as a pre-processing step or as a part of the encoding process. Ultimately, it provides indicative features of the film grain in accordance with the selected parameterized model and supported metadata. It determines film grain model parameters to be sent in the appropriate metadata mechanism, e.g., in an SEI message, to enable content-aware decoder-side film grain synthesis. The analysis process is not mandatory, and manually tuned parameters can be provided to the encoder based on an input configuration file.

Figure 12 depicts a simplified framework for film grain analysis. As an input to the process, besides the source video, the denoised representation is also required. Edge and complex texture analysis is performed in order to determine a map of flat and non-flat regions in the scene. This is done since high-frequency components, such as edges and texture, can interfere with the analysis process of the film grain, which also resides in high-frequencies.

Figure 12: Film grain analysis and parameter estimation general steps.

The presented generalized model is one common approach to estimate noise and noise parameters. Different approaches are possible, for example, by using appropriate technics noise can be estimated without the need for denoised version of the source video. Anyhow, the most commonly used film grain analysis workflow consists of the following commonly implemented steps, divided into two groups.

Film grain model independent steps (independent from film grain parameter format):

- a pre-processing step to produce a noise estimate (a.k.a. film grain image). It consists of:

  - denoising the video and

  - finding the difference between source and denoised pairs to get noise estimate (film grain image).

- a pre-processing step to produce other information about the characteristics of the input video such as edge and texture analysis. [Ed. Note: Is this a pre-processing or pre-analysis step?]

Model specific step:

- film grain analysis and parameter estimation step to determine model parameters based on the outputs from 1) and 2).

  - estimate grain amplitude based on underlying picture intensity

  - estimated frequency limits or auto-regressive parameters on relevant picture intensity levels

Film grain analysis and parameter estimation depend on the selected parameterized model. Some examples of a possible implementation are provided in the following subclauses.

## Denoising and image analysis

### Denoising

[Ed. Note (MR): add some examples of denoising approaches.]

Possible solutions for denoising the input video source include the use of motion adaptive, or motion compensated temporal filtering (MCTF) methods. MCTF methods utilize motion search and motion compensation algorithms to exploit temporal redundancies and isolate the video signal from the noise signal, and therefore achieve denoising. Motion adaptive methods perform simple decisions using temporally neighboring samples to determine noise. Although they are commonly less performing, they usually also have lower complexity, e.g., one variant is given in [10]. Other approaches can be used instead of MCTF. Block-matching and 3D filtering (BM3D) is the most popular hand-crafted image denoising algorithm [11]. In some implementations, reconstructed video sequence can be used, reducing the need for denoised sequence in film grain estimation (depending on the quantization parameter, reconstructed sequence can highly approximate denoised sequence with most of the details preserved, but with low frequencies (noise) removed).

In contrast to traditional analytical denoising approaches, recent advances utilize data driven deep learning techniques to provide state of the art performance [10][11][12]. However, deep learning approaches usually suffer from excessive computational complexity.

After denoising, film grain images that represent noise estimates are generated by computing the difference between the original input frames and the filtered/denoised frames that correspond to the same time instance. It is important to point out that the use of denoised equivalent is not only helpful in the film grain modeling and parameter estimation, but when encoding denoised sequence it also can considerably enhance compression efficiency [15].

### Edge and texture analysis

[Ed. Note (MR): possibly to add short overview of available approaches?]

Edge and texture analysis is usually performed on a source image (rather than the denoised one) so as not to omit certain details (high frequencies) that can be removed during filtering, but still reside in the film grain image. If analysis is performed on the denoised image, certain details may not be detected, and some regions with low frequencies could be used in film grain analysis, leading to wrong estimation.

One possibility could be to use flat block finder that uses the gradient-based features with the thresholds that re more lenient to allow for correct grain modeling in extreme cases, for example, as proposed in [16]. Another approach could involve well established edge detection algorithms such as Canny edge detector in conjunction with morphological operations. To create a map of flat regions, one approach can be to perform analysis on three scales using the source sequence: the original and two subsampled resolutions, one by a factor of 2 and another by a factor of 4 horizontally and vertically. At each scale, a Canny edge detector is applied (without the classical Canny detector blurring step) to determine edges. Morphological operations are applied afterwards at each scale to extend the influence of the edges and to make sure that the edges do not jeopardize the process of estimating the film grain. The obtained maps at the subsampled scales can then be upsampled to the original resolution and combined. Additional morphological operations can be subsequently applied to the final map.

As an illustration, VTM implementation applies 4-pass dilations at original scale, 3-pass dilatation on subsampled scale by factor 2, and 2-pass dilations on subsampled scale by factor 4. After upsampling and combining all scales in one flat region mask at full resolution, 2-pass dilations followed by 1-pass erosion are applied.

The film grain image and map of flat regions are then provided to the noise analysis & parameter estimation module.

## Frequency filtering model parameter estimation

### Determination of intensity intervals and grain variance

Since film grain is dependent on the local characteristics of an image, different scalings of the film grain can be applied for different intensities or intensity intervals of an input image in order to get appropriate intensity of the film grain before finally blending it to the input image. After creating a film grain pattern, scaling to the proper intensity based on the scaling factor is performed. It determines the level at which the film grain will be perceived at the final image, and by doing that it is ensured that the film grain is simulated at the correct scale.

Intensity intervals and the corresponding grain variance values are determined by analyzing the film grain image, the filtered image, and the flat-region map produced during pre-processing. Scaling function, for example, can be represented as polynomial function, or lower-degree approximation of a (polynomial) scaling function can be considered depending on the implementation. For example, piece-wise linear function or stepwise constant scaling function as illustrated in Figure 13 can be considered implementation.

Figure 13: An example of film grain scaling function defined with the intensity interval and film grain variance values. This approach is used in FGC SEI message defined in H.264, HEVC and VVC.

The analysis is typically performed on a grid of non-overlapping bocks, with block size equal to BlockSize. BlockSize can be defined a priori, or it can be determined based on the frame resolution. In any case, the analysis process, or more precisely block size used in analysis process, should be in sync with the synthesis approach, e.g., uses the same block size as during film grain blending.

Thereafter, to estimate intensity intervals and associated scaling factors, first step would be to calculate indicative features for each block that enters the process of film grain parameter estimation (each block within a flat region of the image). The first feature is the average value (avg) calculated on a block within the filtered frame. The second feature is the variance of the film grain block (var), and it is calculated on a block from the film grain image. They define data points (avg, var) that are used in further analysis. Scaling factors estimation is based on (avg,var) data points computed on non-overlapping blocks belonging to flat areas (indicated by the mask provided in the previous step).

Scaling function can be obtained by fitting data points to the polynomial function. Some additional processing of data points can be required before function fitting, for example, to remove outlier and to improve estimated function precision.

### An example of FGC SEI message scaling factor estimation

#### General

Since the FGC SEI message does not specify a means to determine values for the FGS model, an example code for analyzing video to determine FGC SEI message syntax values is included in the VTM reference code and is described in this subclause. The following method at the end provides an example according to the VTM implementation which is based on SMPTE RDD-5 specification with some additional modifications [Ed. Note: add reference to jvet documents], e.g., the specific analysis method described in this section conforms to 7.2.4.

First, BlockSize can be defined as follows:

PicSizeinLumaSamples = PicHeightInLumaSamples * PicWidthInLumaSamples

if PicSizeinLumaSamples <= ( 1920 * 1080 ) 	8-1
    BlockSize = 8
else if PicSizeinLumaSamples <= ( 3840 * 2160 )
    BlockSize = 16
else
    BlockSize = 32

Thus, both film grain image and filtered image are scanned block by block at 8x8 or 16x16 or 32x32 granularity (depending on frame size).

The further process, that operates on data points (avg,var) collected in previous step, is described as follows.

#### Data points regularization and discarding potential outliers

Data points regularization is used to regularize high variations and to control the excessively fluctuating data points. The following regularization function can be used:

double k=3.0;

double m=.5;

double tmp = k * pow((double)(var), m) + .5;

var = (int)tmp;

Coefficient m defines regularization of dispersity/heteroscedasticity of data points.

Coefficient k defines the level of film grain to be added back, it should be aligned to the level of the film grain being removed from the source (during the filtering and quantization).

After regularization is applied, the data points with the var higher than 16 << (bitDepth - 8) are discarded from the calculation to limit data points to meaningful values. If var is higher than the maximum defined value, there is a possibility that variance is biased by edges and other high frequency components removed during the filtering.

#### Curve fitting and curve quantization

##### General

The data points are used to determine scaling factors (grain variance) in two steps:

- derive a scaling function by two-pass curve fitting

- quantize the fitted curve to produce a stepwise function

The process is also illustrated in Figure 14.

a											e

b											f

c											g

d											h

Figure 14: 2-pass curve fitting process [Ed. NOTE (MR): Update plots, add name on axis. It is better to place plots in a table with each subplot being in a cell].

##### Sub-range averaging

On Figure 14a data points are already regularized and limited to max value. Complete intensity dynamic range (horizontal axis) is divided (uniformly quantized) into non-overlapping intervals of size 16. Data points are then grouped per intervals. For each intensity sub-range, the average value of the variance is calculated only if minimum number of data points within the sub-interval is Nmin> 8. It is illustrated on Figure 14b.

##### Discarding potential outliers (1st pass)

If there is single point without any neighbouring points, it is considered as an estimation error and it is removed from further calculations. Single points are filtered out, see the missing point in Figure 14c compared to Figure 14b.

##### Extreme points extrapolation

A step of extension of points to the left and to the right towards the zero is then applied to smooth transition from intensities with film grain to the intensities without film grain. The process finds the most left and most right point and extends the data-point range, e.g., see added points in Figure 14c. At most 4 new points are added to the left and 4 new points to the right.

##### Discarding potential outliers (2nd pass)

Also remove points if intensity (horizontal axis) is less then 40 or larger than 950 (for 10-bit signal; these values are shifted for bitdepths other than 10). Indeed, film grain is usually not added to the very dark and very bright regions.

##### Curve fitting (1st pass)

Next step is to fit the parametrized curve by using 4rd order polynomial fitting, Figure 14d.

##### Discarding potential outliers (3rd pass)

+0.6*std and -1.2*std around the first fitted curve is used to filter out the points outside the given range. This step filters out remaining outliers and biased variance points.

##### Curve fitting (2nd pass)

The remaining points are entering second pass of curve fitting. Previous processes are repeated (8.2.2.2.1 to 8.2.2.2.5 illustrated on Figure 14f to Figure 14h) and again the polynomial curve is fitted, but this time using reduced set of data-points. The final scaling function is illustrated on Figure 14h.

#### Final scaling function approximation

Final step approximates the scaling function represented by the 4rd order polynomial curve by a simplified stepwise scaling function, see Figure 15. The stepwise function is derived by using Lloyd-max non-uniform quantization with 4 quantization levels. Quantizer is adapted (trained) for each new set of points on-fly.

a											b

Figure 15: Fitted curve (left) and quantized curve (right).

The use of stepwise scaling function leads to several intensity intervals and scaling factors as it is illustrated Figure 15b. Note, stepwise scaling function is requirement of the FGC SEI design. Currently, film grain SEI message as defined in VSEI and implemented in VTM comprises the following parameters that define film grain scaling function which is applied at the decoder/synthesis side:

- fg_intensity_interval_upper_bound[ c ][ i ] less than fg_intensity_interval_lower_bound[ c ][ i + 1 ]

- fg_comp_model_value[ c ][ i ][ 0 ],

where fg_comp_model_value[ c ][ i ][ 0 ] represents the scaling factor, i is index of the interval, and c is color component. One example of such scaling function is also given in Figure 13. Note that each step of a given stepwise function represents one intensity interval, defined with its bounds (lower and upper bound) and scaling factor value.

Analysis is performed in the same way for all colour components.

### Determination of cut-off frequencies

Cut-off frequencies can be estimated as follows. The film grain image is scanned block by block, using non-overlapping blocks grid, where block size depends on the specific implementation. Only blocks within the flat part of an image and matching relevant intensity intervals are processed further. For each block coming from a flat region, a forward transform (usually DCT-2) is applied. Then, each coefficient resulting from the transform is squared. Afterwards, an average squared transformed block Bavg is computed over all available squared transformed blocks Bi (pixel-wise) as:

Bavgx,y=1Ki=0K-1Bi(x,y)	8-2

where K is the total number of blocks that is used for the calculations. Thereafter, the average columns Bc and by rows Br vectors are calculated (the DC component for the first row and the first column is discarded from the computation) as follows:

Bcy=163+y>0?1:0i=y>0?0:163Bavg(i,y) 	8-3

Brx=163+x>0?1:0i=x>0?0:163Bavg(x,i).	8-4

The average vectors Bc and Br are regularized to suppress peaks. The average vectors are represented as a curve and its intersection points with a total average value avg of a block Bavg is computed (avg=164*64i=064-1j=064-1Bavg(i,j)). The value of avg is therefore subsequently used as a threshold for other computations. [Ed. Note (PL): taking the average power as a threshold is actually a bit weird (e.g. if noise is full-band). This needs further study]

Based on the analysis of the intersection point(s), cut-off frequencies are obtained. The intersection points are chosen to represent the cut-off frequencies of the frequency-filtered film grain model.

This process can be repeated to estimate frequency limits for different intensity intervals, as is allowed by FGC SEI and could be required to accurately model the film grain, since in a photographic process, its spatial frequency highly depends on exposure.

The scaling parameters signalled for each intensity interval depend on the noise variance estimated in the previous step, but also need to factor in the frequency limits, since on synthesis side, different frequency limits lead to different grain amplitudes. [Ed. Note (PL): to be completed with formulas]

If no intersection points are found, film grain is not present in the input frame. If the analysis process determines that no grain is present in the source video, the appropriate syntax element values could be set to indicate that synthesis will not be performed at the decoder side.

### An example of FGC SEI message cut-off frequency estimation

[Ed. Note (MR): to be completed .]

The following method provides an example according to the VTM implementation which is based on SMPTE RDD-5 specification with some additional modifications [Ed. Note: add reference to jvet documents], e.g., the specific analysis method described in this section conforms to 7.2.4.

The illustrated implementation limits filtering to low pass filtering as defined by SMPTE RDD-5, even that FGC SEI supports band pass filtering.

An implementation of the FGC SEI message parameter estimation that conforms to the method described in the FGC SEI message [2] uses 16x16 arrays to define film grain patterns. However, the methods specified in [5] and subclause 7.2.2 of this document use 64x64 arrays. Thus, 64x64 DCT-2 as defined in [] can be used within analysis process. Some implementations can choose to use 64x64 DCT-2 as described in [] instead.

Figure 16: Cut-off frequency estimation (intersection points x1 and x2 represent estimated frequencies).

Figure 16 illustrates the process of horizontal and vertical cut-off frequency estimation on a 64x64 arrays. For a give intersection points x1 and x2, the model values are set to (according to the SMPTE RDD 5 specification scaling to 16x16 arrays is needed):

comp_model_value[ c ][ i ][ 1 ] = Clip3(2,14,(x1-1)>>2) and

comp_model_value[ c ][ i ][ 2 ] = Clip3(2,14,(x2-1)>>2),

where i is the index of the interval, and c is the colour component index. If no intersection points are found, film grain is not present in the input frame.

[Ed. Note (MR): Frequency estimation can be studied more.]

## Autoregressive model parameter estimation

[Ed. Note (PL): only the very last part (replacing frequency limits) is needed here. AR parameters can be determined by using Yule-Walker equations]

# Film grain metadata: syntax and semantics

## General

The VVC, HEVC, and AVC standards natively support signalling of film grain parameter values using well defined supplemental enhancement information message supplement

The VVC, HEVC, and AVC standards currently directly specify a supplemental enhancement information (SEI) message for indicating the usage of film grain synthesis when decoding and rendering video or image data. In particular, these standards support the film grain characteristics (FGC) SEI message, which is capable of indicating two different film grain models and different blending modes. More specifically, this SEI message supports a frequency filtering and an autoregressive film grain model, and additive or multiplicative blending modes. It can also indicate one or more intensity intervals, the film grain variance for each intensity interval, and the spatial frequency characteristics of the film grain for each intensity interval, providing considerable flexibility to content creators and encoding manufacturers on signaling different types and levels of film grain noise.

An alternative film grain synthesis scheme, such as a different use of an autoregressive model, can also be indicated through the use of the ITU-T T.35 registered or unregistered user data SEI messages that are also supported in these standards. The VVC, HEVC, and AVC standards intentionally do not specify any constraints or limitations on the postprocessing techniques that can be used with decoded data from such user data SEI messages, therefore enabling more applications and implementations.

## Film grain characteristics SEI message

### General

Film grain metadata for use with VVC, HEVC, and AVC can be signalled via an FGC SEI message specified in the corresponding coding standard. For VVC, the SEI payload Type is specified in [1] Annex D and the syntax and semantics are specified in [2] clause 8.5. For HEVC, the FGC SEI message is specified in [3] clauses D.2.13 and D.3.13. For AVC, the FGC SEI message is specified in [4] clauses D.1.21 and D.2.21. The different FGC SEI message versions are similar but have some standard-specific differences. For example, persistence of the FGC SEI message is specified differently for AVC than it is for HEVC and VVC.

### Interpretation of FGC SEI message syntax

The syntax of the FGC SEI message specified in [2] is shown in Table 1. A guide to interpretation of the syntax in the FGC SEI message is shown in Table 2. Similar interpretations apply to the FGC SEI messages specified in AVC [4] and HEVC [3].

Table 1: Film grain characteristics SEI message syntax

| film_grain_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| fg_characteristics_cancel_flag | u(1) |
| if( !fg_characteristics_cancel_flag ) { |  |
| fg_model_id | u(2) |
| fg_separate_colour_description_present_flag | u(1) |
| if( fg_separate_colour_description_present_flag ) { |  |
| fg_bit_depth_luma_minus8 | u(3) |
| fg_bit_depth_chroma_minus8 | u(3) |
| fg_full_range_flag | u(1) |
| fg_colour_primaries | u(8) |
| fg_transfer_characteristics | u(8) |
| fg_matrix_coeffs | u(8) |
| } |  |
| fg_blending_mode_id | u(2) |
| fg_log2_scale_factor | u(4) |
| for( c = 0; c < 3; c++ ) |  |
| fg_comp_model_present_flag[ c ] | u(1) |
| for( c = 0; c < 3; c++ ) { |  |
| if( fg_comp_model_present_flag[ c ] ) { |  |
| fg_num_intensity_intervals_minus1[ c ] | u(8) |
| fg_num_model_values_minus1[ c ] | u(3) |
| for( i = 0; i <= fg_num_intensity_intervals_minus1[ c ]; i++ ) { |  |
| fg_intensity_interval_lower_bound[ c ][ i ] | u(8) |
| fg_intensity_interval_upper_bound[ c ][ i ] | u(8) |
| for( j = 0; j <= fg_num_model_values_minus1[ c ]; j++ ) |  |
| fg_comp_model_value[ c ][ i ][ j ] | se(v) |
| } |  |
| } |  |
| } | u(1) |
| fg_characteristics_persistence_flag |  |
| } |  |
| } |  |

Table 2: Guide to interpretation of the FGC SEI message syntax

| Parameter | Range | Significance |
| --- | --- | --- |
| fg_model_id | 0,1 | Model to be used in grain synthesis. 0: Frequency filtering, 1: Auto-regression |
| fg_separate_colour_description_present_flag | 0,1 | Defines whether the colour description for the film grain specified is same as for the coded video sequence. |
| fg_blending_mode_id | 0,1 | Blending mode used to combine grain and decoded samples.0: Additive, 1: Multiplicative |
| fg_comp_model_present_flag | 0,1 | Defines the presence of film grain model parameters for each colour component |
| fg_num_intensity_intervals_minus1 | 0-255 | Defines the number of intensity intervals for each colour component |
| fg_num_model_values_minus1 | 0-5 | Specifies the number of component model values available in the SEI ( default values will be used for the remaining component model values) |
| fg_intensity_interval_lower_bound | 0-255 | Lower bound for each intensity intervals for which the model is applicable |
| fg_intensity_interval_upper_bound | 0-255 | Upper bound for each intensity intervals for which the model is applicable |
| fg_comp_model_value |  | Component model values have different meaning depending on the value of fg_film_grain_model_id (see below). |
| fg_characteristics_persistence_flag | 0,1 | Indicates the persistence of the FGC SEI message. |

The component model values specify the strength, shape, density, and other characteristics of the film grain. A unique set of component model values can be signalled for each intensity interval for each colour component to be processed. The value range of each syntax element may be constrained by specific practice or implementation.

When the frequency-filtering film grain model is signalled (film_grain_model_id = 0), comp_model_values are interpreted as follows for each colour component, c, and intensity interval, i.

- comp_model_value[ c ][ i ][ 0 ] : the standard deviation of Gaussian noise

- comp_model_value[ c ][ i ][ 1 ] : horizontal high cutoff frequency

- comp_model_value[ c ][ i ][ 2 ] : vertical high cutoff frequency

- comp_model_value[ c ][ i ][ 3 ] : horizontal low cutoff frequency

- comp_model_value[ c ][ i ][ 4 ] : vertical low cutoff frequency

- comp_model_value[ c ][ i ][ 5 ] : correlation between consecutive colour components

fg_comp_model_value[ c ][ i ][ j ] shall be in the range of 0 to 2fgBitDepth[ c ] − 1, inclusive. The derivation of fgBitDepth[ c ] value is specified in [2].

When the autoregressive film grain model is signalled (film_grain_model_id = 1), comp_model_values are interpreted as follows.

- comp_model_value[ c ][ i ][ 0 ] : the standard deviation of Gaussian noise

- comp_model_value[ c ][ i ][ 1 ] : first order correlation for neighboring samples ( x − 1, y ) and ( x, y − 1 )

- comp_model_value[ c ][ i ][ 2 ] : correlation between consecutive colour components

- comp_model_value[ c ][ i ][ 3 ] : first order correlation for neighboring samples ( x − 1, y − 1 ) and ( x + 1, y – 1 )

- comp_model_value[ c ][ i ][ 4 ] : aspect ratio of the modelled grain

- comp_model_value[ c ][ i ][ 5 ] : second order correlation for neighboring samples ( x - 2, y ) and ( x, y - 2 )

fg_comp_model_value[ c ][ i ][ j ] shall be in the range of −2( fgBitDepth[ c ] − 1 ) to 2( fgBitDepth[ c ] − 1 ) − 1, inclusive. The derivation of fgBitDepth[ c ] value is specified in [2].

The frequency-filtering and autoregressive models for film grain synthesis share the following processing steps:

- Determination of applicable intensity intervals for each sample for each applicable colour component. A different set of FGS model parameters may be signalled in the FGC SEI message for each intensity interval.

- Generation of synthesized grain.

- Blending of synthesized grain and decoded image.

The methods for determining intensity intervals and generating synthesized grain depend on the signalled FGS model. Methods for the frequency-filtering model are described in subclause 7.2. Methods for the autoregressive model are described in subclause 7.3.

fg_comp_model_value[ c ][ i ][ 0 ] together with intensity interval boundaries is used to define a scaling function (piece-wise constant scaling function). The scaling function indicates the level at which the film grain will be perceived in the final output frame.

Additive (fg_blending_mode_id = 0) and multiplicative (fg_blending_mode_id = 1) grain blending methods are specified in [2].

## Alternative film grain metadata

### General

An alternative autoregressive film grain synthesis model can be indicated in the VVC, HEVC, and AVC standards using the ITU-T T.35 registered or unregistered user data SEI messages.  [Ed. Note: Currently this section is intentionally left mostly empty]

Example implementations of film grain synthesis technologies

Annex describes several example implementations of FGC SEI message and film grain analysis and synthesis technologies.

FGC SEI message example implementations

The FGC SEI message rewriter included in the VTM allows FGC SEI message parameter values to be modified in existing bitstreams without re-encoding. It also supports inserting FGC SEI messages in a VVC bitstream originally encoded without any FGC SEI messages. The bitstream with the inserted FGC SEI message has an identical MD5sum value to the bitstream having the FGC SEI message enabled during encoding. The tool supports three operational modes, controlled by SEIFilmGrainOption: 0-no change; 1-FGC SEI removal; 2-FGC SEI insertion; 3-FGC SEI rewriter.

- FGC SEI removal	(applied on bitstreams that include FGC SEI messages)

- FGC SEI insertion	(applied on bitstreams without FGC SEI message)

- FGC SEI modification	(applied on bitstreams with FGC SEI message)

The two typical use cases and examples are given below:

- A bitstream (test_fg1.bit) is encoded with a pre-configured FGC SEI message (fg1.cfg), but the film grain parameters are not producing satisfactory film grain effects. One can use the tool to modify the values (fg2.cfg) of the existing FGC SEI message. Example command line: 
./SEIFilmGrainAppStatic -c fg2.cfg -b test_fg1.bit -o test_fg2.bit --SEIFilmGrainOption=3

- A bitstream (test.bit) is encoded without FGC SEI message, so it cannot take advantage of the film grain synthesis. One can use the tool to easily insert the FGC SEI (fg.cfg) into the bitstream. Example command line:

./SEIFilmGrainAppStatic -c fg.cfg -b test.bit -o test_fg.bit --SEIFilmGrainOption=2

The tool manipulates the VVC bitstream at NALU level, so processing time is extremely fast (e.g., to rewrite the FGC SEI message in a 4K/8K resolution bitstream, the entire processing time is less than 0.1 sec). The basic workflow is illustrated in Figure 17.

Figure 17 Basic workflow of FGC SEI message rewriter

[Ed. Note: Option 3 is missing a colon]

Film grain analysis example implementations

[TO BE COMPLETED]

Film grain synthesis example implementations

[TO BE COMPLETED]