| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0023-v1 |
| --- | --- |

| Title: | EE1: Summary of Exploration Experiments on Neural Network-based Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, F. Galpin, Y. Li, M. Santamaria, H. Wang, L. Wang and Z. Xie | Tel: Email: | elena.alshina@huawei.com franck.galpin@interdigital.com yue.li@bytedance.com maria.santamaria_gomez@nokia.com hongtaow@qti.qualcomm.com liqiangwang@tencent.com xiezhihuang@oppo.com |
| Source: | EE Coordinators |  |  |

_____________________________

# Abstract

This document summarizes Exploration Experiment 1 (EE1) tests performed between the JVET-AA and JVET-AB meetings to evaluate Neural Network-based Video Coding (NNVC) technologies, analyze their performance, and analyze their complexity aspects. In NN-based in-loop filter category BD-rate gain over AhG11 NNVC anchor goes up to 10.5% (depending on complexity). By content adaptive post-filter gain of 5% over AhG11 anchor was demonstrated. In Super Resolution category ~7% BD-rate average for 4K content gain over VTM (with enabled adaptive coded picture resolution) can be achieved.

# Introduction

Group continues evaluation of new promising NN-based video coding technologies, answering questions and addressing suggestions from JVET members made during presentation NN-based technologies at JVET-AA meeting. Additionally, a common reference software is now available as NCS-1.0.

Tests were planned to be conducted in three categories: enhancement filters (in-loop or post), super-resolution and E2E NN-based video coding. Unfortunately, the test in E2E NN-based video coding category has been withdrawn.

EE1 test were actively cross-checked in this EE1 round. Cross-check status is reported in section 5 of this document.

Tests were performed on top of VTM-NNVC v2.0 / NCS 1.0.

BD-rate gain over NNVC anchor in Random Access configuration vs computational complexity (in kMAC/pxl) and memory size for Model parameters (in MB) for typical representatives of tests in this EE1 are shown on Fig. 1 and Fig. 2 respectively.

Fig. 1. Gain vs computational complexity. (a) Y-BD-rate gain in RA configuration vs kMAC/pxl, (b) in-loop filters in mode details.

Fig. 2. Gain vs memory complexity. (a) Y-BD-rate gain in RA configuration vs total number of parameters, (b) in-loop filters in mode details.

# Exploration experiments on Enhancement filters

## NN-filter architecture based on NCS-1.0 filter set #1 (JVET-AA0111)

Tests in this sub-category are based on architecture JVET-AA0111 (one of two NN-based filters in NCS). Filter is referred below as NCS-1.0 filter set #1. NCS-1.0 filter set #1 processes Luma separately, two chrominance components together. Additional inputs to the network: partitioning, prediction, QP map, boundary strength (BS). In total four models are used. The difference between the input samples and the NN filtered samples (residues) are scaled by the scaling factors before being added to input samples.  The number of residual blocks is N=8. Worst case 682 kMAC/pixel (block-basis) and total number of parameters 6.24M

JVET-AB0053 EE1-1.2 “EE1-1.2: NN intra model without attention and partitioning strength”

Test was done using code of JVET-AA0111 (identical to NCS-1.0 filter set #1). Model was converted from pytorch to SADL (still float point), minor change in scaling values samples data and QP to float. Training is slightly different from NCS-1.0 filter set #1 (longer).

Fig. 3. Removal partitioning information from JVET-AA0111 filter design.

Sub-test performed:

EE1-1.2.2 Architecture identical to NCS-1.0 filter set #1, but training is by Ericsson: 0.1 % better in RA, 0.3% better in “All intra”, 0.1% better in LDB configuration. Training strategy by Ericsson works better than training from original proponent for NCS-1.0 filter set #1.

EE1-1.2.1 Removal partitioning information from NN-filter design (Fig. 3). Effect: negligible performance deviation (0.02%), saving 18 kMAC/pxl (~3% of total filter complexity).

JVET-AB0073 EE1-1.4 “EE1-1.4: Deep In-Loop Filter with Additional Input Information”

The addition to NCS-1.0 filter set #1 filter design in this test is collocated blocks from reference frames are fed to the model (FIG. 4). Essentially filtration in temporal domain is enabled by this modification.

In order to limit complexity “temporal filtration” is applied for two highest (test EE1-1.4.1) or three highest (test EE1-1.4.2) temporal hierarchy layers.

Fig. 4. Collocated blocks from reference frame are fed to the NCS-1.0 filter set #1 network.

Performance improvement in RA configuration due to temporal filtration is 0.53% (two highest layers involved) and 0.63% (two highest layers involved). Total number of parameters increases 6.24M 7.8M) to handle collocated blocks processing.

JVET-AB0052 EE1-1.5 “EE1-1.5: One luma model with IPB and/or skip for filtering intra and inter luma slices”

This test studies modification of NCS-1.0 filter set #1 (which uses four models). In this test only three models are used. This modification is tested in EE1-1.5.4 (~0.3% loss is observed in RA and LDB). To compensate the drop, the following modification is tested: the tested models use block type information IPB (intra/uni-predicted/bi-predicted) and/or block skip information (bypassed or not) as additional input(s) on top of the inter luma model from

- NCS-1.0 filter set #1 – natural anchor for this test

  - Worst case 682 kMAC/pixel (block-basis) and num. of para. 6.24M

- Test 1.5.1: Test the IPB+skip model used for intra luma and inter luma slices (as proposed in JVET-AA0090)

  - Worst case 696 kMAC/pix and num. of para. 4.70M

  - Vs. NCS-1.0 filter set #1 BDR-Y: -0.17% RA, 0.13% LDB, and -0.13% AI.

- Test 1.5.2: Test the IPB model used for intra luma and inter luma slices.

  - Worst case 682 kMAC/pix and num. of para. 4.68M

  - Vs. NCS-1.0 filter set #1 BDR-Y: -0.11% RA, 0.29% LDB, and -0.16% AI.

- Test 1.5.3: Test the skip model used for intra luma and inter luma slices.

  - Worst case 682 kMAC/pix and num. of para. 4.68M

  - Vs. NCS-1.0 filter set #1 BDR-Y: 0.04% RA, 0.20% LDB, and -0.12% AI.

- Test 1.5.4: Test the original inter luma model from Vs. NCS-1.0 filter set #1 used for intra luma and inter luma slices.

  - Worst case 668 kMAC/pix and num. of para. 4.67M

  - Vs. NCS-1.0 filter set #1, BDR-Y: 0.30% RA, 0.33% LDB, and -0.09% AI

JVET-AB0068 EE1-1.6 “EE1-1.6: RDO Considering Deep In-Loop Filtering”

Only encoder change for NCS-1.0 filter set #1 (which has block level on/off decision for NN-based filter). Filtering consideration is added to RDO process in this test.  Encoding run time increases ~10% in motion compensation scenarios and ~30% for all intra configuration.

Y-BD-rate vs. NCS-1.0 filter set #1: BDR-Y: -0.38% RA (Enc. 108%), -0.33% AI (Enc. 129%).

## NN-filter architecture based on NCS-1.0 filter set #0 (JVET-AA0088)

Tests in this sub-category are based on architecture JVET-AA00088 (one of two NN-based filters in NCS). Filter is referred below as NCS-1.0 filter set #0.

JVET-AB0083 EE1 1.8 “EE1-1.8: More refinements on NN based in-loop filter with a single model”

Position in processing pipeline: reconstructed image after LMCS is fed into the network. Then, the output of NN filter is processed by ALF and CCALF. Content adaptation: weighted sum of CNN filter and “conventional” (de-blocking and SAO) filter output (there possible weights 1, 4/3 or 1/2), residual scaling (same as NCS-1.0 filter set #1) and QP adjustment (which was the goal of the test in this EE1 round).

Fig. 5 Illustration of NN-based in-loop filter in test EE1-1.8 (NCS-1.0 filter set #0 family).

Filter architecture is shown on Fig. 5. Three color components coded together, Base QP, Slice QP and slice type are extra input to the NN-module, One down-sampling CONV, N=32 Residual Blocks (RB), PixelShuffle. In Residual Block number of channels increases from 64 to 160 and reduced after activation layer. Scaling factor (adjusts the strength of the filter) is signalled in slide header.

NN-model is quantized to 16 bits integer, only one model is used. Implementation is block-based, requires 615kMAC/pxl. If implemented at picture level, complexity is 485 kMAC/pxl.

Two sub-tests have been conducted:

Test 1.8.1: the QP adjustment method as it was proposed in JVET-AA0089.

Test 1.8.2: fine tuning of the model from NCS-1.0 filter set #0 based on Test 1.8.1.

In JVET-AA0089, a QP-adjustment method is proposed to add a small offset to the BaseQP. The offset candidates can be -5 and 5. Then, multiple results are inferred by the common one model but with the different inputs {BaseQP, BaseQP - 5, BaseQP + 5} into the BaseQP interface of the network, where the selected index is signaled at the slice-level.

On top of NCS-1.0 filter set #0, the QP adjustment method shows 0.21% (8.71% to 8.92%) luma gain (RA) with 11% (139% to 150%) runtime increment at encoder.

Based on Test 1.8.1, fine tuning improves performance by 0.26% (8.92% to 9.18%) luma gain (RA) / 0.1% luma gain (All Intra).

The corresponding complexity/ performance plots are shown below (Figs. 6). It seems that EE1-1.8.2 achieves a better trade-off, especially for the trade-off between total number parameters/ performance.

Fig. 6 Complexity performance analysis of NCS filter set 0 family tests.

## NN-filter architecture not based on NCS-1.0

JVET-AB0054 EE1-1.3 “EE1-1.3: CNN Based In-Loop Filter with WCDANN”

Filter architecture called WCDANN shown on Fig. 7. Instead of Residual Blocks which are typically used in enhancement filters WCDAB blocks are used. Number of WCDAB blocks is 4. Similar to NCS-1.0 filter set #0, all three color components are processed together. Prediction, partitioning are additional inputs to the network. Only “all Intra” configuration was tested.

In test EE1-1.3.1 “QP map is used as input, single model is used (total number of parameters in 1.3M), but in EE1-1.3.2 five models trained for different QP are used (total number of parameters is 6.5M).

Test results show that ×5 larger number of parameters and switchable models give 0.22% Y-BD rate gain. This complexity is unlikely to be justified by demonstrated moderate gain.

Fig. 7 Network architecture of the proposed Lighter WCDANN. The parameter M is set to 4 (EE1-1.3).

Next series of tests studies effect of weights between Luma and Chroma distortion in Loss function during training. Allocation of higher weights for Luma distortion “moves” gain from Chroma to Luma. Strangely {8,1,1} improves Luma gain with small reduction of Chroma performance, but {10,1,1} already has less Y BD-rate gain compared to {8,1,1}

| Weights {Y,U,V} | BD-rate vs AhG11 anchor (all intra cfg) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| {6,1,1} | -4.36% | -12.59% | -11.95% |
| {8,1,1} | -4.78% | -10.95% | -10.24% |
| {10,1,1} | -4.38% | -11.11% | -9.20% |

From this study weights [8,1,1} in training can be recommended for better Luma/Chroma compression performance balance in all Intra configuration.

JVET-AB0164 EE1 1.7 “EE1-1.7: Capacity Ablation of CNN-based in-loop filtering”

This test tries to compromise between NCS-1.0 filter set #0 and NCS-1.0 filter set #1, re-using elements of both. Filter architecture is shown on FIG. 8. Additionally to reconstructed signal, prediction, partitioning, boundary strength (BS) are used. Luma and Chroma are processed separately. The goal of this study is to check performance difference between pytorch and SADL implementation. The difference on performance of float point implementation in pytorch and SADL is very low (within 0.05% Y-BD-rate in “all intra” configuration). Similary very little performance difference is observed between performance of model with float32 and quantized to int16 parameters.

Fig. 8 NN-filter architecture of EE1-1.7

More interesting test was performed for different number of residual blocks in NN-based filter used for Luma and Chroma enhancement. The higher number of residual blocks the better performance, but, obviously the higher the complexity.

| N residual Blocks |  | Total # params, M | kMAC/pxl | BD-rate vs AhG 11 anchor, RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Luma | Chroma |  |  | Y | U | V |
| 32 | 32 | 7.6 | 619.0 | -10.5% | -22.5% | -24.0% |
| 24 | 16 | 4.9 | 450.0 | -9.9% | -19.5% | -19.3% |
| 16 | 16 | 4.0 | 337.0 | -9.2% | -19.6% | -19.4% |
| NCS filter set #0 (N=32) |  | 1.9 | 485.0 | -8.7% | -18.2% | -18.9% |
| NCS filter set #1 (N=8) |  | 6.24 | 539.0 | -9.4% | -20.7% | -20.4% |

Looking at complexity/ performance plots (Fig. 9), EE1-1.7 test (16, 16) is in between the two NCS filter in BD-rate vs total number of parameters graph, and shows clearly better performance complexity trade-off on BD-rate vs kMAC/pxl graph.

Fig. 9. Comparison of EE1-1.7 tests with NCS filters.

There is no cross-check for this test, only all intra and partially RA test results are available for quantized model, but this test looks like very promising candidate for unified NN-based filter design. Can be recommended for training stage cross-check in the next meeting circle.

## NN-filter based post-filters

JVET-AB0048 EE1-1.1 “EE1-1.1: Content-adaptive post-filter with SADL inference and signalling of NN post-filter characteristics and activation SEI messages”

This test is interesting, since it uses post-filter characteristics and activation SEI messages in JVET-Z0244. The inference was done using int16 precision and SADL library. Content adaptively is achieved by overfitting multiplier m  in convolution:

σW*x+b∙m,
where W is the kernel, * is the convolution operator, x is the input, b is the bias, m is the multiplier and σ is the activation function.

Overfitting requires following time (depending on video resolution)

A1 & A2: 13 hours
B: 6 hours; C: 2 hours

D: 45 mins; F: 4.5 hours

Filter architecture is as follows

Fig. 10. Post filter EE1-1.1 architecture.

It is reported that the BD-rate gain over NNVC 2.0 is 5.0% (Y), 20% (U), 17% (V) in RA configuration.

# NN-based super-resolution

Additional reference for tests in this category is JVET-Z0065 (by LGE) in which scaling factor for coded video was adaptively selected between 1.0 (full size coding), 2.0 (coding at quarter resolution) and 1.5 (coding of down sampled by ratio 1.5 video). Selection of scaling factor was performed at GOP basis, RPR resampling filters were used (no neural network based coding is involved.). Thanks to the wise strategy of scaling factor selection, this test shows gain in average over AhG11 anchor. This test is listed in EE1 summary table as additional reference.

For some tests in this category results for 4K resolutions only available. The comparison of test results for 4K only sequences (all intra cfg)

is shown in table below.

| SR techniques | Y-BD-rate | U-BD-rate | V-BD-rate | Total Params, 106 | kMAC/pxl | 10% rate matching |
| --- | --- | --- | --- | --- | --- | --- |
| RPR (JVET-Z0065) | -2.18% | 7.52% | 6.15% | 0 | 0 | YES |
| EE1-2.1 | -9.24% | 8.82% | -16.39% | 12.5 | 854 | NO |
| EE1-2.2 | -8.50% | 18.77% | -12.61% | 31.0 | 2044 | NO |
| EE1-2.3 | -3.47% | 0.61% | -0.15% | 4.5 | 469 | YES |

JVET-AB0076 EE1 2.1 “EE1-2.1: RPR-Based Super-Resolution Guided by Partition Information”

Unfortunately, for this test:

- no decoding run time provided,

- results shown only for 4K sequences,

- only “all intra” configuration was tested,

- bit-rates are significantly different from anchor (BD-rate is not very reliable).

NN-based processing operates after RPR up-sampling (de facto works as enhancement filter, trained to reduce RPR artifacts).

For “all intra configuration” 9.2% Y-BD-rate gain over NNVC2.0 anchor is reported. In comparison 2.2% can be achieved by adaptive resolution selection and RPR re-sampling (JVET-Z0065).

The proposed network is composed of four parts: feature extraction, reference information generation, mutual information processing and reconstruction (Fig. 11). Specifically, the feature extraction part consists of three convolutional layers, which are used to extract features of the inputs. The convolutional layer is followed by a ReLU activation function. The inputs are the predicted frame and chroma component after RPR up-sampling.

Fig. 11 Illustration of the proposed network architecture for Super-resolution guided by partition information (EE1-2.1).

The reference information generation (RIG) part consists of eight residual blocks. The first four blocks are performed for predicting the CTU partition information from reconstructed frame with an extra block, while the last three blocks are used for reference information generation. Sequentially, reference information features are input to several convolutional layer sets to generate different scales features as the input of the reference feature attention module. Each layer set consists of a convolutional layer with stride 2 and a convolutional layer followed by one ReLU.

JVET-AB0077 EE1 2.2 “EE1-2.2: CNN Filter for Super-Resolution with RPR functionality in VVC”

In opposite to previous test (EE1-2.1) no RPR resampling is used here, reconstructed signal up-sampled using NN-based algorithm, called MMSDANet (Fig. 12). Y component is processed w/o extra information. U and V channels are up-sampled using all three low resolution components Y, U and V as inputs.

MMSDANet consists of 8 MMSDABs, three convolutional layers, one concatenate layer, one shuffle layer and five shortcut connections. Except for the 1x1 and last convolutional layers, all convolutional layers and MMSDABs have 64 input and output channels. 1x1 convolution is used to reduce the number of channels and reduce the complexity of the operation. The output channel of the last convolutional layer is 4 so that the pixel shuffle layer can do 2x up-sampling. Since the input and output resolutions are different, the input needs to be up-sampled when applying global connections.

Y channel

U channel

V channel

Fig. 12 Illustration of the proposed MMSDANet in EE1-2.2.

JVET-AB0084 EE1 2.3 “A CNN-based Super Resolution Method with GOP Level Adaptive Resolution”

In this test similarly to JVET-Z0065 scaling factor for coded video was adaptively selected between 1.0 (full size coding), 2.0 (coding at quarter resolution) and 1.5 (coding of down sampled by ratio 1.5 video). Selection of scaling factor was performed at GOP basis. Additionally to JVET-Z0065 NN-based enhancement is used to improve RPR up-sampling.

NN-based re-sampler fed (for luma) with low resolution reconstructed signal, prediction, base QP and slice QP information (for luma). UV component are up-sampled jointly and use Luma low resolution signal data as extra input to NN-based up-sampler.

for Luma

for Chroma

Fig. 13 The super resolution network architecture in EE1-2.3.

# End-to-End AI video coding

Test EE1-3.1 was withdrawn by proponent.

# Cross-check status

Performance deviation was observed in almost all cases (even when CPU inference is used). Several reasons for performance deviation were identified: computing environment difference, GCC compiler version difference, float point operations.

| Test | proposal | tester | Cross-check | Cross-checker | Comment |
| --- | --- | --- | --- | --- | --- |
| EE1-1.2 | JVET-AB0053 | Ericsson | JVET-AB0063 | Nokia | Training and inference were crosschecked; training time reduction 14% is confirmed for proposed simplification. Performance difference up to 0.1%, computing environments are different |
| EE1-1.5 | JVET-AB0052 | Ericsson | JVET-AB0089 | Tencent | full test results are not an exact match due to different GCC versions |
| EE1-1.6 | JVET-AB0068 | Bytedance | JVET-AB0137 | Ericsson | Training and inference were crosschecked, training by cross-checker was faster than reported by proponent. SW was modified (with help from proponent) to be able to verify training Performance difference is 0.0x% (both RA and LDB cfg). |
| EE1-1.8 | JVET-AB0083 | Tencent/OPPO | JVET-AB0088 | Ericsson | To match performance exactly same GCC complier shall be used (uses GCC 7.4.0 for the full simulation results and GCC 8.2.0 for class B under LDB configuration) |
| EE1-2.1 | JVET-AB0076 | Xidian/OPPO | JVET-AB0105 | Tencent | Results match proponent ones. Small differences in RA (likely due to float inference). |
| EE1-2.2 | JVET-AB0077 | Xidian/OPPO | JVET-AB0106 | Tencent | Results match proponent ones. Small differences in RA (likely due to float inference). |
| EE1-2.3 | JVET-AB0084 | Tencent/LGE | JVET-AB0097 | Ericsson | Results match to proponent for all intra, for RA 0.1% better than reported by proponent |
| EE1-3 | withdrawn | Xidian/OPPO |  | Dolby | Bug was discovered, to be fixed |

# Conclusions

- Training

  - Training was cross-checked for tests EE1-1.2 and EE1-1.6.

  - Training strategy from EE1-1.2.2 (JVET-AB0053) is more successful for NCS-1.0 filter set #1 than one in NCS.

  - Weights {8,1,1} for {Y,U,V} distortion in loss-function was found in EE1-1.3.2 (JVET-AB0090) to provide best balance for Luma / Chroma performance.

- Encoder only modification

  - Additional 0.4% / 0.3% gain (RA/LDB) with 8% / 29% encoding run time increase can be achieved if NN-based filter is considered in RDO (EE1-1.6) (JVET-AB0068)

- NN-filter design

  - Removing partitioning information from NCS-1.0 filter set #1 filter design (EE1-1.2.1 (JVET-AB0053) helps reduction of training time 14%, computation complexity reduction and comes w/o performance degradation.

  - Gain of 0.5~0.6% in RA can be achieved if NCS-1.0 filter set #1 extended to temporal filtration    (EE1-1.4.1, 1.4.2) (JVET-AB0073)

  - EE1-1.7 JVET-AB0164 shows promising performance complexity trade-off, can be recommended for training verification in the next EE1 round.

  - EE1-1.8.2 JVET-AB0083 (which is improvement of NCS-1.0 filter set #0) shows 0.5% better performance NCS-1.0 filter set #0 having the same decoder complexity

- Post-filter SEI

  - Successful usage of the post-filter characteristics and activation SEI messages was demonstrated in test EE1-1.1, 5% (Y) gain in RA can be achieved, extra time required for overfitting is comparable with one Intra Period Segment coding in RA configuration.

# Recommendation

- Block base implementation of NN-based filters is more realistic, it is recommended to refine AhG 11 complexity assessment methodology and report kMAC/pxl per block (all EE1 proponents are doing so already).

- Discuss performance deviation between cross-checker and proponent, ideally specify maximum allowed performance deviation to be used for future NNVC work in JVET.