| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0024-v1 |
| --- | --- |

| Title: | EE2: Summary Report on Enhanced Compression beyond VVC capability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Guichun Li Karam Naser Jacob Ström Martin Winken Xiaoyu Xiu Kai Zhang | Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com guichunli@global.tencent.com Karam.Naser@InterDigital.com jacob.strom@ericsson.com martin.winken@hhi.fraunhofer.de xiaoyuxiu@kwai.com zhangkai.video@bytedance.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document provides a summary report of Exploration Experiment on Enhanced Compression beyond VVC capability. The tests are categorized as intra prediction, inter prediction, screen content coding, transform, and in-loop filtering.

The software basis for this EE is ECM-6.0, released at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-6.0. ECM-6.0 is used as an anchor in the tests.

Software for EE tests is released in the corresponding branches at https://vcgit.hhi.fraunhofer.de/ecm/jvet-aa-ee2/ECM/-/branches.

Test results can be found in input JVET contributions, cross-check results are uploaded to https://vcgit.hhi.fraunhofer.de/ecm/jvet-aa-ee2/simulation-results if cross-check reports are not submitted as they are optional for EE tests.

# List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Intra prediction |  |  |  |
| 1.1 | Reduced complexity spatial GPM | InterDigital K. Naser JVET-AB0129 | Qualcomm B. Ray JVET-AB0212 |
| 1.2 | Reducing the number of GPM partition types and intra mode derivation | Ittiam J. Arumugam Dolby T. Lu JVET-AB0163 | OPPO F. Wang JVET-AB0218 |
| 1.3 | Adaptive blending for spatial GPM | OPPO F. Wang JVET-AB0154 | Dolby T. Lu JVET-AB0221 |
| 1.4 | Smaller and bigger block sizes for spatial GPM | OPPO F. Wang JVET-AB0154 | Dolby T. Lu JVET-AB0221 |
| 1.5 | Test 1.3 + Test 1.4 | OPPO F. Wang JVET-AB0154 | Dolby T. Lu JVET-AB0221 |
| 1.6 | Combination of spatial GPM tests 1.6a: Test 1.5 + Test 1.2 1.6b: Test 1.6a + Test 1.1 | InterDigital K. Naser Ittiam J. Arumugam Dolby T. Lu OPPO F. Wang JVET-AB0155 | Alibaba R.-L. Liao JVET-AB0208 Qualcomm B. Ray JVET-AB0212 KDDI Y. Kidani JVET-AB0240 |
| 1.7a | CCLM with non-linear term | Alibaba X. Li JVET-AB0091 | OPPO L. Xu JVET-AB0196 |
| 1.7b | CCLM with non-linear term as an additional mode | Alibaba X. Li JVET-AB0091 | OPPO L. Xu JVET-AB0196 |
| 1.7c | CCLM with non-linear term with CCCM parameter derivation | Alibaba X. Li JVET-AB0091 | OPPO L. Xu JVET-AB0196 |
| 1.8a | Gradient linear model with luma value | Alibaba X. Li JVET-AB0092 | Kwai C. -W. Kuo JVET-AB0227 OPPO F. Wang JVET-AB0247 |
| 1.8b | Gradient linear model with luma value as additional modes | Alibaba X. Li JVET-AB0092 | Kwai C. -W. Kuo JVET-AB0227 OPPO F. Wang JVET-AB0247 |
| 1.9 | Self-aware filter estimation for CCLM | Bytedance K. Zhang JVET-AB0169 | Alibaba X. Li |
| 1.10 | Template-based multiple reference line intra prediction | OPPO L. Xu JVET-AB0156 | Alibaba X. Li JVET-AB0199 |
| 1.11a | Intra prediction fusion | Qualcomm H. Wang JVET-AB0148 | Xiaomi F. Le Léannec JVET-AB0216 |
| 1.11b | Intra reference fusion | Qualcomm H. Wang JVET-AB0148 | Xiaomi F. Le Léannec JVET-AB0216 |
| 1.12a | Test 1.10 and Test 1.11a | OPPO L. Xu Qualcomm H. Wang JVET-AB0157 | Alibaba X. Li JVET-AB0200 |
| 1.12b | Test 1.10 and Test 1.11b | OPPO L. Xu Qualcomm H. Wang JVET-AB0157 | Alibaba X. Li JVET-AB0200 |
| 1.13a | Template selection scheme for CCCM modes | Qualcomm Y.-J. Chang JVET-AB0143 | Nokia J. Lainema |
| 1.13b | Test 1.13a and CCCM fusion instead of MMLM fusion | Qualcomm Y.-J. Chang JVET-AB0143 | Nokia J. Lainema |
| 1.14 | IntraTMP adaptation for camera-captured content | InterDigital K. Naser JVET-AB0130 | Ofino D. Ruiz Coll JVET-AB0211 |
| 1.15 | Horizontal and vertical planar modes | Alibaba X. Li JVET-AB0127 | WILUS K. Kim JVET-AB0197 |
| 1.16 | Picture-level geometry transform | Bytedance W. Jia JVET-AB0165 | Xiaomi P. Andrivon |
| 2 Inter prediction |  |  |  |
| 2.1a | AmvpMerge for the low-delay picture | LGE H. Jang JVET-AB0078 | ETRI W. Lim |
| 2.1b | Test 2.1a without template matching-based MV refinement | LGE H. Jang JVET-AB0078 | ETRI W. Lim |
| 2.2 | Template matching based BCW index derivation for merge mode | Alibaba R.-L. Liao JVET-AB0079 | OPPO K. Sato JVET-AB0224 Qualcomm Z. Zhang |
| 2.3 | POC based BCW weights derivation | Qualcomm Z. Zhang JVET-AB0124 | Alibaba R. -L. Liao JVET-AB0209 |
| 2.4a | Test 2.2 + Test 2.3 with removing negative BCW weights | Alibaba R.-L. Liao Qualcomm Z. Zhang JVET-AB0140 | Kwai W. Chen |
| 2.4b | Test 2.2 + Test 2.3 with negative BCW weights | Alibaba R.-L. Liao Qualcomm Z. Zhang JVET-AB0140 | Kwai W. Chen |
| 2.5a | Enhanced temporal motion information derivation | Bytedance L. Zhao JVET-AB0118 | Tencent L.-F. Chen JVET-AB0150 |
| 2.5b | Enhanced temporal motion information derivation without template matching for SbTMVP | Bytedance L. Zhao | withdrawn |
| 2.6 | DMVR for affine merge coded blocks | Alibaba J. Chen JVET-AB0112 | Bytedance B. Vishwanath JVET-AB0226 |
| 2.7 | Extended weights for MHP | OPPO K. Sato JVET-AB0153 | ETRI D. Kim JVET-AB0235 |
| 3 Screen content coding |  |  |  |
| 3.1a | IntraTMP for chroma component | InterDigital K. Naser JVET-AB0131 | Xiaomi F. Le Léannec JVET-AB0213 |
| 3.1b | IntraTMP for chroma components using luma block vector | InterDigital K. Naser | withdrawn |
| 3.2 | Using block vector derived from IntraTMP for IBC | ETRI W. Lim JVET-AB0061 | Xiaomi F. Le Léannec JVET-AB0214 |
| 3.3 | Using luma and chroma block vectors derived from IntraTMP for IBC | InterDigital K. Naser ETRI W. Lim JVET-AB0132 | Xiaomi F. Le Léannec JVET-AB0215 |
| 4 Transform |  |  |  |
| 4.1 | Modification of LFNST for MIP coded blocks | Xidian MMC Lab. J.-Y. Huo W.-H. Qiao JVET-AB0067 | Alibaba X. Li JVET-AB0198 |
| 5 In-loop filtering |  |  |  |
| 5.1 | Extended fixed-filter-output based taps for ALF | Bytedance W. Yin JVET-AB0184 | OPPO L. Xu JVET-AB0195 |

# Description of tests

## Intra prediction

### Test 1.1~1.6: spatial GPM tests

These tests are based on the spatial GPM mode described in JVET-AA EE2-1.4 (JVET-AA0118), with the original algorithm as follows: a candidate list is built which includes partition split and two intra prediction modes shown in Figure 1. 26 partition modes and up to 11 MPMs of intra prediction modes are used to form the combinations, the length of the candidate list is set equal to 16. The selected candidate index is signalled.

Figure 1. Spatial GPM candidates

The list is reordered using template shown in Figure 2. GPM blending process is not used in the template, and SAD between the prediction and reconstruction of the template is used for ordering.

Figure 2. GPM template

The SGPM mode is applied to blocks whose width and height meet the same restrictions as in inter GPM.

On top of this base algorithm, Tests 1.1 through to 1.6 test various aspects as summarised in the below table:

|  | Partition modes | Intra prediction modes | Template size | Extended block size | Adaptive blending |
| --- | --- | --- | --- | --- | --- |
| Test 1.1 | 26 predefined modes | IPM list with TIMD | 4 | Yes | No |
| Test 1.2 | Adaptive derivation algorithm | IPM list without TIMD | 1 | Yes | Yes |
| Test 1.3 | 26 predefined modes | MPM list | 4 | No | Yes |
| Test 1.4 | 26 predefined modes | MPM list | 4 | Yes | No |
| Test 1.5 | 26 predefined modes | MPM list | 4 | Yes | Yes |
| Test 1.6a | 26 predefined modes | IPM list without TIMD | 1 | Yes | Yes |
| Test 1.6b | 26 predefined modes | IPM list with TIMD | 1 | Yes | Yes |

In the Tests 1.1 to 1.6, the following modifications are considered:

- Spatial GPM partition modes:

26 predefined modes

Adaptive derivation algorithm based on the horizontal and vertical gradients ratio

- Intra prediction mode selection:

IPM list with and without TIMD:

For each partition mode, an IPM list is derived for each part using intra-inter GPM list derivation. The IPM list size is 3. In the list, TIMD derived mode is replaced by 2 derived modes with horizontal and vertical orientations (using top or left templates) or TIMD derived mode is excluded.

MPM list:

A uniform MPM list, up to 11 elements, is used for all partition modes.

- Template size (left and above): 1 or 4

- Extended block size:

Spatial GPM is extended to be further applied to 4x8, 8x4, 4x16 and 16x4 blocks, which can be described as 4<=width<=64, 4<=height<=64, width<height*8, height<width*8, width*height>=32.

- Adaptive blending:

Adaptive blending is tested for spatial GPM, where blending depth τ shown in Figure 3 is derived as follows:

- If min(width, height)==4, 1/2 τ is selected

- else if min(width, height)==8, τ is selected

- else if min(width, height)==16, 2 τ is selected

- else if min(width, height)==32, 4 τ is selected

- else, 8 τ is selected

Figure 3. GPM blending

All the Tests 1.1~1.6 do not add additional RDO in the encoder side.

### Test 1.7: CCLM with non-linear term

In ECM-6.0, CCLM model can be represented as follows:

predCi,j= α∙recL'i,j+β,

where predCi,j represents the predicted value of a chroma sample, recL'i,j represents the reconstructed value of the collocated luma samples which are down-sampled for the case of non-4:4:4 color format and the linear model parameters α and β are derived based on one row and column reconstructed samples that are adjacent to the current block by the linear minimum mean square error (LMMSE) method.

In the test, a non-linear term of the reconstructed luma sample value into the CCLM method, so that the chroma sample can be predicted by a non-linear model as:

predCi,j= α0∙(((recL'i,j)2+midValue)≫bitDepth)+α1∙recL'i,j+α2∙midValue,

where the model parameters α0, α1 and α2 are derived from the reconstructed values of the top 2 rows and left 2 columns adjacent samples based on the LDL decomposition method used in CCCM.

In Test 1.7a, the non-linear model is applied to all the 6 CCLM modes when slope adjustment factor is equal to 0 without any additional signaling.

In Test 1.7b, the non-linear model is signaled as an additional mode. When one of the 6 CCLM mode is enabled to the current block, an extra flag is signaled to indicate whether to use the non-linear model.

In Test 1.7c, based on Test 1.7b, the CCCM parameter derivation method is used when the non-linear model is selected, that is 6 rows and columns adjacent samples are used to derive the model parameters.

### Test 1.8: Gradient linear model with luma value

In ECM-6.0, GLM utilizes the gradient of luma samples to predict a chroma sample as:

predCi,j=α∙Gi,j+β,#

where predCi,j represents the predicted value of a chroma sample, Gi,j represents the gradient of the corresponding reconstructed luma samples, and the linear model parameters α and β are derived by adjacent reconstructed samples based on the linear minimum mean square error (LMMSE) method as CCLM.

In the tests, a new GLM mode is evaluated that a chroma sample is predicted based on both the gradient Gi,j of luma samples and the reconstructed value recL'i,j of the down-sampled luma sample with different parameters:

predCi,j=α0∙Gi,j+α1∙recL'i,j+α2∙midValue,

where the model parameters α0, α1 and α2 are derived from 6 rows and columns adjacent samples based on the LDL decomposition method as the CCCM mode in ECM-6.0.

For signalling, one flag is signaled to indicate whether GLM is enabled to both Cb and Cr components, and the syntax element that indicates the gradient pattern is coded by truncated unary code.

In Test 1.8a, the original GLM mode is replaced by the tested GLM mode.

In Test 1.8b, the original GLM mode is reserved and the new GLM mode is signalled as an additional mode by signaling one extra flag in the bitstream.

### Test 1.9: Self-aware filter estimation for CCLM

In the test, N luma downsampling candidates are predefined for CCLM mode. For each downsampling candidate, SAD cost is calculated between the reconstructed neighbors (1 above row and 1 left column) and their corresponding predictor derived by using CCLM with the downsampling candidate. The filter candidate with the least SAD cost is selected as the down-sampling filter to perform the CCLM prediction for the current block as shown in Figure 4.

Figure 4. Self-aware filter estimation for CCLM

In the test, number of the downsampling filters is set to 32 and is specified in the below table.

| F0 | F1 | F2 | F3 | F4 | F5 | F6 | F7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 14011011 | 12011000 | 12001001 | 12000011 | 1201-101-1 | 14121-1-2-1 | 120100-10 | 1200100-1 |
| F8 | F9 | F10 | F11 | F12 | F13 | F14 | F15 |
| 010000 | 001000 | 000010 | 000001 | 100000 | 000100 | 14121000 | 14000121 |
| F16 | F17 | F18 | F19 | F20 | F21 | F22 | F23 |
| 12-110-110 | 12-101-101 | 1201-101-1 | 14363-1-2-1 | 1201000-1 | 120010-10 | 12010-100 | 121000-10 |
| F24 | F25 | F26 | F27 | F28 | F29 | F30 | F31 |
| -12-1-12-1 | 001-100 | 10000-1 | 14 101101 | 1101-10 | 0110-11 | 101010 | 010101 |

### Test 1.10: Template-based multiple reference line intra prediction

In template-based multiple reference line intra prediction, instead of signalling the reference line and the intra mode directly, an index to the candidate list is coded to indicate which combination of the reference line and prediction mode is used for coding the current block, a truncated Golomb-Rice coding with a divisor 4 is employed to code selected combinations from the combination list.

The list of 20 candidates is constructed by combining an MPM with the reference line {1, 3, 5, 7, 12}.

The MPM list construction is modified comparing to the regular intra MPM as follows:

- PLANAR mode is excluded from the intra-prediction-mode candidate list

- DC mode is added after the 5 neighboring modes and DIMD modes

- The delta angles from ±1 to ±4 added to the already included to the list angular modes.

There are 5x10 = 50, which are sorted in the ascending order by SAD cost in the template area shown in Figure 5. Since the extended reference line starts from reference line 1, the area covered by reference line 0 is used for the template cost calculation. The 20 combinations with the least SAD cost form the candidate list.

Figure 5. Template area

### Test 1.11: Intra prediction fusion

In this test, intra prediction is formed by fusion intra prediction derived from different reference lines as follows:

- For angular intra prediction modes including the single mode case of TIMD and DIMD, the proposed method derives intra prediction by weighting intra predictions obtained from multiple reference lines represented as pfusion=w0pline+w1pline+1, where pline is the intra prediction from the default reference line and pline+1 is the prediction from the line above the default reference line. The weights are set as w0=3/4 and w1=1/4.

- For TIMD mode with blending, pline is used for the 1st mode (w0=1,w1=0) and pline+1 is used for the 2nd mode (w0=0, w1=1).

- For DIMD mode with blending, the number of predictors selected for a weighted average is increased from 3 to 6.

Intra prediction fusion is applied to luma blocks when angular intra mode has non-integer slope (required reference samples interpolation) and the block size is greater than 16, it is used with MRL and not applied for ISP coded blocks. PDPC is applied for the intra prediction mode using the closest to the current block reference line.

In Test 1.11b, intra prediction fusion is performed on reference lines instead of prediction blocks. Two reference lines (referred to as rline and rline+1) are used for intra prediction fusion. DeltaInt of the corresponding intra prediction angle is considered in the fusion process. Each value in the fused reference line (rfusion[i]) is derived from:

rfusioni= 3⋅rlinei+rline+1i+DeltaInt≫2

### Test 1.12: Combination of Test 1.10 and Test 1.11

The combinations of intra MRL related tests are evaluated.

Test 1.12a: Test 1.10 + Test 1.11a

Test 1.12b: Test 1.10 + Test 1.12b

### Test 1.13: Template selection scheme for CCCM modes

In ECM, there are 3 types of neighboring samples used for CCLM mode derivation: top only, left only, and top and left. The predictors of non-CCLM intra chroma modes, i.e., DM, DIMD, and 4 default modes, can be fused with the predictors of MMLM.

In the tests, two changes are evaluated:

- Extend the CCLM template selection to CCCM to use left, top or top and left templates for model derivation.

- Existed in ECM MMLM fusion is replaced with multi-model CCCM fusion. More specifically, the predictors of a non-CCLM chroma mode can be fused with the predictors of multi-model CCCM mode. The derivation of fusion weights is kept unchanged.

In Test 1.13a, the first change is evaluated. In Test 1.13b, both changes are evaluated together.

### Test 1.14: IntraTMP adaptation for camera-captured content

In the test, IntraTMP is enabled for camera-captured content with the speedup method applied, where the search area is sub-sampled by a factor of 2, which reduces the template matching search by a factor of 4. After finding the best match, a second refinement process is performed in which another template matching search is performed around the best match with a reduced search range defined as min(width, height)/2 of the current block.

### Test 1.15: Horizontal and vertical planar modes

In ECM, planar mode predicts the current sample from the reconstructed values of 4 reference samples as shown in Figure 6. Specifically, linear interpolation in the horizontal direction and vertical direction are performed respectively, and the two results are averaged to obtain the predicted sample, as shown in the following equations:

predVx,y=H-1-y*recx,-1+y+1*rec-1,H≪log2W

predHx,y=W-1-x*rec-1,y+x+1*recW,-1≪log2H

predx,y=predVx,y+predHx,y+W*H≫(log2W+log2H+1)

Figure 6. The reference samples used in planar mode

In Test 1.15, two additional planar mode: horizontal planar mode and vertical planar mode are tested.

For horizontal planar mode, only the horizontal linear interpolation is performed based on the left reference sample and the top-right reference sample to predict the current sample as:

predx,y=W-1-x*rec-1,y+x+1*recW,-1+(W≫1)≫log2W

For vertical planar mode, only the vertical linear interpolation is performed based on the above reference sample and the bottom-left reference sample to predict the current sample as:

predx,y=H-1-y*recx,-1+y+1*rec-1,H+(H≫1)≫log2H

Two additional planar modes are only applied to the luma component and are not used for ISP coded blocks. When the current block enables one of the two proposed planar modes, the block's propagation mode is set to the original planar mode.

### Test 1.16: Picture-level geometry transform

In the test, three geometry transforms are supported at picture level: horizontal flip, vertical flip, and 180°-rotation. It is signaled in the picture header to indicate whether the geometry transform is applied, and which transform is used.

The best geometry transform is selected by rate distortion optimization (RDO) at encoder. To accelerate the encoder, a simplified encoding process is used in RDO and only the selected geometry transform is encoded with the original encoding process. To recover the geometry correlations between luma and chroma components after geometry transforms, the chroma samples may be adjusted correspondingly to a new phase. The encoder will disable geometry transform with resolution smaller than 1080p.

When a geometry transform is selected, the picture will be restored to the original orientation after reconstruction.

## Inter prediction

### Test 2.1: AmvpMerge for the low-delay picture

In ECM, AmvpMerge mode is designed to apply bilateral matching based candidate reordering. Therefore, AmvpMerge has not been applied for the low-delay picture due to the reference pictures being all forward pictures.

In the test, AmvpMerge is enabled for low-delay pictures by bypassing bilateral matching and template matching based candidate reordering, and MV refinement.

In Test 2.1a, AmvpMerge is enabled with bypassing bilateral matching merge candidate reordering process.

In Test 2.1b, on top of Test 2.1a template matching based MV refinement is bypassed as well.

### Test 2.2: Template matching based BCW index derivation for merge mode

In the test, BCW index for merge coded CUs is derived according to template matching (TM) cost and bi-predicted weights are extended for merge mode.

Firstly, the bi-predicted weights for merge mode is extended from {-2, 3, 4, 5, 10} to {-4, -3, -2, -1, 1, 2, 3, 4, 5, 6, 7, 9, 10, 11, 12} if negative bi-predicted weights are supported and to {1, 2, 3, 4, 5, 6, 7} when negative bi-predicted weights are not supported. The negative bi-predicted weights for non-merge mode {-2, 10} are replaced with positive weights {1, 7}.

Secondly, for a CU coded in one of regular merge, template matching, adaptive decoder-side motion vector refinement and MMVD modes, the choice of bi-predicted weight is derived based on TM cost instead of inferring from neighboring blocks. Given a selected merge candidate, the TM cost values are calculated with different bi-predicted weights, and then, the bi-predicted weight with minimum TM cost value is used to predict the merge CU.

Two tests with (Test 2.2b) and without (Test 2.2a) negative weights are performed.

### Test 2.3: POC based BCW weights derivation

In the test, BCW weights derived based on POC difference are added as follows:

- If both reference pictures are from the past or from the future relatively to the current picture, and the current picture is a non-low-delay picture, the weight pair (-2, 10) is added.

- Otherwise, the weight pair (2, 6) is added.

The larger value from the weight pair is assigned to the closest POC reference picture or list 0 reference picture when the POC distance is the same. The proposed method is not used with MHP for AMVP mode. One bypass bin is added to indicate whether to use default BCW weight or POC based BCW weight.

### Test 2.4: Combination of BCW tests

In this test, extended BCW weights from the tests 2.2 and 2.3 are reordered based on TM cost with (Test 2.4b) and without (Test 2.4a) negative weights.

### Test 2.5: Enhanced temporal motion information derivation

In ECM, TMVP for AMVP mode is derived by fetching motion information from the center or the bottom-right of the collocated block. And a similar strategy is also applied to SbTMVP mode, where the motion information from the left neighboring position is used as a motion shift, which is then employed to obtain TMVPs at sub-CU level.

In the test, two aspects are evaluated:

- Two collocated pictures are used for TMVP derivation, the choices of the collocated pictures are signalled and two reference pictures with the least POC distance relative to the current picture are selected in the encoder implementation.

- The motion shift to locate TMVP is adaptively determined from multiple locations according to the template costs. Two motion shift candidate lists are constructed respectively for the two collocated pictures, only the shift candidates, which point to the collocated picture for which list is being constructed, are considered.

At most 4 SbTMVP candidates are included in the sub-block-based merge list and ARMC is modified such that the SbTMVP candidate with the least template matching cost derived from the first collocated picture is placed in the first entry without reordering, while other SbTMVP candidates are sorted together with AFFINE candidates.

### Test 2.6: DMVR for affine merge coded blocks

In the test, DMVR is applied to affine merge coded blocks. Affine motion consists of translation and non-translation parts. The first stage of multi-stage DMVR is applied to the translation part of the affine motion such that a translation MV offset is added to all the CPMVs of the candidate in the affine merge list if the candidate meets the DMVR condition. And the MV offset is derived by minimizing the cost of bilateral matching which is the same as conventional DMVR.

The first stage refinement process consists of 3x3 square search pattern used to loop through the search range which is set as [-3, 3] to find the best integer MV offset. And then half-pel search is conducted around the best integer position and an error surface estimation is performed at last to find an optimal MV offset with 1/16 precision.

### Test 2.7: Extended weights for MHP

In ECM, multi-hypothesis prediction (MHP) has weights {2, -1} / 8.

Extended weight table represented by syntax element add_hyp_weight_idx for MHP is tested in the following configurations:

Test 2.7a: wt = {2, 1, -1, -2} / 8

Test 2.7b: wt = {3, 2, 1, -1, -2, -3} / 8

Test 2.7c: wt = {2, -1, 3, 1, -2, -3} / 8, but for block size smaller than 256 using only {2, -1} / 8

## Screen content coding

### Test 3.1: IntraTMP for chroma component

In ECM, IntraTMP is applied to luma only. In the test, the mode is extended to chroma components, a single flag is used to indicate whether it is applied for both Cb and Cr components.

### Test 3.2: Using block vector derived from IntraTMP for IBC

In the test, block vector (BV) derived from the intra template matching prediction (IntraTMP) is used for intra block copy (IBC). The stored IntraTMP BV of the neighboring blocks along with IBC BV are used as spatial BV candidates in IBC candidate list construction.

### Test 3.3: Combination of Test 3.1 and Test 3.2

This test is a combination of IBC merge mode with block vector differences (Test 3.1) and IBC with reconstruction reordering (Test 3.2), where IBC MBVD coded block does not inherit flip type from a neighbor block.

## Transform

### Test 4.1: Modification of LFNST for MIP coded blocks

In the test, LFNST is enabled for MIP coded blocks of width and height greater than or equal to 4 and DIMD process is applied to MIP prediction before the upsampling of the current block to derive intra prediction mode as shown in Figure 7 to identify LFNST kernels.

Figure 7. MIP prediction samples to build histogram of gradients

## In-loop filtering

### Test 5.1: Extended fixed-filter-output based taps for ALF

In ECM-6.0, an ALF signalled filter consists of 3 filter inputs: spatial samples (i.e., tap #0 ~ #19), samples before deblocking (i.e., tap #20, #21, #24) and output samples of 2 fixed filters (i.e., tap #22, #23) resulting in 25 taps in total as shown in Figure 8.

Figure 8. ALF filter in ECM

In the test, the output samples of the first fixed filter are extended to 5x5 diamond shape as shown Figure 9.

Figure 9. Extended output samples of the first fixed filter

# Test results

See the accompanied Excel table.