| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0046-v1 |
| --- | --- |

| Title: | AHG9: On StrengthControlVal of the NNPFC SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Maria Santamaria, Francesco Cricri Nokia Technologies Tampere, Finland | Email: | {miska.hannuksela, maria.santamaria_gomez, francesco.cricri}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes the following changes related to the neural-network post-filter characteristics (NNPFC) SEI message:

- Moving the normalization of the quantization strength input variable StrengthControlVal from the VSEI text to the use of NNPFC SEI message in the VVC text.

- Changing the normalization of StrengthControlVal from 2(StrengthControlVal − 42) / 6 to a division (÷) by 63.

It is asserted that with the proposed changes StrengthControlVal is in the range of 0..1, which could help in making StrengthControlVal applicable to codecs other than VVC too.

# Introduction

The contribution proposes specification text responding to the following FINB ballot comment:

| FI_05 | te | StrengthControlVal is a variable that is required to be defined when nnpfc_auxiliary_inp_idc is equal to 1. In VVC, it is defined to be the value of SliceQpY of the first slice of the picture. In Table 23, StrengthControlVal is normalized to the input tensor as follows: inputTensor[ ][ ][ ][ ] = 2(StrengthControlVal − 42) / 6 The values 42 and 6 appear to be related to common test conditions and the quantization scheme of VVC. VSEI should not have such direct dependencies on VVC. | In VSEI, Change the StrengthControlVal to be a variable that has been normalized to the range 0..1. In Table 23, insert the given StrengthControlVal directly into the input tensor: inputTensor[ ][ ][ ][ ] = StrengthControlVal (These proposed changes have a consequence that the normalization of the SliceQpY value has to be added to the derivation of StrengthControlVal within the use of the NNPFC SEI message in the VVC text.) |
| --- | --- | --- | --- |

JVET-AB0109v1 provides simulation results comparing the proposed QP normalization against 2(StrengthControlVal − 42) / 6 when using the neural-network post-filter in JVET-Z0082. JVET-AB0109v1 reports that in the RA configuration the BD-rates for the proposed QP normalization compared to 2(StrengthControlVal − 42) / 6 are

- -0.09% (Y), -0.66% (U), -0.58% (V) for class B; and

- -0.01% (Y), 0.05% (U) and 0.28% (V) for class C.

# Proposed specification text

## Proposed changes to VSEI relative to JVET-AA2006

In Table 23, replace all occurrences of

2( StrengthControlVal − 42 ) / 6

with

StrengthControlVal

Examples of such replacements are shown below for nnpfc_inp_order_idc equal to 0.

Table 23 – Process for deriving the input tensors inputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensors

| nnpfc_inp_ order_idc | Process DeriveInputTensors( ) for deriving input tensors |
| --- | --- |
| 0 | for( yP = −overlapSize; yP < inpPatchHeight + overlapSize; yP++) for( xP = −overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { inpVal = InpY( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight, CroppedWidth, CroppedYPic ) ) if( nnpfc_component_last_flag = = 0 ) inputTensor[ 0 ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = inpVal else inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = inpVal if( nnpfc_auxiliary_inp_idc = = 1 ) if( nnpfc_component_last_flag = = 0 ) inputTensor[ 0 ][ 1 ][ yP + overlapSize ][ xP + overlapSize ] = StrengthControlVal2( StrengthControlVal − 42 ) / 6 else inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 1 ] = StrengthControlVal2( StrengthControlVal − 42 ) / 6 } |
| … | … |

## Proposed changes to VVC relative to JVET-AA2005

In clause D.12.11 (Use of the neural network post-filter characteristics SEI message), replace

StrengthControlVal is set equal to the value of SliceQpY of the first slice of the picture to which the neural-network post-filter characteristics SEI message applies.

with

StrengthControlVal is set equal to firstSliceQpY ÷ 63, where firstSliceQpY is equal to the value of SliceQpY of the first slice of the picture to which the neural-network post-filter characteristics SEI message applies.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).