| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0053-v1 |
| --- | --- |

| Title: | EE1-1.2: NN intra model without attention and partitioning strength |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jacob Ström, Du Liu, Kenneth Andersson, Per Wennersten, Mitra Damghanian, and Ruoyang Yu | Tel: Email: | +46702672192 {jacob.strom, du.liu, kenneth.r.andersson, per.wennersten, mitra.damghanian, ruoyang.yu} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents retrained results for test 1 of JVET-AA0081 for training crosschecking purposes. The intra luma model from JVET-AA0111 has been changed by removing the partitioning input as well as the attention branch. The model has been converted to floating point SADL and is evaluated using the JVET-AA0111 software (equivalent to the NCS-1.0 NN filter set 1 software), meaning that chroma filtering and combination with deblocking is done the same way as for JVET-AA0111. The contribution also contains a second test, which keeps the partitioning input and the attention branch intact. This test acts as a control since the training procedure was different from JVET-AA0111. The results are:

EE1-1.2.1: No partitioning input and no attention branch

- Worst case complexity 664 kMACs/s, model size 6.21M parameters.

- vs NNVC-2.0: 	BDR-Y: -9.49% (RA), -7.52% (AI), -x.xx% (LDB)

- vs NCS-1.0 filter2:	BDR-Y: -0.07% (RA), -0.30% (AI), -x.xx% (LDB)

EE1-1.2.2: Partitioning input and attention branch kept

- Worst case complexity 682 kMACs/s, model size 6.24M parameters.

- vs NNVC-2.0: 	BDR-Y: -9.48% (RA), -7.49% (AI), -x.xx% (LDB)

- vs NCS-1.0 filter2:	BDR-Y: -0.06% (RA), -0.26% (AI), -x.xx% (LDB)

Comparing EE1-1.2.1 with EE1-1.2.2 gives

- Worst case complexity 664 kMACs/s down from 682 kMACs/s (-2.6%)

- Model size 6.24M parameters down from 6.21M parameters (-0.5%)

- BDR-Y: -0.01% (RA), -0.03% (AI), -x.xx% (LDB)

A practical aspect is that it is quicker to train the smaller network (-20% training time). It is proposed to adopt EE1-1.2.1 as the intra luma model in NCS-1.0 NN filter set 1.

# Introduction

The same training procedure was used as in JVET-AA0081 [1]. Scripts, including data extraction software, are available at https://vcgit.hhi.fraunhofer.de/jvet-aa-ee1/VVCSoftware_VTM.git in branch EE1-1.2 with instructions in training/README_instructions.txt. In short, training started with 371 epochs with L1 error metrics and learning rate 10-4, and then continued another 7 epochs with MSE error metrics and learning rate 10-5, resulting in 378 epochs in total.

## Differences to JVET-AA0081

The model has undergone a few changes compared to the one presented in JVET-AA0081. These have been made to make VTM evaluation compatible with the NCS 1.0 software. The changes are:

- The scaling value from samples to floating point values both for the input and output has been changed from 1020 to 1024.

- The scaling value from QPs to floating point QP inputs has been changed from 63 to 64.

- The model has been converted to floating point SADL [4] instead of using libtorch during inference.

- The software that was used to do VTM evaluation in JVET-AA0081 has been changed to the software in JVET-AA0111 [2] (which is equivalent to the one in NCS 1.0 using NN filter set 1). This changes the way the chroma models are used, lowering worst case complexity. It also changes the behavior for RA and LDB, turning on combination with deblocking for those two configurations.

With these changes, it is possible to have an apples-to-apples comparison of EE1-1.2.1 with NCS 1.0 NN filter set 1.

# Summary of results

The results are summarized in Table 1. Complexity numbers are obtained using the thop library [3] version 0.0.31-200524190. The intra chroma model in JVET-AA0111 has a complexity of 110 kMAC/sample and 1.566M parameters, and this chroma model has also been used for EE1-1.2.1 and EE1-1.2.2.

Table 1: Summary of EE1-1.2 results for the intra luma model

| Test | BDR-Y over NNVC2 (AI) | BDR-Y over NCS 1.0 filter set 1 (AI) | Inputs used | kMACs per sample (Y) | kMACs per pixel frame-wise (YUV) | kMACs per pixel block-wise (YUV) | Num. of param total (106) | Mem total (106 bytes) | Epochs trained | Train-ing time (h) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NCS 1.0 filter set 1 (JVET-AA0111) | -7.25% | 0.0% | rec, pred, part, BS, QP | 429 | 539 | 682 | 6.24 | 24.94 | 901 | 601 |
| EE1-1.2.1: without part, no attention | -7.52% | -0.30% | rec, pred, BS, QP | 415 | 525 | 664 | 6.21 | 24.84 | 378 | 175 |
| EE1-1.2.2: retrained version of JVET-AA0111 | -7.49% | -0.26% | rec, pred, part, BS, QP | 429 | 539 | 682 | 6.24 | 24.94 | 378 | 140 |

The difference in BD-rate between the first row and the third row must attributed to training, given that it is exactly the same model architecture and the same inference software. It is reasonable that EE1-1.2.2 is getting a better BDR-Y (-0.26%) given its longer training. In addition, whereas JVET-AA0111 is trained using only the DIV2K dataset, EE1-1.2.2 is trained using data extracted from both the DIV2K and the BVI-DVC dataset. There are also other differences of the training procedure that may influence the result.

The training for EE1-1.2.1 and EE1-1.2.2 however is carried out exactly in the same way. Therefore, their BDR-Y figures are comparable. The model architectures differ in that EE1-1.2.1 lacks the partitioning input as well as an attention branch. This does not seem to harm the BDR-Y results. In fact, the BDR-Y of EE1-1.2.1 is higher than EE1-1.2.2, although we believe that to be training noise rather than a real improvement.

# Training and inference information

## Training

| Table 2. Network Information for NN-based video coding tool testing in training stage for the intra luma model. The intra chroma, inter luma and inter chroma models are the same as in JVET-AA0111. |  |  |
| --- | --- | --- |
| Network Information in Training Stage |  |  |
| Mandatory | GPU Type | GPU: NVIDIA A100-SXM-80GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | see Table 1 |
|  | Batch size: | 64 |
|  | Loss function: | L1 and L2 |
|  | Training time: | see Table 1 |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, qp {20, 25, 30, 35, 40} |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128x128 |
|  | Learning rate: | 1e-4 and 1e-5 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

## Inference

| Table 3. Network Information for NN-based video coding tool testing in inference stage for the intra luma model. The intra chroma, inter luma and inter chroma models are the same as in JVET-AA0111. |  |  |
| --- | --- | --- |
| Network Information in Inference Stage |  |  |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | see Table 1 |
|  | Parameter Precision (Bits) | fp32 |
|  | Memory Parameter (MB) | see Table 1 |
|  | Multiply Accumulate (MAC) | see Table 1 |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128, 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

# Results

## Results over NNVC-2.0.

### EE1-1.2.1: Removing partitioning input and removing

Results for RA over NNVC-2.0:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -8.87% | -15.94% | -18.76% | 257% | 84758% |
| Class A2 | -10.22% | -20.25% | -17.37% | 240% | 79663% |
| Class B | -9.08% | -22.99% | -21.85% | 249% | 77336% |
| Class C | -9.91% | -21.70% | -22.37% | 191% | 63475% |
| Class E |  |  |  |  |  |
| Overall | -9.49% | -20.69% | -20.47% | 231% | 75169% |
| Class D | -11.47% | -23.56% | -23.98% | 186% | 61182% |
| Class F | -5.02% | -11.66% | -10.69% | 331% | 32396% |

Encoding and decoding times are not reliable.

Results for AI over NNVC-2.0:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -6.55% | -17.85% | -19.97% | 205% | 55570% |
| Class A2 | -6.85% | -20.25% | -17.41% | 161% | 43892% |
| Class B | -6.82% | -21.86% | -21.75% | 144% | 39355% |
| Class C | -7.61% | -18.69% | -21.87% | 132% | 26349% |
| Class E | -10.22% | -20.82% | -20.60% | 164% | 42036% |
| Overall | -7.52% | -20.05% | -20.56% | 156% | 39257% |
| Class D | -7.46% | -19.15% | -21.10% | 126% | 24717% |
| Class F | -5.27% | -11.77% | -10.92% | 123% | 33667% |

Encoding and decoding times are not reliable.

Results for LDB over NNVC-2.0:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -9.18% | -15.03% | -15.08% | 194% | 66061% |
| Class E | -8.66% | -16.68% | -17.77% | 476% | 64693% |
| Overall |  |  |  |  |  |
| Class D | -10.80% | -19.08% | -17.99% | 189% | 60403% |
| Class F |  |  |  |  |  |

Encoding and decoding times are not reliable.

### EE1-1.2.2: Model from JVET-AA0111 trained the same way as EE1-1.2.1

Results for RA over NNVC-2.0:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -8.85% | -16.00% | -18.72% | 257% | 85905% |
| Class A2 | -10.22% | -20.22% | -17.38% | 243% | 79546% |
| Class B | -9.07% | -23.00% | -21.75% | 251% | 78079% |
| Class C | -9.89% | -21.76% | -22.34% | 193% | 63702% |
| Class E |  |  |  |  |  |
| Overall | -9.48% | -20.71% | -20.43% | 233% | 75662% |
| Class D | -11.44% | -23.61% | -23.91% | 186% | 62135% |
| Class F | -4.91% | -11.68% | -10.64% | 328% | 33114% |

Encoding and decoding times are not reliable.

Results for AI over NNVC-2.0:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -6.53% | -17.95% | -19.89% | 217% | 63090% |
| Class A2 | -6.85% | -20.48% | -17.82% | 168% | 50583% |
| Class B | -6.78% | -21.61% | -21.57% | 156% | 48101% |
| Class C | -7.55% | -18.63% | -21.83% | 136% | 30612% |
| Class E | -10.20% | -20.80% | -20.59% | 173% | 49392% |
| Overall | -7.49% | -20.02% | -20.56% | 165% | 46104% |
| Class D | -7.45% | -19.13% | -21.07% | 127% | 28963% |
| Class F | -5.01% | -11.76% | -10.92% | 127% | 39089% |

Encoding and decoding times are not reliable.

Results for LDB over NNVC-2.0:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -9.17% | -15.01% | -15.35% | 188% | 63556% |
| Class E | -8.71% | -16.89% | -17.44% | 472% | 63302% |
| Overall |  |  |  |  |  |
| Class D | -10.81% | -18.77% | -18.14% | 189% | 60715% |
| Class F | -4.90% | -9.12% | -6.55% | 309% | 32578% |

Encoding and decoding times are not reliable.

## Results over NCS1.0 filter set 1.

### EE1-1.2.1: Removing partitioning input and removing

Results for RA over NCS-1.0 filter set 1:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.01% | 0.20% | 0.03% | 101% | 99% |
| Class A2 | -0.07% | 0.01% | -0.05% | 101% | 100% |
| Class B | -0.10% | -0.08% | -0.05% | 100% | 101% |
| Class C | -0.08% | 0.03% | 0.06% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | 0.02% | 0.00% | 100% | 100% |
| Class D | -0.05% | -0.19% | -0.27% | 99% | 99% |
| Class F | -0.39% | 0.01% | -0.10% | 99% | 94% |

Encoding and decoding times are not reliable.

Results for AI over NCS-1.0 filter set 1:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.14% | 0.19% | -0.25% | 94% | 93% |
| Class A2 | -0.40% | 0.30% | 0.48% | 99% | 94% |
| Class B | -0.35% | 0.00% | -0.23% | 90% | 84% |
| Class C | -0.21% | 0.20% | -0.18% | 90% | 78% |
| Class E | -0.38% | 0.00% | 0.01% | 91% | 82% |
| Overall | -0.30% | 0.12% | -0.06% | 92% | 85% |
| Class D | -0.12% | 0.33% | 0.29% | 93% | 82% |
| Class F | -1.21% | -0.04% | -0.02% | 94% | 104% |

Encoding and decoding times are not reliable.

Results for LDB over NCS-1.0 filter set 1:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.02% | 0.28% | 0.08% | 102% | 105% |
| Class E | -0.17% | 0.08% | -0.43% | 101% | 102% |
| Overall |  |  |  |  |  |
| Class D | -0.02% | -0.90% | 0.01% | 94% | 93% |
| Class F |  |  |  |  |  |

Encoding and decoding times are not reliable.

### EE1-1.2.2: Model from JVET-AA0111 trained the same way as EE1-1.2.1

Results for RA over NCS-1.0 filter set 1:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.03% | 0.13% | 0.08% | 101% | 101% |
| Class A2 | -0.08% | 0.04% | -0.06% | 103% | 99% |
| Class B | -0.10% | -0.08% | 0.09% | 101% | 102% |
| Class C | -0.06% | -0.04% | 0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.01% | 0.06% | 101% | 101% |
| Class D | -0.01% | -0.26% | -0.18% | 99% | 101% |
| Class F | -0.27% | -0.02% | -0.05% | 99% | 96% |

Encoding and decoding times are not reliable.

Results for AI over NCS-1.0 filter set 1:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.12% | -0.03% | -0.19% | 99% | 106% |
| Class A2 | -0.40% | -0.01% | -0.02% | 103% | 108% |
| Class B | -0.31% | 0.34% | 0.02% | 97% | 103% |
| Class C | -0.14% | 0.27% | -0.14% | 92% | 91% |
| Class E | -0.36% | 0.01% | 0.02% | 96% | 96% |
| Overall | -0.26% | 0.15% | -0.06% | 97% | 100% |
| Class D | -0.11% | 0.36% | 0.33% | 94% | 96% |
| Class F | -0.94% | -0.03% | -0.03% | 97% | 121% |

Encoding and decoding times are not reliable.

Results for LDB over NCS-1.0 filter set 1:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.01% | 0.31% | -0.21% | 98% | 101% |
| Class E | -0.22% | -0.15% | -0.05% | 100% | 100% |
| Overall |  |  |  |  |  |
| Class D | -0.03% | -0.53% | -0.19% | 94% | 94% |
| Class F | 0.07% | -0.14% | -0.30% | 97% | 99% |

Encoding and decoding times are not reliable.

## Results comparing EE1-1.2.1 with EE1-2.2 as an anchor

Results for RA over NCS-1.0 filter set 1:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.02% | 0.07% | -0.05% | 100% | 99% |
| Class A2 | 0.01% | -0.03% | 0.01% | 99% | 100% |
| Class B | -0.01% | 0.00% | -0.14% | 99% | 99% |
| Class C | -0.02% | 0.07% | -0.03% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.03% | -0.06% | 99% | 99% |
| Class D | -0.04% | 0.07% | -0.09% | 100% | 98% |
| Class F | -0.12% | 0.04% | -0.05% | 101% | 98% |

Encoding and decoding times are not reliable.

Results for AI over NCS-1.0 filter set 1:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.03% | 0.22% | -0.07% | 94% | 88% |
| Class A2 | 0.00% | 0.31% | 0.50% | 96% | 87% |
| Class B | -0.04% | -0.34% | -0.25% | 93% | 82% |
| Class C | -0.07% | -0.07% | -0.04% | 97% | 86% |
| Class E | -0.02% | -0.02% | 0.00% | 95% | 85% |
| Overall | -0.03% | -0.02% | -0.01% | 95% | 85% |
| Class D | -0.01% | -0.02% | -0.04% | 99% | 85% |
| Class F | -0.28% | 0.00% | 0.01% | 97% | 86% |

Encoding and decoding times are not reliable.

Results for LDB over NCS-1.0 filter set 1:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.01% | -0.03% | 0.29% | 104% | 104% |
| Class E | 0.05% | 0.24% | -0.39% | 101% | 102% |
| Overall |  |  |  |  |  |
| Class D | 0.01% | -0.37% | 0.21% | 100% | 99% |
| Class F |  |  |  |  |  |

Encoding and decoding times are not reliable.

# Conclusion

This contribution strongly suggests that little or no BD-rate gain is lost by moving to a model that has no partitioning input and no attention branch. In fact, our results show a small gain, although we believe that to be training noise (in our experience, different trainings of the same model can vary as much as 0.1%).

There are several benefits to moving to the smaller model. The worst-case complexity goes down from 682 kMACs/pixel to 664 kMACs/pixel, or -2.6%. The total number of parameters goes down from 6.24M to 6.21M, or -0.5%. The memory therefore also goes down a similar amount. Practically speaking, the training time is faster since the model is smaller, in our case from about 175 hours to 140 hours (-20%), although the speedup may differ with GPU and CPU setup.

Therefore, the contribution proposes to adopt EE1-1.2.1 as the new intra model for NCS-1.0 filter set 1.

# References

[1] J. Ström, D. Liu, M. Damghanian, K. Andersson, Y. Li, P. Wennersten, R. Yu, “EE1-1.2: NN intra model without attention, partitioning and boundary strength.”, JVET-AA0081, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 27th Meeting, by teleconference, 13–22 July 2022

[2] Y. Li, K. Zhang, J. Li, L. Zhang, H. Wang, M. Coban, A. M. Kotra, M. Karczewicz, F. Galpin, K. Andersson, J. Ström, D. Liu, R. Sjöberg, “EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation”, JVET-AA0111, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 27th Meeting, by teleconference, 13–22 July 2022

[3] https://pypi.org/project/thop/

[4] F. Galpin, T. Dumas, P. Bordes, E. François, ”AHG11: Small Ad-hoc Deep-Learning Library (SADL) update”, JVET-AA0086, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 27th Meeting, by teleconference, 13–22 July 2022

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Data from this row is taken from the JVET-AA0111 contribution. The number of epochs trained and the training time are likely not comparable to those of the other rows.