| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0056-v1 |
| --- | --- |

| Title: | AHG9: On leading pictures design in EDRAP SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seung Hwan Kim 10680 Treena St San Diego, CA, 92131, USA | Email: | dr.hendry@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

The current EDRAP SEI message has a flag called edrap_leading_pictures_decodable_flag that specifies whether or not leading pictures associated with the EDRAP picture are decodable leading pictures. This design is asserted to have the following problems:

- In the random access from the EDRAP picture event, if the EDRAP picture has associated leading pictures and they are not decodable, they need to be removed / discarded from the bitstream to be decoded. However, currently there is no way to identify which pictures that follow the EDRAP picture in decoding order are leading picture and whether it is decodable leading or non-decodable leading picture without first parsing and decoding the POC value of the picture.

- In the current design, it is a binary indication such that all leading pictures associated with an EDRAP picture are either decodable or unknown. It may be desirable to allow mix of non-decodable and decodable leading pictures such as the design of RASL and RADL pictures associated with a CRA picture. Particularly, if a mechanism is to be specified to allow identification of leading pictures associated with an EDRAP picture without having to derive the POC values, it may not cause additional complexity to allow mix of non-decodable and decodable leading pictures associated with an EDRAP picture.

To overcome the asserted problem, the following is proposed:

- Introduce two new SEI messages to indicate leading pictures associated with an EDRAP picture. The SEI messages are:

  - DADL indication SEI message to indicate DRAP-Decodable Leading (DADL) picture, that is, leading picture associated with an EDRAP picture that is decodable when random accessing from the EDRAP picture.

  - DASL indication SEI message to indicate DRAP-Skipped Leading (DASL) picture, that is, leading picture associated with an EDRAP picture that is not decodable (and need to be skipped) when random accessing from the EDRAP picture.

- Define necessary constraints to ensure DADL and DASL pictures work.

- Replace the current edrap_leading_pictures_decodable_flag in the EDRAP SEI Message with edrap_leading_pictures_present_flag to specify whether the EDRAP picture may or may not be associated with leading pictures.

# Problem Statement

In the current design of EDRAP SEI message, a picture that is indicated as EDRAP picture may be associated with leading pictures (i.e., pictures that follow the EDRAP picture in decoding order but precede the EDRAP picture in output order). The associated leading pictures of an indicated EDRAP may or may not be decodable leading pictures. Whether the associated leading pictures of an EDRAP are decodable or not is specified by a flag edrap_leading_pictures_decodable_flag signalled in the EDRAP SEI message.

When random accessing from an EDRAP picture, leading pictures associated with the EDRAP picture that are not decodable need to be removed from the bitstream to be decoded. Considering that, the current design of leading pictures associated with EDRAP picture is asserted to have the following problems:

- The flag edrap_leading_pictures_decodable_flag only tells that leading pictures associated with the EDRAP are all decodable or unknown but not about how many leading pictures are associated with the EDRAP picture and which pictures that follows the EDRAP picture are the leading pictures. When random accessing from the EDRAP in which the non-decodable leading pictures need to be removed, the only way to identify which pictures that follow the EDRAP picture in decoding order are leading pictures is by deriving the POC value of the pictures until the first non-leading picture associated with the EDRAP is found. This is totally different from removing non-decodable pictures associated with a CRA (i.e., RASL pictures) since those pictures are easily recognized from their NAL unit header.

- In the current design, it is a binary indication such that all leading pictures associated with an EDRAP picture are decodable or unknown. It may be desirable to allow mix of non-decodable and decodable leading pictures such as the design of RASL and RADL pictures associated with a CRA picture. If a mechanism is to be specified to allow identification of leading pictures associated with an EDRAP picture without having to derive the POC values, it may not cause additional complexity to allow mix of non-decodable and decodable leading pictures associated with an EDRAP picture.

# Proposal

To overcome the asserted problems above, we propose that for leading pictures associated with an EDRAP picture, a mechanism to identify those pictures (i.e., without having to parse and decode POC values) and their types (i.e., either decodable or skipped leading pictures) is specified.

The detail of such mechanism is as follows:

- Introduce the following terms:

  - For decodable leading picture associated with an EDRAP picture, it is referred to as DRAP-Decodable Leading (DADL) picture.

  - For non-decodable / skipped leading picture associated with an EDRAP picture, it is referred to as DRAP-Skipped Leading (DASL) picture.

- Specify two new SEI messages to indicate the leading pictures associated with an EDRAP picture. These two SEI message can be an empty body SEI message (i.e., they do not have any member syntax element). They are:

  - DADL SEI Message to indicate DADL picture.

  - DASL SEI Message to indicate DASL picture.

- The following constraints are specified. These constraints are similar to constraints for RADL and RASL

  - It is constrained that any DASL picture associated with an EDRAP picture shall precede any DADL picture associated with the EDRAP picture in output order.

  - It is constrained that any DADL picture associated with an EDRAP shall not refer to any DASL picture associated with the EDRAP picture or any picture that precedes the EDRAP picture in decoding order, with the exception of the referenceablePictures.

  - It is constrained that any picture that follows an EDRAP in both decoding order and in output order shall not refer to any DASL picture associated with the EDRAP picture or any picture that precedes the EDRAP picture in decoding order, with the exception of the referenceablePictures.

- Replace the current edrap_leading_pictures_decodable_flag in the EDRAP SEI Message with edrap_leading_pictures_present_flag to specify whether the EDRAP picture may or may not be associated with leading pictures.

# Text implementation

The text implementation of the above proposed items is shown below. New texts are marked by yellow highlight and removed text are shown in stricken, red font.

...

### DRAP-Skipped Leading (DASL) Indication

| drap_skipped_indication( payloadSize ) { | Descriptor |
| --- | --- |
| } |  |

The picture associated with a DRAP-skipped leading (DASL) indication SEI message is referred to as a DASL picture. A DASL picture associated with an EDRAP picture shall follows the associated EDRAP picture in decoding order but precedes the EDRAP in output order.

The following constraint applies: Any picture that is in the same layer and follows the EDRAP picture in both decoding order and output order does not include, in the active entries of its reference picture lists, any DASL picture associated with the EDRAP picture or any picture that is in the same layer and precedes the EDRAP picture in decoding order, with the exception of the referenceablePictures.

### DRAP-Decodable Leading (DADL) Indication

| drap_decodable_indication( payloadSize ) { | Descriptor |
| --- | --- |
| } |  |

The picture associated with a DRAP-decodable leading (DADL) indication SEI message is referred to as a DASL picture. A DASL picture associated with an EDRAP picture shall follows the associated EDRAP picture in decoding order but precedes the EDRAP in output order.

The following constraints apply:

–	Any DADL picture associated with an EDRAP picture shall follow any DASL picture associated with the EDRAP picture, if present, in output order.

–	Any DADL picture associated with an EDRAP does not include, in the active entries of its reference picture lists, any DASL picture associated with the EDRAP picture or any picture that is in the same layer and precedes the EDRAP picture in decoding order, with the exception of the referenceablePictures.

…

### Extended DRAP (EDRAP)

| extended_drap_indication( payloadSize ) { | Descriptor |
| --- | --- |
| edrap_rap_id_minus1 | u(16) |
| edrap_leading_pictures_decodable_present_flag | u(1) |
| edrap_reserved_zero_12bits | u(12) |
| edrap_num_ref_rap_pics_minus1 | u(3) |
| for( i = 0; i <= edrap_num_ref_rap_pics_minus1; i++ ) |  |
| edrap_ref_rap_id[ i ] | u(16) |
| } |  |

…

edrap_leading_pictures_decodable_flag equal to 1 specifies that both of the following constraints apply:

–	Any picture that is in the same layer and follows the EDRAP picture in decoding order shall follow, in output order, any picture that is in the same layer and precedes the EDRAP picture in decoding order.

–	Any picture that is in the same layer and follows the EDRAP picture in decoding order and precedes the EDRAP picture in output order shall not include, in the active entries of its reference picture lists, any picture that is in the same layer and precedes the EDRAP picture in decoding order, with the exception of the referenceablePictures.

edrap_leading_pictures_decodable_flag equal to 0 does not impose such constraints.

edrap_leading_pictures_present_flag equal to 1 specifies that the EDRAP picture may have associated leading pictures. edrap_leading_pictures_present_flag equal to 0 specifies that the EDRAP picture has no associated leading picture.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).